3GPP TSG SA2 Meeting #139E	S2-2003966

Elbonia, June 1 – 12, 2020

Source:	Huawei, Hisilicon, OPPO

Title:	KI#7, New Solution: Inter-RAN node MBS Session Handover

Document for:	Discussion/Approval

Agenda Item:	8.9

Work Item / Release:	FS_5MBS/Rel-17

Abstract of the contribution: This contribution address the issue related to the MBS session handover.

- Discussion

This solution addresses Key issue #7, in particular it solves the following problem:

"….when a UE is receiving a multicast session, it may move across NG-RAN nodes and it is possible that the UE moves from a NG-RAN node that supports MBS to one that does not support MBS, or vice versa.

The following aspects will be studied:

-	Triggers for delivery mode switching in 5GS.

-	How delivery mode switching between unicast and multicast modes is performed in the 5GS (including the UE) while supporting service continuity. "

This contribution proposes a solution to solve this issue and assumes an enhancements of Solution #3 (TS 23.757 clause 6.3).

- Proposal

It is proposed to have the following changes in TR 23.757:

# * * * * First change * * * *

6.0	Mapping of solutions to key issues

Editor's note:	This clause describes the mapping between solutions and key issues.

Table 6.0-1: Mapping of solutions to key issues

|  | Key Issues |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Solutions | 1 MBS session management | 2 Service levels definition | 3 Levels of authorization for MC | 4 QoS for MC and BC | 5 BC TV and Radio services | 6 Local MBS | 7 MC-UC delivery mode switch | 8 BC-UC delivery method switch | 9 IWK with EPC/eMBMS for Public Safety |
| 1 |  | x |  |  |  |  |  |  |  |
| 2 | x |  |  |  |  |  |  |  |  |
| 3 | x |  |  |  |  |  |  |  |  |
| 4 | x |  |  |  |  |  |  |  |  |
| 5 | x |  |  |  |  |  |  |  |  |
| 6 | x |  |  |  |  |  |  |  |  |
| 7 |  |  |  |  |  | x |  |  |  |
| X |  |  |  |  |  |  | x |  |  |

# * * * * Second change * * * *

6.x Solution #X: Inter-RAN node MBS session Handover

6.x.1	Functional description

This Solution addresses Key Issue 7 and is based on Solution 3: Integrated Multicast and Unicast Transport (see clause 6.3). It is assumed that the following pieces of information are exchanged among UE, NG-RAN and 5GC after the MBS Session has been established.

-	The UE can provide the UE’s MBS capability to the SMF when the UE establishes the PDU session.

-	For the same QoS flow the NG-RAN receives both the unicast part and multicast part in the QoS profile information. The mapping between multicast QoS flow and unicast QoS flow is a 1 to 1 mapping, i.e., one multicast QoS flow is mapped to one unicast QoS flow. Depending on whether the NG-RAN support MBS, the NG-RAN uses either the unicast part profile or multicast part profile to establish the related DRB and tunnel information allocation.

-	The SMF is aware of whether the data packet is transmitted via the multicast path or unicast path from the response from NG-RAN, e.g., sharing downlink tunnel or unicast downlink tunnel information.

The handover include the following scenarios:

- The source RAN support MBS, UE receives data via MBS Session in Source RAN:

  - Target RAN node does not support MBS,

The MBS Session is converted to the unicast PDU Session at the target side.

  - Target RAN node supports MBS

-  If the MBS Session is not established in Target RAN, the MBS Session can be established during the HO procedure.

-  If the MBS Session is established in Target RAN, reuse the existing MBS session in target RAN.

- The source RAN does not support MBS, the Target RAN supports MBS

In source RAN, the MBS data is sent to UE via the unicast PDU Session. When UE moves to Target RAN, the unicast PDU Session is handed over to the Target RAN as normal PDU Session handling.

After handover completion, the SMF triggers the conversion from the unicast PDU Session to the MBS Session.

6.x.2	Procedures

6.x.2.1	Xn based handover

To cover the scenarios defined in 6.X.1, the Xn based handover procedure is shown below:

Figure 6.x.2.1-1: MBS Xn based HO towards a RAN

Note: for simplification in the figure and description the NG-RAN is shown as the gNB. It can be extended to the other NG-RAN node.

Comparing to the existing Xn based handover procedure (see TS 23.502 [8] and TS 38.300 [x]), the following enhancements apply:

2.  In the Handover Request sent to the target RAN.

Depending on whether the source gNB has established the MBS Session or not, in the request sent to Target gNB the Source gNB includes:

- a) both MBS Session information including the associated MBS Session identifier and unicast PDU Session information ; or

- b) only unicast PDU Session information. The unicast PDU Session information also includes the information of the QoS flows which are transferred via the MBS Session.

If the UE receives data via the unicast PDU Session in the Source gNB, the procedure is the same as the existing Xn based handover procedure. After the handover has been executed, step 12 is performed to join the MBS session.

If the UE receives data via the MBS Session in Source gNB, the following step 3-11 applies:

3.	Depend on whether the Target gNB support MBS or not, the MBS Session information is used to allocate resource or not. If the MBS Session information is used, the corresponding QoS flow part information in the unicast PDU Session information is not used.

If Target gNB support MBS and the MBS session for the indicated MBS Session has not been established in Target gNB, the Target gNB allocates the shared downlink tunnel information for receiving the MBS data from 5GC.

If the MBS Session has already been established in Target gNB, clause 6.x.2.3 describe how to guarantee data lossless.

In the response message, the Target gNB includes the admitted unicast PDU Session information, and, if the MBS Session has been accepted at the Target gNB, also the MBS Session information. Based on the response message, the source gNB knows whether the Target RAN supports MBS Session or not.

4. The source gNB sends the handover command to UE.

Depending on whether the Target gNB supports the MBS Session or not, the MBS Session packets are forwarded to the Target gNB as a separated MBS Session flow or as part of the QoS flow within the unicast PDU Session. After UE successfully connected to the Target RAN, the target RAN starts to send the forwarded data to UE.

6. 	The Target gNB sends Path Switch Request to AMF. The Path Switch Request includes N2 SM message for each active PDU Session including the MBS Session information if it is accepted in the Target gNB.

7. 	The AMF invokes Nsmf_PDUSession_UpdateSMContext request towards SMF, the message includes the received N2 SM message.

Based on the received N2 SM message, the SMF can differentiate three cases:

-	Case A) The Target gNB supports MBS and the MBS session has previously been established at the Target gNB.

The SMF does not need to establish the shared tunnel for the indicated MBS Session. The SMF interact with the UPF only for unicast PDU Session.

-	Case B) The Target gNB supports MBS, but the MBS session has not yet been established.

The SMF provides the N2 MBS Session information to the SMF2, i.e. multicast SMF. The SMF2 provided the shared downlink tunnel info to the UPF2 (multicast UPF).

-	Case C) The Target gNB does not support MBS.

Based on the received N2 SM information, the SMF knows that the MBS has been mapped to the unicast PDU Session.

If the path between the UPF(unicast) and UPF2(multicast anchor) has not been already established, the SMF inform SMF2 of the downlink tunnel info of the UPF(unicast) for receiving the MBS packets from UPF2. SMF2 indicates to UPF2 to copy and forward the MBS packets received by UPF2 via the unicast PDU Session tunnel to the target gNB.

8/9. The SMF or UPF sends the end marker packet(s) via old path of the PDU session to the Source RAN.

The Source gNB forwards the end marker to Target gNB. If the MBS Session is a separated forwarding path, the Source gNB copies the end marker from the unicast PDU Session and inserted it into the forwarding tunnel for MBS Session. .

10.	The Target gNB receives the end marker packet from source gNB.

The target gNB starts to send the buffered packets for the PDU Session including buffered MBS data if any to the UE.

Step 12 is performed only for the case: the source gNB does not support MBS, but the Target gNB supports MBS:

12. After handover completion, if the UE and Target RAN supports MBS, the SMF triggers the MBS session establishment, and request Target gNB to establish the MBS Session:

Steps 5 to 15 of Solution 3 (see clause 6.3.2) are performed.

6.x.2.2	N2 based handover

To cover the scenarios defined in 6.X.1, the N2 based handover procedure is shown below:

Figure 6.x.2.2-1: MBS N2 based HO towards a RAN

Comparing to the existing N2 based handover procedure (see TS 23.502 [8] clause 4.9.1.3), the following enhancements is added:

2. 	The source RAN sends the handover required to S-AMF.

Depending on whether the source RAN has established the MBS Session or not, in the RAN container the Source RAN includes

- a) both MBS Session information including the associated MBS Session identifier and unicast PDU Session information ; or

- b) only unicast PDU Session information. The unicast PDU Session information also includes the information of the QoS flows which are transferred via the MBS Session.

Also the Source RAN indicates QoS flow(s) for which forwarding is needed.

4. 	Based on whether the MBS Session is established at the source RAN side, the SMF can differentiate two cases:

If MBS Session has been established at the source RAN, the SMF includes both MBS Session information including the associated MBS Session identifier and unicast PDU Session information to the Target RAN.

If MBS Session has not been established at Source RAN, the SMF only includes the unicast PDU Session information to the Target gNB, i.e. the QoS flows of the PDU Session include the QoS flows for MBS data packet.

An indication of whether forwarding is possible is also included in the N2 SM message.

5.	The Target RAN prepares the radio resource based on the received information.

For the case that the Target RAN received both MBS Session information and unicast PDU Session information:

- If the Target RAN supports MBS, the Target RAN use the MBS Session information allocate resource to deliver the MBS data. The corresponding QoS flow part information in the unicast PDU Session information is not used. If the MBS Session has not yet established, the Target RAN allocates shared downlink tunnel info of the shared tunnel for receiving the MBS data. If forwarding is needed for the MBS, the target gNB allocates the forwarding tunnel info for the MBS.

- If the Target RAN does not support MBS, the MBS Session information is not used. The Target RAN uses the unicast PDU Session information to allocate resource to deliver MBS data. The MBS data are transmitted as one of the QoS flows in the unicast PDU Session.

For the case that the Target RAN received MBS QoS profile with only unicast PDU Session information:

- Since the MBS data are transmitted via the unicast QoS flows of the PDU Session, the Target RAN just allocates resource for the unicast PDU Session as usual.

6. The Target RAN sends handover request Ack to T-AMF, includes N2 SM message for each accepted PDU Session and MBS Session (if need and accept) as allocated at the step 5.

7. 	The T-AMF invokes Nsmf_PDUSession_UpdateSMContext request towards SMF, the message includes the N2 SM message.

8.  Based on the N2 SM message, the SMF can differentiate two cases:

If the target RAN support MBS, and indirect forwarding is needed, the SMF provides the Target RAN forwarding tunnel for the MBS Session to UPF and requests the UPF to allocate forwarding Tunnel info for the MBS Session, i.e. the forwarding tunnel for MBS data and unicast data packet is separated.

If the target RAN does not support MBS, and indirect forwarding is needed, the forwarding tunnel for unicast PDU Session is used for the MBS data. The indirect forwarding tunnel is established as in existing N2 based handover procedure in TS 23.502 [8].

12. The source RAN sends the Handover Command to UE.

If separated indirect forwarding tunnel for the MBS session has been established, the source RAN copies the MBS data received from 5GC and forwards them to Target RAN via the indirect MBS Session forwarding tunnel.

If no separated indirect forwarding tunnel for the MBS session has been established, the source RAN copies the MBS data received from 5GC, and forwards the packet(s) via the unicast PDU session forwarding tunnel to Target RAN.

15. Based on the received N2 SM message at step 7, the SMF can differentiate three cases:

- Case A) The Target RAN supports MBS and the MBS session has previously been established at the Target RAN.

The SMF does not need to establish the shared tunnel for the indicated MBS Session. The SMF interact with the UPF only for unicast PDU Session.

- Case B) The Target RAN supports MBS, but the MBS session has not yet been established.

The SMF provides the N2 MBS Session information to the SMF2, i.e. multicast SMF. The SMF2 provided the shared downlink tunnel info to the UPF2 (multicast UPF).

- Case C) The Target RAN does not support MBS.

Based on the received N2 SM information, the SMF knows that the MBS has been mapped to the unicast PDU Session.

If the path between the UPF (unicast) and UPF2 (multicast anchor) has not been already established, the SMF inform SMF2 of the downlink tunnel info of the UPF (unicast) for receiving the MBS packets from UPF2. SMF2 indicates to UPF2 to copy and forward the MBS packets received by UPF2 via the unicast PDU Session tunnel to the Target RAN.

16.The SMF or UPF sends the end marker packet(s) via old path of the PDU session to the Source RAN.

The Source RAN forwards the end marker to Target RAN. If the MBS Session is a separated forwarding path, the Source RAN copies the end marker from the unicast PDU Session and inserted it into the forwarding tunnel for MBS Session.

Step 18 and 19 are performed only for the case: the source RAN does not support MBS, but the Target RAN supports MBS:

18. After handover completion, if the UE and Target RAN supports MBS, the SMF triggers the MBS session establishment, and request Target RAN to send MBS data to UE via the MBS Session:

Steps 5 to 15 of clause 6.3.2 are performed.

6.x.2.3	Lossless packet transferring

To accommodate lossless MBS data packet transmission it is assumed that the sequence number is contained for each data packet of the MBS session from UPF2 (i.e. MBS Session Anchor UPF) to NG-RAN.

During the handover procedure the Target NG-RAN compares the data packet sequence number of the forwarded MBS data from Source NG-RAN and the data packet sequence number of the MBS data received directly from 5GC. Based on the comparison the Target NG-RAN determines when to sends the MBS data received directly from 5GC to the UE. For example, when the data packet sequence number of the forwarded MBS data from Source gNB is equal to the data packet sequence number of the MBS data received directly from 5GC, the Target NG-RAN switches from unicast to multicast.

If the target side does not support MBS, the MBS data is sent to Target gNB as unicast PDU Session from UPF2 via the UPF (PSA). SMF set a timer to hold the MBS data packet at the UPF (PSA). When the timer expires, the SMF enable the MBS data sent from UPF to Target gNB. It is assumed when this timer is expired, the data packet gap of the MBS data between the two different gNB has been filled via the forwarding data path.

6.X.3	Impacts on services, entities and interfaces

SMF

-	Maps the MBS into PDU Session or to MBS session depending on UE MBS capability and RAN capability

-	Set timer to hold/enable MBS data packet forwarding at the UPF to avoid packet loss in case target RAN does support MBS

UPF

-	Add sequence number into MBS packets

RAN

-	Handle sequence number of forwarded MBS packets and MBS packets received from CN to reduce packet loss during inter RAN node handover.

-	Support to manage the separated forwarding tunnel of the MBS Session. This includes:

-	Copy the End Marker from the unicast PDU Session into the forwarding tunnel of the MBS Session.

UE

-	Provide UE MBS capability to SMF to support the mode switch

# * * * * End of change * * * *