3GPP TSG SA2 Meeting #139E	S2-20xxxxx

June 01 - 12, 2020, Elbonia

Source:	Lenovo, Motorola Mobility, Huawei? CATT? LG Electronics? Alibaba?

Title:	KI#2, new solution: UE aware edge relocation

Document for:	Discussion/Approval

Agenda Item:	8.3

Work Item / Release:	FS_enh_EC / Rel-17

Abstract of the contribution: This contribution proposes a solution to support UE aware edge relocation.

- Introduction

This contribution proposes the solution to support edge application server relocation with the UE aware of the edge application server relocation, covering both the scenarios triggering the edge application server relocation by the user plane management notification with PSA relocation/DNAI change and by the application side.

- Proposal

It is proposed to have the following changes in TR 23.748:

* * * * 1st Change * * * *

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

| Solutions | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 5 |
| #1: Provisioning URSP configuration to the UE to establish PDU Sessions for edge applications | X |  |  |  |
| #2: Local DNS based edge server address discovery | X |  |  |  |
| #3: DNS AF | X |  |  |  |
| #4: Providing the DNS authoritative server with IP addressing information about where the UE is located | X |  |  |  |
| #5: Server Discovery using DNS, IP Routing and URSP | X |  |  |  |
| #6: Discovery of EAS based on DNS | X |  |  |  |
| #7: SMF/I-SMF selection based on DNAI | X |  |  |  |
| #X: UE aware edge relocation |  | X |  |  |

* * * * 2nd Change (all new)* * * *

6.X	Solution #X:  UE aware edge relocation

6.X.1	General

The solution addresses Key Issue #2: Edge Relocation. The UE is aware of the edge relocation.

This solution supports the Session Breakout model and describes re-discovery of Edge Application Server at edge application server relocation after the UE accessing the edge application to support the service continuity. The enhancement for packet loss is not considered in this solution.

The edge application relocation may happen in the following scenarios:

- the application server change is triggered by the user plane management notification with PSA relocation/DNAI change e.g. due to UE mobility;

- the application server change is initiated from the application side e.g. due to the serving Edge Application Server becoming congested or being in outage condition.

The application can decide whether the edge application server relocation is transparent or not to the UE, without impacting the user experience for the application.

6.X.2	Procedures

6.X.2.1	Procedure for edge relocation triggered by CN

Figure 6.x.2.1-1: Procedure for edge relocation triggered by CN

The AF can get the information of whether server changes and the information of the target application server based on the user plane management notification.

- The AF request with traffic routing information is sent to the Core network per application, it may include the domain name of the application, the application relocation possibility indication and the 'AF acknowledgement to be expected' indication.

- The PCF generates PCC rules based on the AF request and provides the PCC rules to the SMF. The PCC rules include the domain name of the application. This step may happen during establishment of the PDU Session or during modification of the PDU Session.

- The SMF determines to relocate the ULCL/L-PSA. The ULCL/L-PSA relocation may be triggered by UE mobility.

- Based on the AF subscription, the SMF sends an early notification to the AF, including the corresponding target DNAI(s).

- Application server relocation is triggered by the notification received in step 4 based on the target DNAI(s). The AF can get the information of target application server.

- The AF sends a positive response to the early notification to the SMF. In the positive response, the AF includes the indication to indicate whether the application server relocation is transparent to the UE, and the information of target application server (the information of target application server may be the information on the target application server for the application (e.g. the IP address of the target application server), or the indication for application server relocation (e.g. a trigger for UE re-initiating DNS Query for the application) and the FQDN of the target application server).

- The SMF configures the new ULCL and new L-PSA.

- The SMF decides to inform UE the target AS information of this application based on the information received in step 6.

- The SMF sends a PDU Session Modification Command message with AS change notification information to UE. In the AS change notification information, the SMF includes the information of target application server.

- The UE rediscovers the target application server for the application based on the information received in step 9.

NOTE:	The UE behaviour for rediscovering the target application server can be one of the following case:

1) If the information is the IP address of the target application server for the application, the UE establishes a new connection with the target application server and exchanges the service data with the target application server for the corresponding application.

2) If the information includes the trigger for UE initiating DNS Query for the application, the UE disables the old DNS query result for the corresponding application and re-initiates a new DNS query for the corresponding application to the DNS server.

Whether and how the lower layer and the application client interact within the UE may be decided by implementation.

Editor's note: It is FFS how to identify and exchange new EAS’s information to the application client in the UE.

Editor's note: It is FFS the how to figure out the conflict between the information of target application server provided by NAS layer and that provided by HTTP adaption layer.

- The UE sends PDU Session Modification Command Ack NAS message to SMF.

6.X.2.2	Procedure for edge relocation triggered by application

Figure 6.x.2.2-1: Procedure for edge relocation triggered by application

The AF can get the information of whether server changes and target application server for the edge relocation triggered by application.

1a. PDU session is established with the PSA UPF and breakout with the ULCL1/L-PSA1.

1b. The AF request with traffic routing information is sent to the Core network per application.

1c. Edge application server relocation is triggered by the application due to the serving Edge Application Server becoming congested or being in outage condition.

- The AF updates the AF request for the edge application relocation. It includes the indication to indicate whether the application server relocation is transparent to the UE, and the information of target application server (the information of target application server may be the information on the target application server for the application (e.g. the IP address of the target application server), or the indication for application server relocation (e.g. a trigger for UE re-initiating DNS Query for the application) and the FQDN of the target application server)..

- The PCF updates the PCC rule with the information received in step 2 to the SMF. If the ULCL/PSA relocation is not triggered by the edge application relocation, step 4 to step 7 are skipped.

- The SMF determines to relocate the ULCL/L-PSA based on the edge application relocation.

- Based on the AF subscription, the SMF sends an early notification to the AF, including the corresponding target DNAI(s).

- The AF sends a positive response to the early notification to the SMF.

- The SMF configures the new ULCL and new L-PSA.

- The SMF decides to inform UE the target AS information of this application based on the information received in step 2.

  - the same as step 9 to step 11 in clause 6.x.2.1.

6.X.2.3	Procedure for edge relocation triggered by application with application relocation exposure

Figure 6.x.2.3-1: Procedure for edge relocation triggered by application with application relocation exposure

The AF can get the information of whether server changes and target application server for the edge relocation triggered by application.

- PDU session is established with the PSA UPF and breakout with the ULCL1/L-PSA1.

- The AF request with traffic routing information is sent to the Core network per application. The information of the capability for application relocation exposure to the core network is included.

- The PCF generates PCC rules based on the AF request and provides the PCC rules to the SMF. The PCC rules include the information of the capability for application relocation exposure to the core network.

- The SMF subscribes to the AF for the Application relocation exposure event based on the information for Application relocation exposure capability received in step 3.

- Edge application server relocation is triggered by the application due to the serving Edge Application Server becoming congested or being in outage condition.

- The AF notifies the event for the edge application relocation to the SMF. It includes the indication to indicate whether the application server relocation is transparent to the UE, and the information of target application server (the information of target application server may be the information on the target application server for the application (e.g. the IP address of the target application server), or the indication for application server relocation (e.g. a trigger for UE re-initiating DNS Query for the application) and the FQDN of the target application server). If the ULCL/PSA relocation is not triggered by the edge application relocation, step 7 to step 10 are skipped.

- The SMF determines to relocate the ULCL/L-PSA based on the edge application relocation.

- Based on the AF subscription, the SMF sends an early notification to the AF, including the corresponding target DNAI(s).

- The AF sends a positive response to the early notification to the SMF.

- The SMF configures the new ULCL and new L-PSA.

- The SMF decides to inform UE the target AS information of this application based on the information received in step 6.

  - the same as step 9 to step 12 in clause 6.x.2.1.

6.X.3	Impacts on Existing Nodes and Functionality

SMF:

-  Decides to inform UE the target AS information of this application based on the edge application server relocation information received from AF.

-  Informs the edge application server relocation information to the UE.

-  For 6.X.2.3, additionally, subscribes to the AF for the Application relocation exposure event.

AF:

-	Provides the edge application server relocation information to CN, the information includes the indication to indicate whether the application server relocation is transparent to the UE and the information of target application server.

-  For 6.X.2.3, additionally, exposes the edge relocation capability to the CN and notifies the edge application server relocation event to the SMF.

UE:

-	NAS layer receives and forwards the information of target application server sent by the network to the upper layer.

-  Impact on UE kernel, operation system kernel (e.g. Android kernel) to support to identify and exchange the information of the target application server.

-  Impact on HTTP adaption layer when it is applied.

-  Impact on Edge Enabler Client, when it is applied.

-  Application client re-discovers the target application server for the application based on the edge relocation information received and exchanges the service data with the target application server for the corresponding application.

* * * * End of Change * * * *