3GPP TSG SA2 Meeting #139E	S2-2003999r01

June 01 - 12, 2020, Elbonia

Source:	Lenovo, Motorola Mobility, Intel? China Telecom?

Title:	KI#2, new solution: CN-based edge relocation

Document for:	Discussion/Approval

Agenda Item:	8.3

Work Item / Release:	FS_enh_EC / Rel-17

Abstract of the contribution: This contribution proposes a solution to support CN-based edge relocation.

# Introduction

This contribution proposes the solution to support edge application server (EAS) relocation for the Session Breakout connectivity model, covering both the scenarios triggering the edge application server relocation by the user plane management notification with PSA relocation/DNAI change and by the application side.

This contribution focuses on the procedure on required information exchange between the AF and the CN and CN actions needed to setup new ULCL/PSA to support the service continuity.

# Proposal

It is proposed to have the following changes in TR 23.748:

* * * * 1st Change * * * *

# 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

| Solutions | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 5 |
| #1: Provisioning URSP configuration to the UE to establish PDU Sessions for edge applications | X |  |  |  |
| #2: Local DNS based edge server address discovery | X |  |  |  |
| #3: DNS AF | X |  |  |  |
| #4: Providing the DNS authoritative server with IP addressing information about where the UE is located | X |  |  |  |
| #5: Server Discovery using DNS, IP Routing and URSP | X |  |  |  |
| #6: Discovery of EAS based on DNS | X |  |  |  |
| #7: SMF/I-SMF selection based on DNAI | X |  |  |  |
| #X: CN-based edge relocation |  | X |  |  |

* * * * 2nd Change (all new)* * * *

## 6.X	Solution #X:  CN-based edge relocation

### 6.X.1	General

The solution addresses Key Issue #2: Edge Relocation. The UE is Edge Computing service agnostic, so it is unaware of the edge relocation

This solution supports the Session Breakout model at edge application server relocation after the UE accessing the edge application to support the service continuity.

The edge application relocation may happen in the following scenarios:

- the application server change is triggered by the user plane management notification with PSA relocation/DNAI change e.g. due to UE mobility;

- the application server change initiated from the application side e.g. due to the serving Edge Application Server becoming congested or being in outage condition.

The CN role is to ensure connectivity to the new EAS by setting up a proper ULCL and PSA. The following assumptions are made that make any further enhancements unnecessary

- The Edge Computing Application in the UE will be notified at application layer about the new EAS to use after the relocation takes place

- The connection between the UE and the EAS is robust to IP address changes both on the UE and EAS side. This could be achieved e.g., by using enhanced transport protocols, e.g., QUIC.

### 6.X.2	Procedures

#### 6.X.2.1	Procedure for edge relocation triggered by CN

Figure 6.x.2.1-1: Procedure for edge relocation triggered by CN

The AF can get the information of whether server changes and the target application server based on the user plane management notification.

- The AF request with traffic routing information is sent to the Core network per application, it may include the domain name of the application, the application relocation possibility indication and the 'AF acknowledgement to be expected' indication.

- The PCF generates PCC rules based on the AF request and provides the PCC rules to the SMF. The PCC rules include the domain name of the application. This step may happen during establishment of the PDU Session or during modification of the PDU Session. 
The edge application traffic is set up using one of the mechanisms proposed for EAS discovery and selection (e.g., Solution 3) towards the Old AS through the ULCL 1 and L-PSA1 that are set up based on the PCC rules for this application

- The SMF determines to relocate the ULCL/L-PSA. The ULCL/L-PSA relocation may be triggered by UE mobility.

- Based on the AF subscription, the SMF sends an early notification to the AF, including the corresponding target DNAI(s).

- Based on target DNAI(s) of the notification received in step 4 the AF (or related function) determines that application server relocation is needed. The AF can get the information of target application server IP address.

- The AF sends a traffic influence request to the SMF for this UE, as described in Clause 4.3.6.4 of TS 23.502. In this, the AF includes the information of target application server (e.g. the IP address of the target application server,or an IP address range covering all potential EASs the UE might need to open up connections to later on)).

- The SMF sets up the new ULCL and new L-PSA determined in Step 3 and configures the filters based on information received from the AF in Step 6.

- The SMF sends event exposure to AF (late notification) about the new ULCL and new L-PSA. If NEF is used, then this translates to Nnef_TraffcInfluence_Nofity to the AF.
Note: traffic is still flowing through the old ULCL and old PSA to the old EAS.

- Context migration between old and new EAS, triggered by AF.

- Application level redirect message to UE app by Old EAS specifying new EAS IP address. Traffic starts through new ULCL and new PSA.
Note: SMF could set inactivity timers for the traffic through the old ULCL and old PSA based on which they are removed shortly after this step.

#### 6.X.2.2	Procedure for edge relocation triggered by application

The procedure described in clause 6.x.2.1 can be re-used, starting from Step 5, assuming that the AF can get the information of whether the edge application server needs to change and the IP address (ranges) related to the target application server. Also note that server relocation events might involve multiple UEs, in which case in Step 6 the AF will invoke a Nnef_TrafficInfluence service with the required information to identify the group of UEs, as described in Clause 4.3.6.2 of TS 23.502.

### 6.X.3	Impacts on Existing Nodes and Functionality

SMF:

-

AF:

-

* * * * End of Change * * * *