SA WG2 Meeting #139E	S2- 2004183

1 June – 12 June 2020

Source:	Samsung

Title:	KI #14, New Sol: new application detection

Document for:	Approval

Agenda Item:	8.1

Work Item / Release:	FS_eNA_ph2/ Rel-17

Abstract of the contribution: This contribution proposes a new solution for NWADF-assisted application detection.

- Discussion

To differentiate an application traffic handling, the first step is to distinguish application traffic from other traffics. In 5G network, an application can be distinguished by a set of packet headers (SDFs) or application IDs. The most common approach to detect traffic is a set of packet headers, that usually contains source and destination address. The application ID can be used for referring the UPF’s specific application detection filter. For managing detection information associated with application IDs, AF may provide PFDs according to clause 4.18 of TS 23.502 to update application detection filter information.

Nowadays, application deployment environment is getting complex due to the introduction of cloud computing and virtualization/containerization techniques. For example, with the introduction of cloud computing, an address and port based application detection approach is not an efficient way because the application are easily migrated to other addresses and the ports could be changed. In other hand, the number of application is also increasing rapidly. It means that need to update UPF’s specific filters more frequently to provide differentiated service on time. To address these problems, a network need to a way to detect application traffic considering unique characteristics of network traffic such as a payload signature, statistical characteristics, and traffic patterns.

The objective of this solution is to extract network traffic signature using the network data analytics. To realize this solution, one of the main feature is 1) collect measurement of an application to extract statistical characteristics, 2) collect payload of packets to extract payload signature (such as domain name contained in the payload). The baseline usage of the generated analytics is to store the captured application characteristics as a PFD, and the PFD is used by SMF and UPF to detect an application defined as TS 23.502.

Proposal. NWDAF collects packets and measurement, and extract unique application signature as an output analytic.

2.	Proposal

It is proposed to include the added text into the TR 23.700-91 as a solution for Key Issue #14.

* * * * First Change * * * *

6.0	Mapping Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

| Solutions | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| 1 | 11 |  |  |  |
| 2 | 4 |  |  |  |
| 3 | 15 |  |  |  |
| 4 | 12 |  |  |  |
| X | 14 |  |  |  |

* * * * Second Change (all new text) * * * *

6.X	Solution X: NWDAF assisted new application detection

6.X.1	Description

This solution is for Key Issue #14: NWDAF-assisted application detection.

To differentiate an application traffic handling, the first step is to distinguish application traffic from other traffics. In 5G network, an application can be distinguished by a set of packet headers (SDFs) or application IDs. The most common approach to detect traffic is a set of packet headers, that usually contains source and destination address. The application ID can be used for referring the UPF’s specific application detection filter. For managing detection information associated with application IDs, AF may provide PFDs according to clause 4.18 of TS 23.502 [X] to update application detection filter information. A PFD contents could contain flow description (service-side 3-tuple), URL, and domain name/protocol information. When an AF deliver PFD to NEF (PFDF), it will be distributed to SMFs and UPFs to enable flow detection according to clause 5.8.2 of TS 23.501 [X].

The objective of this solution is to automate application detection method using PFDs for newly/updated applications by extracting network traffic signature using the network data analytics. To realize this solution, one of the main features is 1) collect measurement of an application to extract statistical characteristics, 2) collect payload of packets to extract payload signature (such as domain name contained in the payload). The baseline usage of the generated analytics is to store the captured application characteristics as a PFD, and the PFD is used by SMF and UPF to detect an application defined as TS 23.502.

The below is the principles of the solution:

- A consumer (e.g. NEF) request to NWDAF to detect new applications, not matched with SDF or application IDs. The triggering can be configured by local configuration or OAM.

- NWDAF collect SMF’s data for user plane usage report and/or packets for default QoS flow (not matched with any SDF or application ID)

- NWDAF analyse the collected data to generate unique packet signature, traffic characteristic, and traffic patterns.

- The consumer stores PFD associated with an application ID to enable the detection of application traffic in the future. (out of scope)

6.X.2	Input Data

NWDAF collects QoS flow related data from SMF for a specific S-NSSAI, DNN, and UE. This solution collects data for matched on default QoS flow, which no SDF or Application IDs are known. The detailed data are described in Table 6.X.2-1.

Table 6.X.2-1 input data to detect new application from NFs

| Information | Source | Description |
| --- | --- | --- |
| SUPI |  |  |
| S-NSSAI | SMF | Identifies the Network Slice for which analytics information is provided. |
| DNN | SMF | Identifies the data network name (e.g. internet) for which analytics information is provided |
| Start/end time | SMF | Start and end time of traffic detection |
| Data volume | SMF | Measured data traffic volume (per UL/DL) |
| Data duration | SMF | Measured data traffic duration (per UL/DL) |
| QoS flow Bit Rate | SMF | The observed bit rate (per UL/DL) |
| Packet transmission | SMF | The observed number of packet transmission (per UL/DL) |
| Payload packets | SMF | The user plane packets collected. |

Editor’s Note: For collecting packets from UPF to SMF, the forwarding mechanism is defined in clause 5.8.2.5.2 of TS 23.501. It is FFS, how expose SMF the forwarded packets to NWDAF and how many packets need to be collected.

6.X.3	Output Analytics

The output analytics of NWDAF is defined in Table 6.X.3-1.

- Table 6.X.3-1. Output for new application detection

| Information | Description |
| --- | --- |
| S-NSSAI | Identifies the Network Slice for which analytics information is provided. |
| DNN | Identifies the data network name (e.g. internet) for which analytics information is provided |
| Flow descriptor | Flow descriptor containing 3-tuple, server side (destination address, port, and protocol) |
| URL | the significant parts of the URL to be matched, e.g. host name defined in |
| Domain name information | a Domain name matching criteria and information about applicable protocol(s). |

6.X.4	Procedures

For the procedure to extract the unique characteristic of application traffic such as traffic pattern, URL, and/or domain name related information. The procedure depicted in 6.X.4-1 shows new application detection analytics and store as PFDs by NEF.

Figure 6.X.4-1. A procedure to extract application characteristics

Step 1. NEF request to NWDAF to request application detection information. This request maybe be triggered by local configuration or OAM.

Step 2. A target UE establishes PDU session for communication

Step 3. NWDAF request to subscribe session related information and request to forward packets.

Editor’s Note: For collecting packets from UPF to SMF, the forwarding mechanism is defined in clause 5.8.2.5.2 of TS 23.501. It is FFS, how expose SMF the forwarded packets to NWDAF and how many packets need to be collected.

Step 4. A NWDAF extracts the application traffic characteristics.

Step 5. NWDAF notifies the analytic to the NEF with outputs defined in Table 6.X.3-1

6.X.5	Impacts on services, entities and interfaces

NWDAF:

- Needs to provide a new analytic for application detection information

- Needs to process user plane data for extracting traffic characteristics

SMF:

- Needs to forward user plane packets to NWDAF

* * * * End of change * * * *