3GPP TSG-SA WG2 Meeting #139E	S2-2004189

# June 01 - 12, 2020, Elbonia

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2377 | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Truncated 5G S-TMSI for early UE capability retrieval |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16, 5G_CIoT |  |  |  |  |  | Date: |  |  | 2020-05-22 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In LS R2-2003935 RAN2 requests SA2: RAN2 respectfully asks SA2, CT1 and RAN3 - To consider feasibility of introducing enhancements to enable the (ng-)eNB for early UE capability retrieval for eMTC UEs connected to EPC and 5GC in Rel-16, and if confirmed feasible, specify the required changes based on truncated 5G-S-TMSI solution in their specifications. From system perspective, the only main system impact is the configuration of 5G S-TMSI, as the AS procedures for capability retrieval are in RAN WGs domain. Since RAN2 is intending to use the truncated 5G S-TMSI, which currently is limited to NB-IoT use ony, also for LTE-M, the solution for configuring the rules for truncated 5G S-TMSI in the UE need to be extended to LTE-M. In terms of N2 procedure, note that this early UE capability retrieval is already suported in the context of CP optimization procedure for NB-IoT. From TS 23.502, 4.24.1: In the NB-IoT case, during step 1 the NG-RAN, based on configuration, may retrieve the NB-IoT UE Priority and the Expected UE Behaviour Parameters from the AMF, if not previously retrieved. Based on such parameters, the NG-RAN may apply prioritisation between requests from different UEs before triggering step 2 and throughout the RRC connection. The NG-RAN may retrieve additional parameters (e.g. UE Radio Capabilities). Therefore the only clarification is to mention that this can be performed for NB-IoT and LTE-M in general. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Extend the AMF provisioning of truncated 5G S-TMSI information to UEs accesing via LTE-M. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Early capability retrieval feature would not work for LTE-M case, only for NB-IoT. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.31.4.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** BEGIN CHANGES ***

##### 5.31.4.x	Support of Early capability retrieval

The NG-RAN may perform Early UE Capablity Retrieval for UEs accessing via NB-IoT or LTE-M (see TS 36.300 [27]) by requesting UE radio capabilities to AMF using the UE's 5G-S-TMSI as the identifier.

In the Early UE capability retrieval procedure, the UE provides a Truncated 5G-S-TMSI as the UE identifier. The NG-RAN is configured with the sizes of the components of the Truncated 5G-S-TMSI and it is configured with how to recreate the AMF Set ID, the AMF Pointer and 5G-TMSI from the equivalent truncated parameters (see TS 23.003 [19]).

NOTE:	In case of RAN sharing the sharing partners are assumed to agree on a common configuration, i.e. common component sizes of the Truncated 5G-S-TMSI and how to recreate the AMF Set ID, the AMF Pointer and 5G-TMSI from the equivalent truncated parameters.

If the AMF determines the RAT type to be LTE-M or NB-IoT, the AMF configures the UE with the Truncated 5G-S-TMSI Configuration that provides the sizes of the components of the Truncated 5G-S-TMSI as described in TS 24.501 [47] during the Registration. The configuration of these parameters are specific to each PLMN.

*** END CHANGES ***