SA WG2 Meeting #139-e	S2-2004230

01 - 12 June 2020, Electronic meeting

Source:	Intel

Title:	KI #3, New Sol: Enhancement to 5GS TSN bridge capability reporting and configuration

Document for:	Approval

Agenda Item:	8.5

Work Item / Release:	FS_IIoT

Abstract of the contribution: This contribution proposes enhancement to the NEF (exposure) framework toward the TSN AF so that the NEF can expose network capabilities to support TSC. A TSN Bridge exposure service allowing the 5GS to report NW-TT capabilities of specific TSN 5GS bridge to the TSN AF via the NEF, and allowing the TSN AF to configure the NW-TT via the NEF.

# 1		Discussion

It is proposed to agree the following solution for inclusion in TR 23.700-20.

*** Start Change ***

## 6.X	Solution #X: NW-TT capability reporting and configuration

### 6.X.1	Introduction

This solution addresses the general part of Key Issue #3 summarized as follows:

This key issue is about enhancing the NEF (exposure) framework towards AF so that NEF can expose network capabilities to support Time Sensitive Communication.

The Rel-16 architecture has the following principles:

-	NW-TT capabilities of a TSN 5GS bridge are reported from NW-TT to TSN AF (as part of a NW-TT PMIC or a BMIC) via the PCF by “hijacking” a random PDU Session.

-	NW-TT configuration is provided from TSN AF to NW-TT via the PCF (as part of NW-TT PMIC or a BMIC) by “hijacking” a random PDU Session (the latter being indirectly selected by the TSN AF via the DS-TT MAC address).

-	The initial DS-TT configuration is sent via the PCF to the TSN AF using a preconfigured AF over a preconfigured AF session.

Some of the shortcomings of the current architecture include the following:

-	Use of a preconfigured AF and preconfigured AF session.

-	Transfer of non-PDU Session information inside the 5GS by misusing PDU Session-related procedures.

-	Impossibility to configure the 5GS bridge before at least one PDU Session has been established.

This solution proposes that capability reporting from NW-TT to TSN AF, as well as configuration from TSN AF to NW-TT be sent via the NEF using new NEF and SMF services. The information that is NW-TT specific and can be exchanged between NW-TT and TSN AF includes the following information:

-	NW-TT Port Management Information Container (NW-TT PMIC),

-	Bridge Management Information Container (BMIC).

DS-TT Port Information Container (DS-TT PMIC) is also exceptionally conveyed to the TSN AF via the NEF when there is no AF session established for this PDU Session (typically at PDU Session establishment). This allows to circumvent the use of a preconfigured AF and a preconfigured AF session.

The new service defined in this solution provide an alternative for exchange of NW-TT PMIPs and BMICs between the TSN AF and the NW-TT without relying on PDU Session-specific signalling. When this solution is used, the Rel-16 solution for exchange of NW-TT PMIPs and BMICs between the TSN AF and the NW-TT is not used.

### 6.X.2	Functional Description

#### 6.X.2.1	Nnef_TSNBridge service

##### 6.X.2.1.1	General

Nnef_TSNBridge is a new NEF service allowing the TSN AF to subscribe for TSN bridge and NW-TT port management information notifications and to push TSN bridge and NW-TT port management information configuration into the 5GS.

Table 6.X.2.1-1: NEF Services for NW-TT reporting and configuration

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nnef_TSNBridge | Subscribe | Subscribe/Notify | AF |
|  | Unsubscribe |  | AF |
|  | Notify |  | AF |
|  | Configure | Request/Response | AF |

##### 6.X.2.1.2	Nnef_TSNBridge_Subscribe operation

Service operation name: Nnef_TSNBridge_Subscribe

Description: The consumer subscribes for notifications of TSN bridge and port management information of a bridge identified via Bridge ID.

Inputs (required): Bridge ID.

Inputs (optional): None.

Outputs (required): None.

Outputs (optional): None.

##### 6.X.2.1.3	Nnef_TSNBridge_Unsubscribe operation

Service operation name: Nnef_TSNBridge_Unsubscribe

Description: The consumer unsubscribes for notifications of TSN bridge and port management information of a bridge identified via Bridge ID.

Inputs (required): Bridge ID.

Inputs (optional): None.

Outputs (required): None.

Outputs (optional): None.

##### 6.X.2.1.4	Nnef_TSNBridge_Notify operation

Service operation name: Nnef_TSNBridge_Notify

Description: The producer notifies the consumer with TSN bridge or port management information of a bridge identified via Bridge ID.

Inputs (required): None.

Inputs (optional): None.

Outputs (required): Bridge ID.

Outputs (optional): [NW-TT/DS-TT port number(s), Port Management Information Container(s)], [Bridge Management Information Container].

##### 6.X.2.1.5	Nnef_TSNBridge_Configure operation

Service operation name: Nnef_TSNBridge_Configure

Description: The consumer pushes TSN bridge and port management information into the producer for a bridge identified via Bridge ID.

Inputs (required): Bridge ID.

Inputs (optional): [NW-TT port number(s), Port Management Information Container(s)], [Bridge Management Information Container]

Outputs (required): None.

Outputs (optional): [NW-TT port number(s), Port Management Information Container(s)], [Bridge Management Information Container].

#### 6.X.2.2	Nsmf_TSNBridge service

##### 6.X.2.2.1	General

Nsmf_TSNBridge is a new SMF service allowing the TSN AF to subscribe for TSN bridge and NW-TT port management information notifications and to push TSN bridge and NW-TT port management information configuration into the 5GS.

Table 6.X.2.2.1-1: SMF Services for NW-TT reporting and configuration

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nsmf_TSNBridge | Subscribe | Subscribe/Notify | NEF |
|  | Unsubscribe |  | NEF |
|  | Notify |  | NEF |
|  | Configure | Request/Response | NEF |

##### 6.X.2.2.2	Nsmf_TSNBridge_Subscribe operation

Service operation name: Nsmf_TSNBridge_Subscribe

Description: The consumer subscribes for notifications of TSN bridge and port management information of a bridge identified via Bridge ID.

Inputs (required): Bridge ID.

Inputs (optional): None.

Outputs (required): None.

Outputs (optional): None.

##### 6.X.2.2.3	Nsmf_TSNBridge_Unsubscribe operation

Service operation name: Nsmf_TSNBridge_Unsubscribe

Description: The consumer unsubscribes for notifications of TSN bridge and port management information of a bridge identified via Bridge ID.

Inputs (required): Bridge ID.

Inputs (optional): None.

Outputs (required): None.

Outputs (optional): None.

##### 6.X.2.2.4	Nsmf_TSNBridge_Notify operation

Service operation name: Nsmf_TSNBridge_Notify

Description: The producer notifies the consumer with TSN bridge and port management information of a bridge identified via Bridge ID.

Inputs (required): None.

Inputs (optional): None.

Outputs (required): Bridge ID.

Outputs (optional): [NW-TT/DS-TT port number(s), Port Management Information Container(s)], [Bridge Management Information Container].

##### 6.X.2.2.5	Nsmf_TSNBridge_Configure operation

Service operation name: Nsmf_TSNBridge_Configure

Description: The consumer pushes TSN bridge and/or port management information configuration into the producer for a bridge identified via Bridge ID.

Inputs (required): Bridge ID.

Inputs (optional): [NW-TT port number(s), Port Management Information Container(s)], [Bridge Management Information Container].

Outputs (required): None.

Outputs (optional): [NW-TT port number(s), Port Management Information Container(s)], [Bridge Management Information Container].

### 6.X.3	Procedures

#### 6.X.3.1	General

TSN bridge and NW-TT port management information reporting is described in clause 6.X.3.2.

TSN bridge and NW-TT bridge and port management configuration is described in clause 6.X.3.3.

SMF discovery and selection is described in clause 6.X.3.4.

#### 6.X.3.2	5GS TSN bridge and port management information reporting

When integration with TSN applies the AF may initiate the 5GS TSN bridge and port management information reporting procedure for a 5GS TSN bridge identified via its Bridge ID.

Figure 6.X.3.2-1: 5GS TSN bridge and port management information reporting operations

1.	The AF subscribes for notifications for a specific 5GS TSN bridge using the Nnef_TSNBridge_Subscribe request message including the Bridge ID identifying the 5GS TSN bridge.

2.	The NEF performs SMF discovery using the Bridge ID parameter and subscribes for notifications using the Nsmf_TSNBridge_Subscribe request message including the Bridge ID identifying the 5GS TSN bridge.

3.	Upon PDU Session establishment the UE provides the DS-TT MAC address, UE-DS-TT Residence Time and a Port Management Information Container. As part of the PDU Session establishment procedure the UPF assigns a port number for the DS-TT Ethernet port. The SMF forwards the DS-TT MAC address, DS-TT Residence time, Port Management Information Container and related DS-TT port number to the NEF using the Nsmf_TSNBridge_Notify message. The NEF forwards the DS-TT MAC address, DS-TT Residence time, Port Management Information Container and related DS-TT port number to the AF using the Nnef_TSNBridge_Notify message.

NOTE:	For subsequent 5GS TSN port management information reporting from the DS-TT the SMF uses the PCF path i.e. uses the SMF initiated SM Policy Association Modification procedure described in clause 4.16.5.1. The reason for using the NEF path for the initial request is because at that point there is no association between the PCF and the AF.

4.	As part of the N4 node level procedure described in clause 4.4.3 the UPF may provide a Bridge Management Information Container and/or Port Management Information Container(s) and related port number(s) for NW-TT Ethernet port(s) for a specific Bridge ID. The SMF transparently forwards the Bridge Management information Container and/or Port Management Information Container(s) and related port number(s) for NW-TT Ethernet port(s) to the NEF using the Nsmf_TSNBridge_Notify message. The NEF forwards the Bridge Management Information Container and/or Port Management Information Container(s) and related port number(s) for NW-TT Ethernet port(s) to the AF using the Nnef_TSNBridge_Notify message.

#### 6.X.3.3.1	5GS TSN bridge and port management information configuration

When integration with TSN applies the AF may initiate the 5GS TSN bridge and port management information configuration procedure for a 5GS TSN bridge identified via its Bridge ID.

Figure 6.X.3.3-1: 5GS TSN bridge and port management information configuration operations

1.	AF triggers the Nnef_TSNBridge_Configure request message via the NEF including the Bridge ID and either the Bridge Management Information Container or the Port Management Information Container(s) and the related port number(s) for NW-TT Ethernet port(s).

2.	The NEF determines the SMF based on the Bridge ID received in the AF request and transparently transports the received Bridge Management Information Container or the Port Management information Container and the related port number(s) for NW-TT Ethernet port(s) to the SMF using the Nsmf_TSNBridge_Configure request message.

3.	The SMF transports the received Bridge Management information Container or Port Management information Container(s) and the related port number(s) for NW-TT Ethernet port(s) to the UPF in N4 Session Modification Request message.

4-5.	The outcome of the configuration operation is reported back as part of a Bridge Management Information Container or Port Management Information Container(s) and related port number(s) for NW-TT Ethernet port(s) to the AF using the Nsmf_TSNBridge_Configure and Nnef_TSNBridge_Configure response messages.

NOTE:	5GS TSN Bridge configuration information targeting a specific DS-TT port is delivered using the PCF initiated SM Policy Association Modification procedure described in clause 4.16.5.2.

#### 6.X.3.4	SMF discovery and selection

The SMF(s) handling the UPF of a specific 5GS bridge needs to register with the NRF using the Bridge ID of the 5GS bridge.

The AF is preconfigured with the Bridge ID of the 5GS TSN bridge.

The NEF queries the NRF using the Bridge ID of the 5GS bridge to discover SMF instance(s) that handle the UPF of a specific 5GS bridge.

### 6.X.4	Impacts on existing entities and interfaces

AF:

-	Support for the new Nnef_TSNBridge service.

-	The AF is preconfigured with the Bridge ID of the 5GS TSN bridge.

NEF:

-	Support for the new Nnef_TSNBridge and Nsmf_TSNBridge services.

-	Support for SMF discovery by querying the NRF.

SMF:

-	Support for the new Nsmf_TSNBridge service.

-	Support for SMF discovery by registering the Bridge ID with the NRF.

NRF:

-	Support for SMF discovery with Bridge ID as the search key.

*** End Change ***