# 3GPP TSG-WG SA2 Meeting #139E e-meeting	S2-2004643

# Elbonia, June 01 – 12, 2020 	(revision of S2-2003992)

Source:	Huawei, HiSilicon, Samsung

Title:	KI #2, Sol #3: Update to clarify the procedures and impacts

Document for:	Approval

Agenda Item:	8.5

Work Item / Release:	FS_IIoT / Rel-17

Abstract of the contribution: This contribution proposes to resolve editor’s notes in solution #3.

Discussion

For solution #3, there are two editor’s notes to be resolved.

Editor's note:	For the second bullet will be revisited with detailed procedures.

Editor's note:	This clause lists impacts to existing entities and interfaces.

The AF calculates the bridge delay using the UE-DS-TT Residence Time and pre-configured delay between UE and UPF/NW-TT. The SMF sets the DL TSCAI Burst Arrival Time as the sum of TSN QoS Burst Arrival Time and CN PDB as described in clause 5.7.3.4 of TS23.501. When AF provides the BAT for DL stream for PDU Session2, it can calculates it with the BAT of UE-UE stream and bridge delay for PDU Session 1. So the SMF can provide the DL TSCAI Burst Arrival Time as defined in clause 5.27.2 of TS 23.501.

Proposal 1: AF calculates the BAT of DL stream for PDU Session 2 with the BAT of UE-UE TSC Stream and bridge delay for PDU Session 1, then the SMF can provide the DL TSCAI Burst Arrival Time as defined in clause 5.27.2 of TS 23.501.

Proposal 2: Clarify that the SMF creates the local switch forwarding rule on the UPF to support UE-UE TSC Stream when the UE’s PDU Session supports 5GLAN and the SMF receives the TSC QoS Flow requests.

Proposal

This paper proposes to resolve editor’s notes in solution #3.

* * * Start of Change * * * *

6.3	Solution #3 UE-UE TSC communication with VN group

6.3.1	Introduction

This solution address Key Issue #2 "UE-UE TSC communication".

VN group mechanism as defined in Rel-16 clause 5.29 is reused to achieve the UE-UE TSC communication.

The UE-UE TSC communication with VN group is applied for that both PDU Sessions of UEs that served by the same UPF and within the same 5G VN group.

Editor's note: It is FFS how the solution works when the SMFs are different, while 5G VN uses a single SMF

6.3.2	Functional Description

This solution is based on the following principles:

-	The 5GS knows the UE/DS-TT pairs which can perform the UE-UE TSC communication under condition that UEs/DS-TT pairs belonging to the same 5G VN.

-	The SMF and optionally AF can know the VN group information as defined in TS 23.501 [2].

-	When the SMF knows the two UEs/DS-TTs are belonging to the same VN group and served by the same (anchor) UPF, SMF will pair them and report the port pair numbers to the TSN AF. Bridge delay of this UE-UE port pair is the sum of the two PDU Sessions associated to this port pair and is calculated by the TSN AF.

Editor's note: It is FFS how the TSN AF uses the information about VN group membership

-	When the TSN AF receives the traffic forwarding information from the CNC, it determines the flow for the UE-UE communication and divides the flow into two PDU Sessions, one for UL and one for DL. Then UL and DL streams are respectively triggered by the TSN AF to be established.

-	When the SMF receives PCC rules for the UL or DL stream for a TSC PDU Session that supports 5GLAN, it determines that the flow is for UE-UE communication and creates the forwarding rule on the UPF to be local switched as defined in clause 5.8.2.13 in TS 23.501 [2].

Editor's note: It is FFS based on what criteria and conditions the SMF derives forwarding rules from PCC rules.

Editor's note: It is FFS how the SMF determines whether forwarding rules need to be derived for a given flow or it is already in place

-	TSCAI for UL and DL streams are respectively calculated by the SMF during UL and DL streams establishment as defined in TS 23.501 [2].

6.3.3	Procedures

For the scenario generating TSC stream from UE1 to UE2, procedures for bridge delay reporting and traffic forwarding rule establishment is as figure 6.3.3-1 illustrates.

Figure 6.3.3-1 Procedure for bridge delay reporting and traffic forwarding rule establishment

1.	PDU Session Establishment for UE1 as specified in clause 4.3.2.2.1 of TS 23.502 [3].

2.	Bridge Delay reporting for UE1 as specified in clause 4.16.5.1 of TS 23.502 [3].

3.	PDU Session Establishment for UE2 as specified in clause 4.3.2.2.1 of TS 23.502 [3]. Optionally the SMF indicates the port number of UE1 PDU Session as part of port pair to the AF if the SMF determines the PDU Sessions of UE1 and UE2 are in the same 5G VN Group.

4.	Bridge Delay reporting for UE2 as specified in clause 4.16.5.1 of TS 23.502 [3].

5.	The AF determines the port numbers associates to PDU Sessions of UE1 and UE2 are port pairs based on 5G VN Group information, or optionally according to the port pair indication in step 3. Then the AF calculates and reports the bridge delay between UE1 and UE2 to the TSN CP.

6.	When the TSN CP creates a TSN stream, the TSN CP informs the AF the Traffic Forwarding information as specified in clause 5.28.2 of TS 23.501 [2].

7-8. 	The AF determines that it is UE-UE TSC according to the ingress port number and egress port number provided in Traffic Forwarding information as they all associated to PDU sessions. Then the AF triggers the 5GC to generate a UL stream for PDU Session of UE1 and a DL stream for PDU Session of UE2 according to the PDU Session Modification Procedure as specified in clause 4.3.3.2 of TS 23.502 [3]. AF calculates the TSN QoS BAT of DL stream for PDU Session of UE2 as the sum of the TSN QoS BAT of UE-UE TSC and the bridge delay for PDU Session of UE1.

During step 8 of PDU Session Modification procedure, the SMF configures the UPF to handle the UL and DL stream via local switch as defined in clause 5.8.2.13 in TS23.501 since the PDU session supports 5GLAN and TSN AF indicates that this is for UE-UE TSC.

If Port Management Container with scheduling information is provided to the SMF in step 7 or step 8, the SMF will respectively calculate the TSCAI for the UL and DL stream. The SMF needs to perform TSCAI calculation considering two associated PDU sessions like the following.

-	TSCAI Burst arrival time for PDU session for UE1 for UL stream is calculated the same as described in clause 5.27.2 of TS 23.501 [2].

-	TSCAI Burst arrival time for PDU session 2 for DL stream is calculated the same as described in clause 5.27.2 of TS 23.501 [2].

Editor's note: It is FFS how local switching delay is considered in the delay model

6.3.4	Impacts on existing services and interfaces

SMF:

- In case the AF does not maintain the 5G VN Group information, the SMF determines the UE port pair base on 5G VN Group information.

AF:

- In case the AF maintains the 5G VN Group information, the AF determines the UE port pair base on 5G VN Group information.

- Need to perform TSCAI calculation considering two associated PDU sessions and indicates the UE-UE TSC to 5GS.

Npcf_SMPolicyControl_Update request:

- In case the AF does not maintain the 5G VN Group information, the SMF indicates PCF the (DS-TT, DS-TT) port pair.

Npcf_PolicyAuthorization_Notify:

- In case the AF does not maintain the VN Group information, the PCF indicates AF the (DS-TT, DS-TT) port pair.

* * * End of Change * * * *