SA WG2 Meeting #140-e	S2-2004857

August 19th -September 2nd 2020, Online	(revision of S2-20xxxx)

Source:	Nokia, Nokia Shanghai Bell, Telecom Italia….

Title:	KI#6: Sol. Update: Solution 6.28 update

Document for:	Approval

Agenda Item:	8.4

Work Item / Release:	FS_eNS_Ph2

Abstract of the contribution: xxx

# Discussion

Some editor's notes require resolution

# Proposal

The following clarifications are proposed to be added to TR 23.700-40

## 6.28	Solution #28: Compatibility Class based support for simultaneous use of the network slice

## 6.28.1	Introduction

This solution, which addresses KI#6, allows the system to enforce policies on Simultaneous use of Network Slices. It is based on network side awareness of the Classes of Network Slices a S-NSSAI is compatible with (based on subscription information) and the provision of this information to the UE as part of the network slicing configuration.

The benefit of providing the compatibility class information to the UE is that this is a policy of compatibility between Network Slices that the UE needs to be aware of, so it requests a set of network slices in compatible classes without trial and errors and thus it is possible minimize the failed attempts to access a desired and compatible Network Slices Set.

### 6.28.2	High-level Description

The S-NSSAI subscription information is augmented with a "simultaneous use information" defining the compatibility with other Network Slices.

The Compatibility is decided by the HPLMN as it is a policy of compatibility of usage irrespective of deployment constraint or choices (which might also exists and be additional constraints – e.g. as per KI#7 we may have additional constraints), hence it is part of the subscription data so it can be enforced and provided to the UE by the serving AMF in any PLMN when determining the allowed NSSAI.

The simultaneous use information is defined by a compatibility class, which may be operator defined or standardised.

The following compatibility classes are proposed to be standardised:

Table 6.28.2-1: Standardised Compatibility Classes

| Compatibility Class | Rule |
| --- | --- |
| 0 | S-NSSAI can be used alongside S-NSSAIs with same SST field value |
| 1 | S-NSSAI can be used alongside S-NSSAIs with same SD field value |
| 2 | S-NSSAI cannot be used alongside any other S-NSSAI |

The need of standardization is based on the standardised compatibility classed defined in GSMA NG.116 and these need to be provided to the UE and the AMF by the UDM (both in roaming or in non-roaming case).

If there is no such simultaneous use information associated to a S-NSSAI, then the PLMN assumes this S-NSSAI can be included alongside any other S-NSSAI in an Allowed NSSAI, when the S-NSSAI can be supported in the current TA of the UE.

The S-NSSAIs marked as default S-NSSAI shall be mutually compatible with each other. The reason is that these are the ones which shall be provided together in the Allowed NSSAI to the UE when the UE indicates no S-NSSAI.

A UE shall signal its support of "incompatible network slices handling" feature in the Registration Request message. If a UE does not support this feature, it shall only be configured with Network Slices which can be used simultaneously.

A UE shall not include in the Requested NSSAI S-NSSAIs that map to HPLMN S-NSSAIs which are incompatible with each other.

If a UE attempts to register with a set of incompatible S-NSSAIs, and it supports the "incompatible network slices handling" feature, it shall be configured with a new Configured NSSAI, Mapping to HPLMN S-NSSAIs, HPLMN S-NSSAIs and related "simultaneous use information". The NSSF/AMF provide an Allowed NSSAI including only compatible S-NSSAIs based on their policies (e.g. any default S-NSSA is or the broadest set of compatible slices including the Default S-NSSAIs), and the other S-NSSAIs are rejected with cause code "incompatible S-NSSAIs with the S-NSSAIs in the Allowed NSSAI". The UE may also be requested to immediately re-register using the new configuration information. Also, the UE nonetheless may autonomously re-register with the new configuration even if the Network does not request it, so the UE can request S-NSSAIs more adequate to run the applications the user is more interested in.

If a UE attempts to register with a set of incompatible S-NSSAIs, and it does not support the "incompatible network slices handling" feature, it shall be configured with a new Configured NSSAI, Mapping to HPLMN S-NSSAIs, HPLMN S-NSSAIs including only S-NSSAIs compatible with each other. The NSSF/AMF provide an Allowed NSSAI including only compatible S-NSSAIs based on their policies (e.g. any default S-NSSAIs or the broadest set of compatible slices including the Default S-NSSAIs). The UE may also be requested to immediately re-register using the new configuration information. Also, the UE nonetheless may autonomously re-register with the new configuration even if the Network does not request it, so the UE can request S-NSSAIs more adequate to run the applications the user is more interested in.

The HPLMN shall provide to a V-PLMN only Subscribed S-NSSAIs which are compatible with each other when the V-PLMN, based on roaming agreement, is known to not support the handling of mutually incompatible slices. These S-NSSAI(s) shall be provided with no compatibility class information to these V-PLMNs.

### 6.28.3	Procedures

The registration procedure in TS 23.502 [6] clause 4.2.2.2.2 is impacted as follows:

Figure 6.28.3-1

1.	UE includes indication it supports the "incompatible network slices handling" feature and is shall include only compatible S-NSSAIs in the requested NSSAI.

2.	The AMF selection shall include checking the compatibility of the S-NSSAIs in the Requested NSSAI. The Compatibility class information is provided to the NSSF alongside the Subscribed S-NSSAIs.

21.	Registration accept may include cause code "incompatible S-NSSAIs with the S-NSSAIs in the Allowed NSSAI" in the Rejected S-NSSAIs if applicable.

The Registration with AMF re-allocation procedure in TS 23.502 [6] in clause 4.2.2.2.3 is impacted as follows (if the PLMN supports handling of incompatible S-NSSAIs).

Figure 6.28.3-2

3a.	The slice selection subscription data includes the compatibility class information, if applicable.

4a.	The AMF includes the "Compatibility class information" associated to any S-NSSAI in the subscription data that includes such information

7a/7b.	If all the S-NSSAIs in the requested NSSAI are incompatible with the S-NSSAIs in the current AMF and they are served by another AMF, and the operator knows there is no routability between the source and target AMF, then the UE is de-registered with a request to immediately register instead of executing any of steps 7a/b.

### 6.28.4	Impacts on services, entities and interfaces

UE: support of "incompatible network slices handling " as outlined.

AMF: support of "incompatible network slices handling" feature as outlined.

UDM: support of new compatibility information per UE in subscription data as outlined.