3GPP TSG SA WG2 Meeting #140-e		S2-2005689

Electronic, 19 Aug – 02 Sep, 2020

Title:	LS on System support for Multi-USIM devices

Response to:

Release:	Rel-17

Work Item:	FS_MUSIM

Source:	SA2

To:	RAN2, RAN3

Cc:	-

Contact person:	Sašo Stojanovski

saso stojanovski intel com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

SA2 would like to inform RAN2 and RAN3 that they have progressed the study on FS_MUSIM (TR 23.761) and have agreed several interim conclusions on all three key issues. The interim conclusions list a set of principles that SA2 will consider for further study.

SA2 point out that these interim conclusions are subject to further revision taking into account feedback received from other WGs.

The table below lists the principles that are considered for further study and have RAN impact or require RAN feedback. The table also contains the applicability to key issues in TR 23.761, as well as questions to RAN WGs.

| Solution principle for further study in SA2 | Applicability to Key Issues | Question to RAN WGs |
| --- | --- | --- |
| Paging Cause (as in Solution #1) shall be considered for further study for both EPS and 5GS, pending confirmation from SA3 about any privacy issues with sending the Paging Cause as cleartext. | KI#1. Basic functionality | Q1: Please confirm feasibility of sending a Paging Cause in [Uu] Paging message for both EPS and 5GS. [RAN2, RAN3] |
| Busy indication (as in Solution #3) should be considered for further study for 5GS, pending confirmation from RAN WGs about feasibility. | KI#1. Nice to have. | Q2: Please confirm feasibility for sending an RRC Busy Indication when the UE determines that it cannot respond to the paging. Please provide any feedback about complexity (if applicable). [RAN2, RAN3] |
| UE-requested 5G-GUTI reassignment using the Mobility Registration Update procedure (as in Solutions #14 and #20) shall be considered for further study for 5GS. It is assumed that UE does not need to provide any assistance information to 5GC because the probability that the newly assigned 5G-GUTI will result in collisions is minimal. | KI#2. Applicable to 5GS-5GS and 5GS-EPS scenarios. Executed in 5GS only. | Q3: While the 5G-GUTI reassignment principle has no RAN impact, SA2 welcomes any RAN2 feedback on the assumption that no assistance information is needed from the UE in order to minimize the probability that the new 5G-GUTI will also result in PO collisions. [RAN2] |
| UE Identity Index Value offset value negotiated between UE and MME (as in solution #16) shall be considered for further study. | KI#2. Applicable to EPS-EPS scenarios. Executed in EPS only. | Q4: Please confirm feasibility of using a UE Identity Index Value offset for calculation of POs. [RAN2] |
| RRC-based leaving.and returning (as in all KI#2 solutions) with the following assumptions: - the leaving is initially considered as “short absence duration” i.e. the UE does not need to indicate the time duration of the absence. If the UE does not return within a network-controlled time interval, the “short absence duration” is automatically converted into a “long absence duration”. The RRC states corresponding to “short” and “long” duration are to be determined by RAN WGs. - The UE can use the negotiated absence to perform a MO procedure (e.g. periodic mobility registration, keep-alive message, etc.) or a MT procedure (e.g. pick-up an SMS, inspect a MT service invite, respond to a network-initiated C-plane procedure, etc.) in the other network. - It is assumed that the RRC request for leaving is explicitly acknowledged by the network. However, in the absence of network acknowledgement the UE can still proceed with leaving. | KI#1, KI#2. For applicability to KI#1 refer to Sol#1 | Q5: Please confirm feasibility of the RRC-based leaving and returning procedure in 5GS. [RAN2, RAN3] Q6: SA2 expects that RRC-based leaving would also apply to 5GS/E-UTRA (Option 5), but there were concerns that this may not be in scope of the RAN work item. Please confirm whether changes to 5GS/E-UTRA (Option 5) to support this functionality are in scope of the RAN work item. [RAN2, RAN3] |

SA2 would also like to point out that TR 23.761 also contains several solutions to KI#2 that have RAN impact and could be categorized into one of the following principles:

-	Paging on consecutive POs as in Sol#18. For this solution SA2 was not clear whether the network requires an explicit MUSIM capability so that a different paging strategy can be applied to MUSIM- and non-MUSIM capable devices.

-	Implementation-based solution to address overlapping POs (Sol#19).

-	Access Stratum-based solution with scheduling gap (Sol#21).

Q7: SA2 would like to ask RAN2 and RAN3 to take these solution into consideration and provide any feedback or recommendation on the applicability of these solutions, or any other Access Stratum-based solutions for KI#2.

# 2	Actions

To RAN2, RAN3

ACTION: 	SA2 kindly asks RAN2 and RAN3 to take into consideration the information above and provide answers to questions Q1, Q2, Q5, Q6 and Q7.

To RAN2

ACTION: 	In addition, SA2 kindly asks RAN2 to provide answers to questions Q3 and Q4.

# 3	Dates of next TSG SA WG 2 meetings

SA2#141E		12-23 October				Electronic meeting

SA2#142E		16-20 November			Electronic meeting