3GPP TSG-WG SA2 Meeting #140E	S2-2004919

19 August - 2 September, 2020, Elbonia

Source:	Ericsson

Title:	KI #3A, Sol #5: Update to clarify monitoring events

Document for:	Approval

Agenda Item:	8.5

Work Item / Release:	FS_IIoT/ Rel-17

Abstract of the contribution: This contribution proposes changes to Solution #5 in TR 23.700-20 related to the support for Determinists QoS and proposes to remove some Editor’s Notes.

# Discussion

The existing solution#5 defines that the AF may learn Determinists QoS Capability information to aid the AF in formulating a request for TSC QoS that will be acceptable to the 5GS. The Determinists QoS Capability information includes:

-	5GS Support for TSC.

-	5GS Support for providing TSC assistance information.

-	Support for UE and UPF Hold and Forward Buffers.

-	Minimum 5GS Delay supported.

Then, the same solution #5 defines a new procedure for “5GS Capability exposure” triggered by the AF to NEF and says that NEF obtains 5GS capability information using services provided by other NFs. Here we assume that 5GS capability information is the same as the Determinists QoS Capability information defined above.

In this document two points are discussed:

- Which the Determinists QoS Capability information contains and

- How NEF learns the Determinists QoS Capability information to be exposed to the AF.

1- Which the Deterministic QoS Capability contains:

The Determinists QoS capability is proposed to be defined as “the capability to deliver a traffic flow bound to a maximum and minimum delay”. Then, the network implements some mechanisms to ensure that the delay remains bound to these limits, these mechanisms are both the support for TSC assistance information or Hold and Forward buffer. The TSC assistance information is not related to the support for Deterministic QoS but rather for optimization of scheduling in RAN.

Proposal 1: The Determinists QoS Capability information includes:

-	5GS Support for Determinists QoS (yes/no).

-	Minimum and maximum 5GS Delay supported (provided if 5GS Support for Deterministic QoS set to “yes”).

If the AF receives the indication that determinists QoS capability is supported, then the AF may provide TSN assistance information.

2- How NEF learns the Determinists QoS Capability information to be exposed to the AF:

Even if solution#5 defines that NEF learns the 5GS capability from services provided by other NFs, which these services are is not defined as new services nor specified in 23.502 so far. Therefore, there is no solution defined so far.

TS 23.501 defines in clause 5.27 that the 5G System provides the Bridge delay for each port pair and traffic class of the 5GS bridge. The 5GS Bridge delay includes:

- the UE-DS-TT Residence time provided by the PCF at PDU session establishment and

- the per traffic class minimum and maximum delays between the UE and the UPF/NW-TT that is configured in the TSN AF.

Based on the above, it is proposed to follow the same approach for both TSN AF and non TSN AF.

Proposal 2: When the non TSN AF is within the operator’s domain, the AF is preconfigured with the minimum and maximum delay between the UE and the UPF/NW-TT per Type of Service. The AF learns the UE-DS TT Residence time from the PCF. This a no TSN AF is interested to learn 5GS delay. When the AF is outside the operator´s domain, the NEF is preconfigured with the minimum and maximum delay between the UE and the UPF/NW-TT per Type of Service instead.

Other aspects:

- Given that already existing monitoring events have been listed in TS 23.502 and TS 23.503, and since the text of the solution explains: “Additional events and PCRTs may be considered in the normative phase.”, then ENs on the need to list events and NFs that detect them have been removed.

- Hold and Forward buffering EN is moved to clause 6.5.3 and modified to remove “and how”. Furthermore, we add a Note indicating that: If Hold and forward buffers are applied for IP PDU Sessions and Ethernet PDU sessions when CNC is not present, then how to apply them is up to implementation.

# Proposal

It is proposed to add the following change to solution #5 in TR 23.700-20.

* * * * Start changes * * * *

## 6.5	Solution #5: Deterministic QoS for Native 5GS

### 6.5.1	Introduction

For Key Issue #3A: Exposure of deterministic QoS, a solution is needed that allows "any AF that has knowledge of deterministic application requirements" to "be able to request TSC services from the 5GS and as authorized, be notified of pertinent network events." This solution provides a mechanism to do that whereby the AF may learn 5GS capabilities to support TSC, request QoS with specified requirements and supply information that can be used to derive TSCAI for 5GS QoS flows. The solution is applicable for all PDU Session Types (IP, Ethernet and unstructured), and is independent of DN TSN protocols (IEEE 802.1Qbv [8], IEEE 802.1Qci, IEEE 802.1Qcc [7]).

To enable the above capabilities, this solution proposes to enhance External Exposure of Network Capability (see TS 23.501 [2], clause 5.20), specifically for Provisioning and Monitoring capabilities, to notify for the support of the capability to provide deterministic QoS. To reuse the "Setting up an AF session with required QoS" procedure in TS 23.502 [3], clause 4.15.6.6 is proposed for setting QoS for TSC.

To cover a wide range of possible QoS parameters for TSC applications, the QoS Reference currently used in AF QoS requests is supplemented with requirements for specific parameters. When a requirement for a specific parameter is provided in the AF request, the PCF may, for the referenced QoS profile, dynamically replace default values with values that meet the specific requirement provided by the AF.

### 6.5.2	Functional Description

The following capabilities are proposed using QoS request from AF:

-	The AF requests TSC QoS and provides traffic pattern as assistance parameters via procedure “Setting up an AF session with required QoS procedure”, supplying the NEF. with requirements for one or more of a 5GS delay, Guaranteed Flow Bit Rate, Flow Direction, Burst Arrival Time at UE (uplink) or UPF (downlink), Burst Size, Burst Periodicity, Survival Time, and a Timing Domain. The AF also provides a Traffic Description, Target UE PDU session Identification, AF Identification, and a QoS Reference. AF may also request subscription to events defined in Table 6.1.3.18-1, TS 23.503 [12]. Example of relevant events are Resource allocation outcome, QoS targets can no longer (or can again) be fulfilled, QoS Monitoring parameters. Service Data Flow deactivation (when the QoS flow is removed); and then the PCF may decide to subscribe to PCRTs in the SMF example PCRTs that may be used as defined in, Table 6.1.3.5-1, TS 23.503 [12] such as GFBR of the QoS Flow can no longer (or can again) be guaranteed. Additional events and PCRTs may be considered in the normative phase. Note GPSI may be applied to identify the individual UE in a manner similar to that used for AF influence on Traffic Routing (see TS 23.501 [2], clause 5.6.7). The NEF authorizes the AF request and forwards the request to the PCF. Then:

-	The PCF may, according to PCC rule authorization, choose a 5QI based on the QoS Reference and dynamically set the PDB and/or MDBV according to the received 5GS Delay and Burst Size requirements. As authorized, AF specified parameter values are used to over-ride default values, for the 5QI. If an AF request for a parameter value exceeds an authorization, the PCF may assign the highest authorized value. The GFBR for QoS Flow is set according to the Guaranteed Flow Bit Rate.

-	The 5GS may set the TSCAI Burst Arrival Time (BAT), Periodicity and Survival Time according to the requested Periodicity, Burst Arrival Time and Survival Time received from the AF.

-	If Timing Domain information was supplied by the AF and the 5GS uses the same Timing Domain, then no adjustment is needed for deriving TSCAI information. If the Timing Domain provided by AF is different from 5GS Timing Domain and the SMF has clock drift information for that Timing Domain (i.e. clock drift between 5G timing and AF supplied Timing Domain), then 5GS may adjust the TSCAI information so that it reflects the 5GS Clock. If Timing Domain information is not provided or the SMF does not have synchronization information for a requested Timing Domain, then the TSCAI information will be used without adjustment.

-	If the AF provides burst spread, the 5GS will provide burst spread as part of TSCAI to the NG-RAN.

-	When subscribed events are met the PCF reports them to the NEF, and NEF will notify the AF. The subscribed event may be reported once the SMF reports a PCRT to the PCF.

-	Following the procedure in clause 4.16.5.1, TS 23.502 [3], PCF reports the events to the NEF, and NEF notifies the AF.

-	The 5GS exposes the following Deterministic QoS Capability information to aid the AF in formulating a request for TSC QoS that will be acceptable to the 5GS:

-	5GS Support for Determinists QoS (yes/no).

-	Minimum and maximum 5GS Delay supported (provided if 5GS Support for Deterministic QoS set to “yes”).

.

-	The 5GS provides the response to a request from an AF for TSC QoS.

The following capabilities are proposed for TSC connectivity monitoring:

-	The AF sends request to NEF to monitor the status of TSC connectivity via a set of events for a UE or group of UEs.

-	If the request is authorized, the NEF subscribes the required monitoring events to suitable NFs (e.g. AMF, SMF).

-	When the NF detects the subscribed events occurs, the NF sends the event notification to NEF. The NEF forwards to the AF the received reporting event.

To support notification, the additional events may be added to the list of monitoring capabilities specified for the NEF in TS 23.502 [3] (see clause 4.15.3.1). Existing events in that list that are relevant for TSC and can be subscribed to are: PDU Session Status (detected by SMF), Communication failure (detected by AMF), and Loss of Connectivity (detected by AMF).

### 6.5.3	Procedures

An overall procedure for Native 5GS deterministic QoS is illustrated in Figure 6.5.3-1:

Figure 6.5.3-1: Procedure for Exposure the deterministic QoS

1.	PDU Session Establishment for UE1 as specified in clause 4.3.2.2.1 of TS 23.502 [3].  If the AF is located inside the operator’s domain, the PCF exposes the UE-DS-TT residence time to the AF as defined in TS 23.502 [3] and the AF is configured with the UE to UPF/NW-TT delay per Type of Service.

2.	If the AF is located outside the operator’s domain, the  NEF exposes Deterministic QoS Capability to AF, as shown in Figure 6.5.3-2. If the NEF learns the UE-DS-TT residence time from PCF and NEF is configured with the UE to UPF/NW-TT delay per Type of Service.

3.	Setting up an AF session with required QoS procedure specified in the TS 23.502 [3] clause 4.15.6.6. is executed with the following modifications:

-	The parameters described in clause 6.5.2 "Function Description" may be specified in the AF request to the NEF

-	The NEF may forward to the PCF the additional parameters received at the NEF, and the PCF derives the 5QI and sets the PCC Rule accordingly. The PCF includes the Time Domain to the PCC Rule.

-	The response from PCF to NEF, and NEF to AF contains additional information on the TSC resources allocated for the request.

4.	The PCF creates the PCC rule and provides to SMF. And PCF also includes the TSC traffic pattern information from NEF/AF.

5.	The SMF may calculate the TSCAI similar to TS 23.501 [2] clause 5.27.2, with the following modifications:

The SMF assigns the TSCAI with the Time Domain as received from the PCF to a QoS Flow. If the SMF receives clock drift information from UPF for that Timing Domain (i.e. clock drift between 5G timing and AF supplied Timing Domain), then SMF may adjust the TSCAI information of the QoS Flow so that it reflects the 5GS Clock.

6.	The SMF configures the U-plane and sends the N2 message with TSCAI to NG-RAN. The SMF distributes the parameters for Hold and Forward Buffering to the UE and UPF.

Editor's note:	How the parameters for H&F buffering are created and sent to the UE and UPF is FFS.

Editor's note:	Whether to apply Hold and forward buffers for IP PDU Sessions and Ethernet PDU sessions if CNC is not present is FFS.

NOTE 1:	If Hold and forward buffers are applied for IP PDU Sessions and Ethernet PDU sessions when CNC is not present, then how to apply them is up to implementation.

AF requested TSC QoS:

The following procedure is used to expose 5GS information to aid the AF in formulating a request for TSC QoS (step 3 of Figure 6. 5.3-1).

Figure 6.5.3-2: TSC Capability exposure towards external AF

The NEF obtains 5GS capability to support Determinists QoS for a PDU session using the PCF provided information and the local configuration in NEF. The AF receives 5GS bridge delay information.

- The AF requests the 5GS Capabilities to support Determinists QoS. The AF provides the UE MAC address or the GPSI and the Application Identifier (VIAPA).

- The NEF checks finds the PCF that serves the PDU session, then NEF requests PCF whether a UE-DS-TT residence time can be provided.

- The PCF provides the UE-DS-TT Residence time for the DS-TT port MAC address if available.

- If no DS-TT Residence time was provided, the NEF determines that the PDU session does not support Determinists QoS. If the DS-TT Residence time is provided the NEF determines that the PDU session supports Determinists QoS and provides to the AF the minimum and maximum delay for the Application Identifier.

NOTE 2: NEF aggregates UE-DS-TT residence time and per QoS configured UE to UPF/NW-TT delay to determine the delay to be provided to the AF.

TSC connectivity monitoring:

Figure 6.5.3-3: Procedure for TSC connectivity monitoring

1.	The AF sends Nnef_EventExposure_Subscribe Request to NEF to monitor the status of TSC connectivity via a set of events for a UE or group of UEs. The events are those listed in clause 4.15.3.1 of TS 23.502 [3].

Event Reporting Information defines the type of reporting requested (e.g. one-time reporting, periodic reporting or event-based reporting, for Monitoring Events).

2.	If the reporting event subscription is authorized by the NEF, the NEF sends the Subscribe Requests to the related NFs that detect the events, as described in TS 23.502 [3] procedures for subscription to these events.

3.	The NEF sends Nnef_EventExposure_Subscribe Response to AF as described in TS 23.502 [3] procedures for subscription to these events.

4.	The NFs detects the monitored event occurs and sends the event report to the NEF as described in TS 23.502 [3] procedures for subscription to these events.

5.	The NEF sends to the AF the notification of TSC communication monitoring event changed as described in TS 23.502 [3] procedures for subscription to these events.

### 6.5.4	Impacts on services, entities and interfaces

Impacts on existing entities and interfaces

The solution has the following impacts:

1.	An additional Nnef service is used to expose 5GS TSC capabilities to support Deterministic QoS

2.	Additional parameters are included in existing messages in the "AF session with required QoS procedure" specified in the TS 23.502 [3] clause 4.15.6.6.

3.	The PCF uses the additional AF supplied parameters to calculate a PCC rule

4.	"Burst Spread" is sent from the PCF to the SMF, which uses it to determine a Burst Spread TSCAI parameter.

5.	Time Domain is provided from PCF to the SMF, which is used by the SMF to associate the QoS Flow to a Time Domain. The solution assumes that the 5GS can be made aware of the timing in the AF supplied Timing Domain, this can be provided via Time Synchronization service as described in Solution #7.

Editor's note:	UE and UPF impacts are FFS.

AF:

-	Deliver TSC connectivity monitoring request message to NEF.

-	Deliver TSC QoS request with TSC QoS reference, TSC traffic pattern, and subscription to events to the PCF.

* * * * End changes * * * *