3GPP TSG-SA/WG2 Meeting #140E	S2-2004957

19August -02 September 2020, Electronic meeting

Source:	Ericsson

Title:	KI #1, Sol #12: Update to remove ENs and editorial updates

Document for:	Agreement

Agenda Item: 	8.3

Work Item / Release:	FS_enh_EC / Rel-17

Abstract of the contribution: This contribution proposes updates to Solution #12: DNS triggered re-anchoring related to some editorial issues and to answer the Editors Notes from the previous SA2 meeting

# 1	Introduction

This contribution proposes updates to Solution #12: DNS triggered re-anchoring to address the Editors Notes from the previous SA2 meeting and to correct for some editorial mistakes.

# 2	Discussion

This contribution proposes updates to Solution #12: “DNS triggered re-anchoring”. It removes the Editor’s Note and   corrects for some editorial mistakes.

We propose to remove this Editor’s Notes: “The procedure for the case when I-SMF is needed is FFS.”

This EN can be removed as it does not apply to the proposed scenario. In this solution the assumption is that SMF controls local PSA(s)/DNAI(s) where the session can be re-anchored. The SMF (aka LDNSR) selection of PSA for re-anchoring is triggered by DNS and limited to the PSAs/DNAIs that the SMF controls. The procedure used applies to re-anchoring of SSC#3 sessions without change of SMF. These aspects have been made explicit in the solution description.

# 3	Proposal

It is proposed to have the following changes in TR 23.748:

* * * * 1st Change * * * *

## 6.12	Solution #12: DNS triggered re-anchoring

### 6.12.1	Solution description

This solution addresses the Key Issue #1: Discovery of Edge Application Server (EAS). The UE is Edge Computing Service agnostic.

The proposed solution supports the “Distributed Anchor Point” connectivity model. It is assumed that at PDU session establishment, the SMF selects a central PSA but local PSA is available to the SMF and, and re-anchoring is used to transition to the “Distributed Anchor Point” model. The UE issues a DNS query related to an EC service. A new function in the SMF (aka LDNSR) is involved in the DNS communication of the UEs authorized for edge services (e.g. at session establishment the SMF configures the required steering in the UPF). The DNS discovery request is received by the SMF that authorizes the UE/service(domain) and checks if PSA relocation to the edge is needed and, if so, it initiates the re-anchoring procedure.

It is assumed that re-anchoring happens without change of SMF, where the SMF has full control over the re-anchoring. This is the case when the re-anchoring can be done using one of the following existing 5GC procedures for PSA change:

- SSC mode 3 with IPv6 Multi-homed PDU Session (clause 4.3.5.3 of TS 23.502 [3])

- SSC mode 3 with multiple PDU Sessions (clause 4.3.5.2 of TS 23.502 [3]), without SMF Reallocation.

### 6.12.2	Procedures

The solution uses a DNS component functionality embedded into the SMF. This is shown in Figure 6.12.2.2-1 below.

Figure 6.12.2.2-1 PSA relocation during Edge Application Server Discovery using DNS component in SMF

0a.	The SMF is pre-configured with SLA info, e.g., which EC FQDNs would trigger session re-anchoring

0b.	PDU Session establishment. SMF instructs UPF to forward the DNS traffic to SMF

1.	The EC service is identified by a FQDN, the AS-FQDN. The application in the UE does a DNS discovery request to discover the EAS. The DNS request is forwarded by central PSA (UPF1) to the SMF.

2.	The SMF checks whether the received FQDN is an AS-FQDN. If yes, then it buffers the DNS request. That decision could either be based on SLA information locally configured in SMF, or on the PCCs received from PCF for the PDU Session.

NOTE1:	Steps 1 and 2 assume DNS query triggering the procedure. DNS query may reach SMF as in Step 1, but it can also reach an external LDNSR that notifies in turn the SMF about the DNAI to use. Other alternatives are also possible, e.g., the MNO uses a DPI engine (e.g., at the UPF) to differentiate traffic of specific applications. Discovery of certain applications for certain users will then trigger procedure. Such traffic differentiation can be based in DPI as an example on the TLS client hello Server Name Indication (SNI), on destination IP ranges published or provided by the Application provider, etc.

3.	SMF shall select a UPF/PSA closest to the user. [Conditional, if latest UE Location is not available] SMF gets UE location from AMF by invoking Namf_EventExposure service with OneTime Report type (as in TS 29.518, ch 5.3.1).

4.	SMF initiates Change of PDU Session Anchor to the PSA selected in step 3 using one of the following methods:

-	SSC mode 3 with IPv6 Multi-homed PDU Session (clause 4.3.5.3 of TS 23.502 [3])

-	SSC mode 3 with multiple PDU Sessions (clause 4.3.5.2 of TS 23.502 [3])

Usage reporting for the relevant EC flows is activated to track activity.

NOTE 2:	Further re-anchoring (to a central UPF) may be triggered if EC application terminates.

NOTE 3:	As for all use of SSC modes 2 and 3, all IP traffic on the PDU session is affected since IP address is changed from old PSA (in this case central PSA) to new PSA (in this case local PSA).

5.	SMF drops the DNS query

NOTE 4:	The SMF can also initiate re-direct the DNS query to a local MNO DNS Server configured to handle UE DNS queries using distributed anchors.

6.	The UE sends again the DNS query (after expiration timer at the UE or after receiving the redirect message). The DNS query goes through the Local PSA to the DNS resolver provided at the session establishment of the new session, and it is resolved to an Edge AS as described in Solution 6.10.

7.	DNS response includes an EAS that is closest to PSA (with DNS state of the art many Authoritative (DNS) Name servers already today return different responses based on the perceived topological location of the user (see e.g. solution #10 for DNS based EAS discovery with distributed anchor)

8.	The Application Traffic then starts towards the selected Edge AS.

NOTE 5: The assumption is that PDU session setup is complete before the new DNS request is sent in step 6. DNS retransmission timers at UE (DNS client) are implementation dependent. Typical retransmission timer value recommendations in RFC 1536 is 4 seconds and increases exponentially.

### 6.12.3	Impacts on services, entities and interfaces

-	SMF: the additional logic needed to support DNS triggered re-anchoring according to the descriptions in Clause 6.12.2.2 above.

*************** End Changes ***************