3GPP TSG-WG SA2 Meeting #140E e-meeting 	S2-2005286r1

Elbonia, August 19 – September 1, 2020	(revision of S2-200xxxx)

Source:	Charter Communications

Title:	KI #1, New Sol: steering mode with location attribute

Document for:	Approval

Agenda Item:	8.6

Work Item / Release:	FS_ATSSS_ph2 / Rel-17

Abstract: This contribution proposes to add location attribute for steering mode.

# 1. Introduction

The access selection using the 4 steering modes defined for R-16 can be used for use cases related to offloading, performance improvement, and resiliency/redundancy. In general, the core network decides the ATSSS rule and the UE follows, and this rule can be dynamically adjusted based on the PCF decision in which case an updated ATSSS rules is then sent to the UE within the life of that PDU session.

During network deployment planning phase, there may be areas in which the ATSSS rule is applied differently. For example, an area where new WiFi AP are being replaced/upgraded and operator would like UE to disable ATSSS behaviour in these areas for maintenance reasons. Using R16 method, core network would have to dynamically update the ATSSS rules to each UE that is entering to the area (e.g., when PCF gets a trigger that UE has entered to certain TAI) to update the ATSSS policy. This type of “reactive” may not scale so we like to introduce a “pro-active” way of activating/deactivating ATSSS rules in the UE by introducing “location” as an attribute in the ATSSS rules in the UE. There are certainly other use cases with this type of enhancement.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-93.

# * * * * First change * * * *

## 6.0	Mapping Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

| Solutions | Title | Key Issue(s) |
| --- | --- | --- |
| #1 | QUIC-LL Steering Functionality | 2 |
| #2 | New steering mode – Autonomous steering mode | 1 |
| #3 | New steering mode – Autonomous steering mode with advanced PMF | 1 |
| #4 | New steering mode – Redundant steering mode | 1 |
| #5 | Replacing 3GPP access leg of MA-PDU Session with PDN connection in EPC | 3 |
| #6 | MPQUIC-LL Steering Functionality | 2 |
| #7 | Proposed solution based on MP-QUIC | 2 |
| #8 | Proposed solution based on QUIC | 2 |
| #X | New steering mode –location attribute to ATSSS rules | 1 |

# * * * * Second change – all New Texts* * * *

## 6.Y	Solution #X: New steering mode – Location attribute to ATSSS rule

### 6.Y.1	Introduction

This solution addresses KI#1 on Additional Steering Modes.

Location Criteria is added to ATSSS rule such that it can be used by the UE as a validation criteria to enable/disable Access selection for the matching traffic.

### 6.Y.2	High-level Description and Proposal

Same 3GPP procedure as done in R16 is used to deliver the ATSSS rules to the UE. The enhancement is to allow “UE location” (i.e, same principle as Location Criteria for URSP) to determine if this ATSSS rule is to be activated or not. This can be used e.g., when operator wanted to disable access steering from 3GPP to N3GPP only in certain area, due to plan long network maintenance (e.g., ~x days).

See below (highlighted row is new) for the proposal

(ref: 23.501: Table 5.32.8-1: Structure of ATSSS Rule)

| Information name | Description | Category | SMF permitted to modify in a PDU context | Scope |
| --- | --- | --- | --- | --- |
| Rule Precedence | Determines the order in which the ATSSS rule is evaluated in the UE. | Mandatory (NOTE 1) | Yes | PDU context |
| Traffic Descriptor | This part defines the Traffic descriptor components for the ATSSS rule. | Mandatory (NOTE 2) |  |  |
| Application descriptors | One or more application identities that identify the application(s) generating the traffic (NOTE 3). | Optional | Yes | PDU context |
| IP descriptors (NOTE 4) | One or more 5-tuples that identify the destination of IP traffic. | Optional | Yes | PDU context |
| Non-IP descriptors (NOTE 4) | One or more descriptors that identify the destination of non-IP traffic, i.e. of Ethernet traffic. | Optional | Yes | PDU context |
| Access Selection Descriptor | This part defines the Access Selection Descriptor components for the ATSSS rule. | Mandatory |  |  |
| Steering Mode | Identifies the steering mode that should be applied for the matching traffic. | Mandatory | Yes | PDU context |
| Steering Functionality | Identifies whether the MPTCP functionality or the ATSSS-LL functionality should be applied for the matching traffic. | Optional (NOTE 5) | Yes | PDU context |
| Access Selection Validation Criteria | This part defines the Access Selection Validation Criteria components | Optional |  | PDU context |
| Location Criteria | UE location where the Access Selection is considered applicable for the matching traffic. | Optional | Yes | PDU context |
| NOTE 1: Each ATSSS rule has a different precedence value from the other ATSSS rules. NOTE 2: At least one of the Traffic Descriptor components is present. NOTE 3: An application identity consists of an OSId and an OSAppId. NOTE 4: An ATSSS rule cannot contain both IP descriptors and Non-IP descriptors. NOTE 5: If the UE supports only one Steering Functionality, this component is omitted. |  |  |  |  |

### 6.Y.3	Procedures

No changes from R16.

Editor’s Note: How to handle traffic when there is no valid ATSSS rule in the UE and whether and how to enforce validity criteria in the UPF is FFS.

### 6.Y.4	Impacts on services, entities, interfaces and IETF Protocols

This solution will impact the following entities in 5GS:

- SMF: Supports sending the ATSSS rules to UE and via N4 to UPF with Location Criteria attribute..

- PCF: Supports Location Criteria attribute in the ATSSS control information to SMF.

- UPF: Supports the new N4 ATSSS rule with Location Criteria attribute.

- UE: Supports the new steering modes with Location Criteria attribute.

- 5G-AN/ NG RAN: No impact.

# * * * * End of changes * * * *