SA WG2 Meeting #S2-140E	S2-2004845

Aug 19th – Sept 1st, 2020 ; Elbonia       	(revision of S2-2004845)

Title:	[draft] LS on IP address to GPSI translation

Reply to	S6-200947/ S2-2004784

Release:	Rel-17

Work Item:	EDGEAPP

Source:	Nokia [will be SA2]

To:	SA6, SA3

Contact Person:

#### Name:	 Laurent Thiebaut

E-mail Address:	 Laurent.thiebaut@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	.

1. Overall Description:

SA2 thanks SA6 for their LS. SA2 have discussed the incoming SA6 LS and would like to answer the following

- SA6 Question: Is there a core network supported method to provide a UE’s GPSI to an EAS directly?

SA2 Answer: No

- SA6 Question: Does the core network provide an API to translate a UE's IP address (private and public) to its GPSI, and if not, would it be feasible to provide such functionality in Rel-17 in order to address the SA6 requirement for UE IP address translation?

- SA6 Question: While providing the functionality requested in bullet 1, is it feasible to provide application-specific GPSIs, to ensure that a single GPSI can not be used to track an end user's activity across applications (EASs), to protect end user privacy?

SA2 Answer to Q2 and Q3: .

- About “Allowing NEF to provide the AF with a corresponding GPSI “, security concerns were raised and SA2 would support this part of SA6 request only if agreed by SA3

- SA2 needs more time to discuss this topic and define a solution about allowing an AF to issue a request targeting a UE identified by IP addressing information especially when the IP addressing information corresponds to a NATed address.

- SA2 understands that the core network API would primarily be used in scenarios where NAT is deployed. When NAT is not deployed, or for IPv6 addresses, SA2 assumes that the EES can resolve the UE identity by querying the EEC directly via the EDGE1 reference point.

2. Actions:

To SA6 group.

ACTION: 	SA2 kindly requests SA6 to take the answer into account.

To SA3 group.

ACTION: 	SA2 kindly requests SA3 to comment about security issues related with “Allowing NEF to provide the AF with a corresponding GPSI “.

3. Date of Next SA2 Meetings:

|  | DATES | LOCATION | CTRY |
| --- | --- | --- | --- |
| SA2#141e | Oct 12th – 23rd | Elbonia | any |
| SA2#141e | Nov 16th – 20th | Elbonia | any |