SA WG2 Meeting #S2-140E	S2-2004966r06

19 Aug - 2 Sep, 2020, Elbonia

Source: 	Ericsson

Title: 	KI#1: Evaluation and conclusion

Document for: 	Approval

Agenda Item: 	8.2

Work Item / Release:	FS_eNPN / Rel-17

Abstract of the contribution: This paper proposes the evaluation and conclusion for KI#1

Proposal

Add the following changes to TR 23.700-07.

*** BEGIN CHANGES ***

*** BEGIN OF CHANGE 1 ***

7	Evaluation

*** BEGIN OF CHANGE 2 ***

8	Conclusions

8.X	Key Issue #1: Enhancements to Support SNPN along with credentials owned by an entity separate from the SNPN

In the conclusion the entity owning the credentials is called Home SNPN (HSNPN) when the owner is a SNPN (identified with PLMN ID + NID) or Home PLMN (HPLMN) when the owner is a PLMN (identified with PLMN ID).

The below conclusions are only valid for SNPN subscriptions i.e., the credential owner is a HSNPN.

Editor’s note: FFS for conclusions on PLMN subscriptions based on SA1 input on service requirements.

It is recommended to use an architecture as depicted in figure 6.1.1-1.

Editor’s note: An architecture to support AAA interfaces to service provider (SNPN) is pending input from SA3 and is FFS.

HSNPN subscriptions are provisioned in the UE.

Editor's note: It is FFS what lists are configured in the UE for doing network selection.

The UE can be configured with a list of preferred SNPNs and other information for doing network selection, which can be updated using any of the existing procedures UE configuration update or UE parameter update or other provisioning methods can be considered by Stage 3 WGs.

Editor's note: SIB enhancements to enable the above are FFS.

If credentials from multiple HSNPNare provisioned in the UE, it is assumed that the UE, before the network selection procedure, determines what HSNPNis used. How this is done is not specified.

First part of network selection (if the UE's HSNPN is not available) is to evaluate the UE configured list and other information and if there is a match between SIB information broadcast by an available SNPN and an entry in the list, the UE will attempt to register to that SNPN.

After UE registers to a SNPN, UE may reselect and register to another SNPN with the new network selection information from HSNPN.

Editor’s note: CT1 needs to confirm the network selection solution.

Authentication is done with AKA based methods for PLMN subscriptions using existing authentication procedure. For SNPN (standalone) subscriptions, non-AKA authentication procedure can be used as described in annex B of TS 33.501 [7].

Editor’s note: Registration procedure to support AAA interfaces to service provider (SNPN) is pending input from SA3 and is FFS including the subscription data V-SNPN should use in this scenario.

*** END OF CHANGE 2 ***

*** END CHANGES ***