3GPP TSG-WG SA2 Meeting #140E e-meeting  	S2-2005061

# Elbonia, August 19 – September 1, 2020	(revision of S2-200xxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 2342 | rev |  | Current version: | 16.5.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gp.p.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Re-ordering of interactions in the BDT policy re-negotiation procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA |  |  |  |  |  | Date: |  |  | 2020-08-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The current description of background data transfer policy renegotiation lists the removal of the no longer valid background data transfer policy from the UDR as an independent action which happens before the potential AF selection of a new background data transfer policy. This removal triggers an immediate URSP rule update to potentially many UEs as other PCFs have subscribed to notifications on changes in background data transfer policy in UDR. After the AF has selected another background data transfer policy, the UDR is updated again and this will trigger another URSP rule update to potentially many UEs. Given that the background data transfer policy re-negotiation is only executed by one PCF while other PCFs may be resposible for the URSP rule update to the UEs, the no longer valid backgroud data transfer policy should not be only removed after the AF selected a new one (or did not select a new one). |  |  |  |  |  |  |  |  |
| Summary of change: |  | The procedure is updated so that the background data transfer policy removal in the UDR happens after the AF selection. This ensures that there is only one URSP rule update to the UEs. It is also clarified that the previously negotiated BDT policy shall be kept if the PCF does not find an alternative BDT policy to offer to the AF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Triggering of URSP rule update to many UEs followed by another URSP rule update when a background data transfer policy is re-negotiated. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.16.7.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

#### 4.16.7.3	Procedure for BDT warning notification

Figure 4.16.7.3-1: The procedure for BDT warning notification

1.	The negotiation for Background Data Transfer (BDT) described in clause 4.16.7.2 is completed. In addition, the PCF has subscribed to analytics on "Network Performance" from NWDAF for the area of interest and time window of a background data transfer policy following the procedure and services described in TS 23.288 [50].

2.	The PCF is notified when the network performance in the area of interest goes below the criteria set by the operator from the NWDAF as described for the Network Performance analytics in TS 23.288 [50].

3.	The H-PCF may request from the UDR the stored BDT policies using Nudr_DM_Query (Policy Data, Background Data Transfer) service operation

4.	The UDR provides all the Background Transfer Policies together with the relevant information received from the AF (as defined in clause 6.1.2.4, TS 23.503 [20]) to the H-PCF. The H-PCF identifies the BDT policies affected by the notification received from NWDAF. For each of them, the H-PCF determines the ASP of which the background traffic will be influenced by the degradation of network performance and which requested the H-PCF to send the notification. The PCF then decides based on operator policies, whether a new list of candidate Background Data Transfer policies can be calculated. If the PCF does not find any new candidate BDT policy, the previously negotiated BDT policy shall be kept and no interaction with the ASP shall occur.

NOTE 1:	The BDT Policies of ASP which did not request to be notified are kept and no interaction with those ASPs occurs.

5-6.	The PCF removes the BDT policy from UDR.

NOTE 1:	The BDT Policies that are applicable for future sessions are checked by the PCF in step 4.

75.	The PCF sends the notification to the NEF by invoking Npcf_PolicyControl_Notify (Background Data Transfer Reference ID, list of candidate Background Data Transfer policies) service operation.

6.	The NEF notifies the AF by invoking Nnef_BDTPNegotiation_Notify (Background Data Transfer Reference ID, list of candidate Background Data Transfer policies) service operation.

7.	The AF selects one of the new Background Data Transfer policies included in the candidate list in the BDT warning notification.

8.	the steps 8-13 from clause 4.16.7.2 are executed.

12.	If there is a new Background Data Transfer policy stored in the UDR, the PCFs are notified by the UDR about the storage of the new Background Data Transfer Policy. The PCFs check if the corresponding URSP rules need to be updated and if so, use the procedure defined in clause 4.16.12.2 to update URSP rules for the relevant UEs.

The AF can send a Stop notification by invoking Nnef_BDTPNegotiation_Update service, when the AF requests not to receive the notification anymore. Then, the NEF invokes Npcf_BDTPolicyControl_Update service in order to provide this information for the H-PCF.

# * * * * End of changes * * * *