SA WG2 Meeting SA2-140e	S2-2005101r21

19th August – 2nd September, 2020, E-meeting	(revision of S2-201xxxx)

Source:	Intel, Ericsson

Title:	KI #4, Evaluation and interim conclusion – User plane solutions

Document for:	Discussion / Approval

Agenda Item:	8.2

Work Item / Release:	FS_eNPN / Rel-17

Abstract of the contribution:

# 1	Discussion

This discussion paper focus on deriving the basic principles across all the user plane solutions agreed so far in TR 23.007-07 addressing key issue for the onboarding and provisioning for SNPN.

SO = SNPN

27, 30, 34

5, 27,29, 31, 33, 35, 37, 39, 40

CP

UP

28

7, 28, 32, 38

SO = PNI-NPN

For the case where the SO is a SNPN, we propose provisioning via UP as focus for Rel-17 as we see minimal system impact with this approach even for the generic scenario. Generic scenario here is:  O-SNPN and SO-SNPN are different networks, PS and DCS owned by different entities.

Proposal 1 : For the case where subscription owner is SNPN, propose provisioning via UP as focus for Rel-17.

In this paper, we aim to evaluate the user plane solutions for UE Onboarding and provisioning where the subscription owner is an SNPN. In this process, we analyse how each of the solution address the following questions to solve component 1 and component 2 described in KI#4 (clause 5.4.1):

- How to identify the Provisioning Server?

- How does the Provisioning Server select SO-SNPN?

- How does the O-SNPN access the DCS?

- How is the initial access to O-SNPN performed ?

- What is the authentication procedure used for onboarding registration?

Solution 5 :

- PS identity is preconfigured in the UE (Step A in clause 6.5.3.1) or PS identity is manually entered by the user (Step B in clause 6.5.3.1) or the SO-SNPN may provide the PS identity to the UE in PDU Session Establishment Accept as PCO paramter. If the UE does not have stored PS identity, the UE constructs a well known FQDN to perform PS discovery.

- UE is preconfigured with the identity of the SO-SNPN or SO-SNPN identity is manually entered by the user or PS determines the SO-SNPN by comparing the UE identity with a configured onboarding list.

- UE includes DCS identity in the configuration PDU session establishment Request which is used by the SMF for PDU session estb auth/authentication (clause 4.3.2.3) with the DCS.

- The NG-RAN indicates this support in the SIB as “Support for onboarding” indicator.

- No primary authentication (cf. RLOS); During configured PDU session establishment, secondary authentication is performed with DCS.

Solution 27 :

- The identity of the PS is provided by the DCS. The AMF of the O-SNPN selects an SO-SNPN provided by the DCS and contacts the PS in the SO-SNPN.

- DCS is provisioned with the identity of SO-SNPN; Identity of the SO-SNPN for the UE is retrieved by the O-SNPN from the DCS; The SO-SNPN selects if CP or UP provisioning is to be used, based on UE capabilities, SLA; PS is owned by the SO-SNPN.

- The ON inspects the UE certificate, selects the DCS of the device manufacturer, and contacts the DCS for authenticating the UE.

- The NG-RAN indicates this support in the SIB as “Support for onboarding” indicator.

- Primary authentication is required.

Solution 29 :

- Same as solution 5

- Same as solution 5

- Same as solution 5

- UE is provisioned with temporary Network Identifier/Network readable name for Onboarding in addition to default UE credentials and device identifier. Before onboarding is performed, SO-SNPN selects what O-SNPN will be used to onboard the UE and provides the O-SNPN with a temporary Network Identifier/Network readable name for Onboarding. The UE will then select the O-SNPN that is broadcasting the temporary Network Identifier/Network readable name for Onboarding. RAN broadcasts temporary Network Identifier/Network readable name instead of a one bit indicator "Support for onboarding" as described in solution 5.

- Same as solution 5

Solution 35:

- Same as solution 5

- Same as solution 5

- SO-SNPN owns the DCS i.e., UDM in SO-SNPN plays the role of DCS.Since the SO has ownership of on-boarding UE, the solution assumes that the SO could get the UE's default UE credential (e.g. out of band method, for example, by scanning QR code on the on-boarding UE, reading from USB flash from device vendor, etc.), and configures it in its UDM.

- Same as solution 5, except UE constructs SUCI based on unique ID and on-boarding information (RAN broadcasts on-boarding information). Only credential is sent over user plane while the configuration information is sent over control plane

- Primary authentication is required.

Solution 33 :

- eUICC platform in the role of PS.

- eUICC platform in the role of PS.

- eUICC platform in the role of DCS.

- Obtain initial IP connectivity using default operator profile stored in the eUICC in the UE.

- Authentication based on the default operator profile in eUICC.

Solution 31(Case 2 - ON is an SNPN) :

- PS address controlled by the ON network as local configuration information used by the SMF during PDU session establishment procedure.

- Not described in the solution, how the PS selects the SO-SNPN. Nor is this information provided from the UE.

- AAA-S = DCS. During initial registration, the AMF in ON performs the authentication between UE and DCS using default UE credentials.

- UE is provisioned with default configuration at manufacture which includes (for each onboarding SNPN) PLMN+NID ID, priority and optionally subscribed SO ID. ON broadcasts PLMN+NID ID, support of UE onboarding, Supported SO-ID list. During PDU session establishment, the AMF selects an SMF that supports onboarding based on local configuration. The SMF obtains the local configuration instead of retrieving the subscription data from UDM. The local configuration includes the restriction information of the PDU Session for the purpose of performing remote provisioning.

- Primary authentication is required.

Solution 37 :

- UE is provisioned with default configuration UE identity, default UE credentials, default OPF address, optional list of ON SNPN. OPF (Onboarding and provisioning function) performs the function of DCS and PS.

- The Onboarding and Provisioning Function (OPF) is pre-configured with UE provisioning data, e.g. subscriber identifier, individual credential, etc.

- The default OPF address is used for O-SNPN to discover the OPF which can provide the O&P service for the UE.

- Same as solution 5 with prmary authentication.

- Primary authentication is required. New interfaces need to be defined between AMF and OMF, between two OMFs.

Solution 39 :

- As part of the agreement between the UE manufacturer and the HPLMN, the UE subscription in the HPLMN allows access to a DN that provides connectivity to the Provisioning Server;The UE subscription corresponding to the USIM profile contains a default Subscribed S-NSSAI and a default DNN that are appropriate to reach the Provisioning Server; consequently a PDU Session that provides access to the Provisioning Server is established without the UE having to indicate any NSSAI or DNN information;The HPLMN provides the GPSI of the UE subscription to the Provisioning Server e.g. using header enrichment and the provisioning server relies on the GPSI to identify the UE.

- Same as solution 5 except the Provisioning Server determines the future SNPN by comparing the GPSI with a configured onboarding list.

- The UDM in the HPLMN plays the role of DCS.

- The UE is configured with Default credentials in USIM to attach to any PLMN in which the UE can register with the Default credentials, i.e. an Onboarding Network (ON);It is assumed that the UE manufacturer has a business relationship with one PLMN operator (referred to as HPLMN in the solution);performs normal PLMN selection, registers and establishes a PDU Session using normal procedures.

- Primary authentication with HPLMN is required.

Solution 40 :

- -

- Out of scope.The provisioning server pushes the UE's credentials to the UE. The desired SNPN stores UE's credentials in UDM, the provisioning server pushes the UE's credentials to the SNPN's UDM via the NEF.

- (Remote attestation Procedures architecture IETF)UE connects to PS over application layer protocol. This contains UE identity, attestation token. The PS (acts as a relay) forwards the attestation token to DCS for verification.

- Same as solution 5. Obtain network connectivity for the UE that can communicate with PS.

- Same as solution 5.

# 2	Proposal

It is proposed to update TR 23.700-07 as indicated below.

* * * * Start of Change (All text is new)* * * *

# 7	Evaluation

## 7.X	Key Issue #4: UE Onboarding and remote provisioning

### 7.X.1	User plane based solutions

Solution 5 :

- PS identity is preconfigured in the UE (Step A in clause 6.5.3.1) or PS identity is manually entered by the user (Step B in clause 6.5.3.1) or the SO-SNPN may provide the PS identity to the UE in PDU Session Establishment Accept as PCO paramter. If the UE does not have stored PS identity, the UE constructs a well known FQDN to perform PS discovery.For example, provisioning-server.3gpp.org, ps-onborading.snpn.3gpp.org .

- UE is preconfigured with the identity of the SO-SNPN or SO-SNPN identity is manually entered by the user or PS determines the SO-SNPN by comparing the UE identity with a configured onboarding list.

- UE includes DCS identity in the configuration PDU session establishment Request which is used by the SMF for PDU session estb auth/authentication (clause 4.3.2.3) with the DCS.

- The NG-RAN indicates this support in the SIB as “Support for onboarding” indicator.

- No pPrimary authentication is performed during registration(cf. RLOS); During configured PDU session establishment, secondary authentication is performed with DCS.

Solution 27 :

- The identity of the PS is provided by the DCS. The AMF of the O-SNPN selects an SO-SNPN provided by the DCS and contacts the PS in the SO-SNPN.

- DCS is provisioned with the identity of SO-SNPN; Identity of the SO-SNPN for the UE is retrieved by the O-SNPN from the DCS; The SO-SNPN selects if CP or UP provisioning is to be used, based on UE capabilities, SLA; PS is owned by the SO-SNPN.

- The ON inspects the UE certificate, selects the DCS of the device manufacturer, and contacts the DCS for authenticating the UE.

- The NG-RAN indicates this support in the SIB as “Support for onboarding” indicator.

- Primary authentication is required.

Solution 29 :

- Same as solution 5

- Same as solution 5

- Same as solution 5

- UE is provisioned with temporary Network Identifier/Network readable name for Onboarding in addition to default UE credentials and device identifier. Before onboarding is performed, SO-SNPN selects what O-SNPN will be used to onboard the UE and provides the O-SNPN with a temporary Network Identifier/Network readable name for Onboarding. The UE will then select the O-SNPN that is broadcasting the temporary Network Identifier/Network readable name for Onboarding. RAN broadcasts temporary Network Identifier/Network readable name instead of a one bit indicator "Support for onboarding" as described in solution 5.

- Same as solution 5

Solution 35:

- Same as solution 5

- Same as solution 5

- SO-SNPN owns the DCS i.e., UDM in SO-SNPN plays the role of DCS.Since the SO has ownership of on-boarding UE, the solution assumes that the SO could get the UE's default UE credential (e.g. out of band method, for example, by scanning QR code on the on-boarding UE, reading from USB flash from device vendor, etc.), and configures it in its UDM.

- Same as solution 5, except UE constructs SUCI based on unique ID and on-boarding information (RAN broadcasts on-boarding information). Only credential is sent over user plane while the configuration information is sent over control plane

- Primary authentication is required.

Solution 33 :

- eUICC platform in the role of PS.

- eUICC platform in the role of PS.

- eUICC platform in the role of DCS.

- Obtain initial IP connectivity using default operator profile stored in the eUICC in the UE.

- Authentication based on the default operator profile in eUICC.

Solution 31(Case 2 - ON is an SNPN) :

- PS address controlled by the ON network as local configuration information used by the SMF during PDU session establishment procedure.

- Not described in the solution, how the PS selects the SO-SNPN. Nor is this information provided from the UE.

- AAA-S = DCS. During initial registration, the AMF in ON performs the authentication between UE and DCS using default UE credentials.

- UE is provisioned with default configuration at manufacture which includes (for each onboarding SNPN) PLMN+NID ID, priority and optionally subscribed SO ID. ON broadcasts PLMN+NID ID, support of UE onboarding, Supported SO-ID list. During PDU session establishment, the AMF selects an SMF that supports onboarding based on local configuration. The SMF obtains the local configuration instead of retrieving the subscription data from UDM. The local configuration includes the restriction information of the PDU Session for the purpose of performing remote provisioning.

- Primary authentication is required.

Solution 37 :

- UE is provisioned with default configuration UE identity, default UE credentials, default OPF address, optional list of ON SNPN. OPF (Onboarding and provisioning function) performs the function of DCS and PS.

- The Onboarding and Provisioning Function (OPF) is pre-configured with UE provisioning data, e.g. subscriber identifier, individual credential, etc.

- The default OPF address is used for O-SNPN to discover the OPF which can provide the O&P service for the UE.

- Same as solution 5 with prmary authentication.

- Primary authentication is required. New interfaces need to be defined between AMF and OMF, between two OMFs.

Solution 39 :

- As part of the agreement between the UE manufacturer and the HPLMN, the UE subscription in the HPLMN allows access to a DN that provides connectivity to the Provisioning Server;The UE subscription corresponding to the USIM profile contains a default Subscribed S-NSSAI and a default DNN that are appropriate to reach the Provisioning Server; consequently a PDU Session that provides access to the Provisioning Server is established without the UE having to indicate any NSSAI or DNN information;The HPLMN provides the GPSI of the UE subscription to the Provisioning Server e.g. using header enrichment and the provisioning server relies on the GPSI to identify the UE.

- Same as solution 5 except the Provisioning Server determines the future SNPN by comparing the GPSI with a configured onboarding list.

- The UDM in the HPLMN plays the role of DCS.

- The UE is configured with Default credentials in USIM to attach to any PLMN in which the UE can register with the Default credentials, i.e. an Onboarding Network (ON);It is assumed that the UE manufacturer has a business relationship with one PLMN operator (referred to as HPLMN in the solution);performs normal PLMN selection, registers and establishes a PDU Session using normal procedures.

- Primary authentication with HPLMN is required.

Solution 40 :

- Out of scope.The provisioning server pushes the UE's credentials to the UE. The desired SNPN stores UE's credentials in UDM, the provisioning server pushes the UE's credentials to the SNPN's UDM via the NEF.

- (Remote attestation Procedures architecture IETF)UE connects to PS over application layer protocol. This contains UE identity, attestation token. The PS (acts as a relay) forwards the attestation token to DCS for verification.

- Same as solution 5. Obtain network connectivity for the UE that can communicate with PS.

- Same as solution 5.

* * * * Next Change * * * *

# 8	Conclusions

## 8.X	Key Issue #4: UE Onboarding and remote provisioning

Editor’s Note: These are interim conclusions for Key Issue #4.

UE onboarding for SNPN (Component 1 of KI#4)

-

-	Procedures for UE onboarding with Default UE credentials used for primary authentication and O-SNPN as Onboarding Network (ON) may be specified pending SA3 confirmation that primary authentication is required for user plane Onboarding;

-	In order to support UE onboarding using Default UE credentials and O-SNPN as the Onboarding Network (ON) the distribution of security functions between the O-SNPN and SO-SNPN should be decided by SA3;

-

Onboarding Network (ON) is a role that can be assumed by the SO-SNPN or by a network different than SO-SNPN.. In other words, O-SNPN and SO-SNPN could be the same SNPN or O-SNPN and SO-SNPN can be different.

-	Only Onboarding network (ON) interacts with the DCS which includes the case where the SO-SNPN is in the role of O-SNPN during UE onboarding.

-	Primary authentication by O-SNPN is always needed unless SA3 decides that absence of primary authentication should also be allowed for onboarding;

NOTE:	The decision on whether primary authentication is required during initial access to the O-SNPN is dependent on SA3 feedback.

-	The O-SNPN determines the DCS identity based on the realm part of the SUPI.

NOTE:	It is FFS whether some alternative way to identify the DCS is to be added.

-	The NG-RAN of the Onboarding network includes information in the SIB so that the UE can discover the O-SNPN.

-	The UE selects an O-SNPN based on information that is broadcast by the NG-RAN in the SIB. During Registration procedure, the UE provides a RRC information in the AS layer that can be used by the NG-RAN to select an AMF for onboarding.

Editor’s Note: 	It is FFS whether the information in the SIB is a single onboarding supported indication bit or if additonal information is required to ensure that the UE discovers the correct O-SNPN for connecting to the right SO-SNPN (e.g. should NG-RAN indicate whether onboarding is supported on a per SO-SNPN basis is FFS?).

Editor’s note: 	Whether new RRC information is needed or existing RRC information can be used e.g. NSSAI information is FFS.

-	Using PLMN credentials for UE onboarding and PLMN as Onboarding Network (ON) e.g. as described in Solution #39 is already possible and does not require any further standardisation work.

-	Support for restricted registration used for on-boarding service shall be provided from the system procedures.

-	The UE initiates de-registration from the on-boarding network after finishing the remote provisioning or the on-boarding 5G network initiates the de-registration after successful completion of onboarding;;

Editor’s note: 	It is FFS whether the UE or network shall initiate deregistration upon completion of onboarding.

UE Onboarding for PNI-NPN (Component 1 of KI#4)

-	No enhancement for the UE onboarding (component 1 of KI#4) is needed for the case of PNI-NPN credentials provisioning since the UE selects and registers with the PLMN based on the PLMN credentials and configuration for network selection.

* * * * End of Change * * * *