SA WG2 Meeting #S2-140E	S2-2005127

19 Aug - 2 Sep, 2020, Elbonia

Source: 	Ericsson

Title: 	Evaluation and conclusion for KI#3

Document for: 	Approval

Agenda Item: 	8.4

Work Item / Release:	FS_eNS_Ph2 / Rel-17

Abstract of the contribution: This paper proposes the evaluation and conclusion for KI#3.

Discussion

This is a proposal for interim evaluation and interim conclusions for KI#3 that will be a starting point and need to be improved when Editor´s Notes in the different solutions, possible new solutions and overall conclusions are reached.

The evaluation criteria are taken from the proposal from rapporteur during the conference call for eNS_ph2.

Proposal

Add the following changes to TR 23.700-20.

*** BEGIN CHANGES ***

7	Evaluation

Editor's note:	This clause will provide a general evaluation of the solutions.

7.x	Key Issue #3: Limitation of data rate per network slice in UL and DL per UE

Editor's note:	This clause will provide some interim evaluation based on solutions #13, #20, #21, #22 that will need further updates to address e.g. roaming aspects.

The other detail criteria for evaluation includes aspects as following:

. System architectural impacts

- Solution#21 introduces a new NF controls the Slice-MBR value without impacting other NFs other than SMF that needs to contact NSQ per each PDU session establishment and modification in addition to the PCF, UDM, DN-AAA. The need to define a new NF to control QoS parameters is not justified, since QoS control is part of the PCF functionality.

- All solutions impact the existing NF services to a certain extent, so this is not considered as a differentiation factor.

Signalling overhead

- Solution #21 adds some additional signalling for each PDU session establishment, and new triggers for the PDU session modification.

- Solution#13, #20, #22 does not add new signalling to the PDU session establishment.

Administration and configuration overheads

- Solution#20 method 1 does not add any new requirements on policy data or subscription data. It requires an SLA in place on the Slice-MBR value for a UE, as such it defined requirements on configuration by OAM in each PCF supporting this feature.

- Solution #13, #21, #22 and Solution#20 method 2 add new requirements on subscription data to store the Slice-MBR in the UDR.

The administration and configuration overheads are not a differentiator factor.

Backwards compatibility

- To be defined.

Other aspects:

- In relation to measurements for counting of packets performed at the UPF according to 23.501, solution #22 may cause that packets are counted at the UPF and which in non-GBR QoS Flow later are dropped at the RAN. This is however already happening with the delayed UE AMBR in existing systems and 5GS.

8	Conclusions

Editor's note:	This clause will capture conclusions for Key Issue #<X>.

8.x	Key Issue #3: Limitation of data rate per network slice in UL and DL per UE

Editor's note:	This clause will provide some interim conclusion based on solutions #13, #20, #21, #22 that will need further updates to address e.g. roaming aspects.

*** END CHANGES ***