3GPP TSG-SA WG2 Meeting #140E	 		S2-2005164r04

Elbonia, 19 – 31 Aug. 2020, Electronic meeting

Title:	[draft] LS on Cell Configuration within TA/RA to Support Allowed NSSAI

Release:	Rel-17

Work Item:	FS_eNS_Ph2

Source:	SA2

To:	RAN2, RAN3, CT1

Cc:

Contact Person:

#### Name:			Tricci So

Tel. Number:

E-mail Address:			TSO@ZTETX.COM

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

The Design of the 5GS in SA2 has assumed that the UE obtains an Allowed NSSAI depending on the TA where the UE is under.  SA2’s assumption is that all S-NSSAIs in the Allowed NSSAI are supported within the TA and also in all TAs of the RA. SA2 would like to consult with CT1, RAN2 and RAN3 colleagues with the following:

1) In Rel-15 and 16, is it expected that each cell in the tracking area supports the same S-NSSAI(s)? (or, said otherwise, do all cells advertising the same TAC support the same set of S-NSSAIs?).

2) If the answer is "no" can RAN WGs and CT1 explain how the case is handled of a UE that due to local radio conditions can only use a cell in the TA where not all S-NSSAIs the TA supports are supported? Is there a CT1 error code to handle this case (i.e. can a S-NSSAI be rejected depending on which cell the UE camps on even though it is supported in the TA, for the reason that it is not supported in the cell). Is, there any provisions in the RAN or CT1 specifications to handle this case?

Note: SA2 is not considering changing any of the Rel-15 and Rel16 assumption on support of the S-NSSAI in the TA in order to avoid creating deployments incompatible with UEs in the field.

2. Actions:

To CT1, RAN2 and RAN3:

ACTION:

SA2 kindly requests to provide us your feedback for the four questions above.

3. Date of Next TSG-SA2 Meetings:

3GPP TSG SA2#141E	12th – 23rd October 2020	  eMeeting

3GPP TSG SA2#142E     16th – 20th  November 2020	eMeeting