SA WG2 Meeting #140E (e-meeting)	S2-2005228r02

# Aug 19 – Sep 2, 2020, Electronic, Elbonia

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | draft | rev | - | Current version: | 16.5.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of new RATs in 5GS integrating satellite access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_ARCH |  |  |  |  |  | Date: |  |  | 2020-08-02 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel 17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | According to the conclusion to the KI#4 in the TR 23.737, the solution 2 is considered as the baseline to support new RATs in 5GC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introducing new RAT types into 5GS for satellite access. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Satellite access can not be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.x(New), 5.x.x(New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First change * * *

## 5.x	Support for integrating satellite access into 5GS

5.x.1	General

This clause describes the support of satellite access and the support of using satellite link as backhaul in in 5GS.

NR satellite access affects the system behaviour as follows:

-	NR satellite access cells are identified by the AMF which informs also other NFs of the CN RAT Types (NR(LEO), NR(MEO), NR(GEO), NR(OTHERSAT)

-	Awareness of NR satellite access RAT Type is used for adjusting system behaviour in UE network selection, access control, QoS, etc.

Editor's note:	this clause is intended as a placeholder for high level NR satellite access functionality

### 5.x.2	Support of RAT types defined in 5GC for satellite access

Following RAT types are introduced for satellite access to distinguish the different satellite enabled RAN nodes in 5GC:

-	"NR (LEO)": which RATs, would be NR RAN's, based on LEO satellite constellations, for which the satellite altitude would be below 2000 km.

-	"NR (MEO)": which RATs, would be NR RAN's, based on MEO satellite constellations, for which the satellite altitude would be between 8000 km and 25000 km.

-	"NR (GEO)": which RATs would be NR RANs, based on GEO satellites constellations, for which the satellite altitude would be on geo-stationary orbit 35786 km.

-	"NR (OTHERSAT)": which RATs, would be NR RAN's, based on other type of satellites that are not identified yet.

When a UE is accessing to the network via satellite access, the AMF determines the RAT type based on the TA indicated by the RAN node serving the UE.

Editor's note: Avoiding repetition of the AMF NR satellite access detection criteria is strongly recommended. The feasibility of defining the AMF detection criteria in just one place, such as here, and then referencing to that only definition from the other clauses that are affected by NR satellite access is FFS (and recommended).

NOTE:	It is assumed that TA allocation for satellite access is per RAT defined above. For a gNB with Remote Frenquency Remote Unit/gNB-DU deployed on different altitude of satellites, the gNB shall distinghuish the TAC range for different Remote Frenquency Remote Unit/gNB-DU deployed on different altitude of satellites.

The AMF should signal the determined RAT type for satellite access to the SMF and PCF for the following purposes:

-	QoS and policy control;

-	Mobility procedures, such as Mobility Restrictions (per RAT Type and per TA);

-	Charging specific to satellite access (per RAT Type);

-	Location services for satellite access (discovery of LMF supporting the appropriate positioning methods);

-	Regulatory requirement (to authorise satellite access, possibly per RAT).

* * * End of changes * * *