SA WG2 Meeting #140-E	S2-2005249

Aug 19 – Sep 01, 2020, Electronic

Source:	vivo, OPPO, Xiaomi

Title:	KI#1, Sol#16: Updates to add impact

Document for:	Approval

Agenda Item:	8.3

Work Item / Release:	FS_enh_EC / Rel-17

Abstract of the contribution: This paper proposes some impacts to solution #16.

# 1 Discussion

In solution #16 of TR 23.748, it proposes that the UE gets Edge Configuration Server Information carried in the PCO from the SMF during the PDU Session Establishment procedure.

By using NAS to carry ECS info, the impacts on UE OS kernel (Android) is inevitable, UE OS kernel has to design new standard interface (such API) for functional invocation and adaptation. At least for now, the information from NAS is not allowed to communicated with application client directly. These impacts need to be reflected.

# 2 Proposal

It is proposed to add the following solution in TR 23.748.

# * * * * Start change * * * *

### 6.16.4	Impacts on services, entities and interfaces

UE:

-	Provides an indication of Requesting Edge Configuration Server Information to the AMF during Registration.

-	Receives Edge Configuration Server Information from the AMF during the (re-)Registration and UE Configuration Update procedures.

-	It will be necessary to expose the ECS information to upper layer application (i.e. the Edge Enabler Client).

UDM/UDR:

-	A UE's subscription information may include Edge Configuration Server Information (i.e. one or more FQDNs and/or IP Address(es) of Edge Configuration Servers).

SMF:

-	May receive an indication of Requesting Edge Configuration Server Information from the UE during PDU Session Establishment.

-	Sends Edge Configuration Server Information to the UE during the PDU Session Establishment procedures.

-	Receives Edge Configuration Server information from the UDM/UDR during PDU Session Establishment (when Nudm_SDM_Get is invoked).

# * * * * End of changes * * * *