SA WG2 Meeting #S2-140E	S2-2005273

August 19 ~ September 2, 2020, Elbonia

Source:	vivo

Title:	Conclusion for 5MBS KI#1

Document for:	Approval

Agenda Item:	8.9

Work Item / Release:	FS_5MBS / Rel-17

Abstract of the contribution: Conclusion for KI#1.

# 1	Introduction

This documents captures SA2 agreed conclusions for Key issue #1.

# 2	Proposal

It is proposed to capture the following interim agreements into TR 23.757.

# * * * * First change * * * *

# 7	Evaluation

## 7.X	Evaluation for Key Issue #1: MBS session management

Evaluation should be based on the following principles:

-	The network shall support multicast session join/leave operations, the UE shall support multicast session join/leave operations ,

Editor’s Note: It is FFS if session join/leave operation via UP e.g. IGMP Join, and/or in the CP are supported.

Editor’s Note: Support for QoS e.g. resource reservation for multicast session is FFS.

-	The network and the UE may support 5GC Individual MBS Traffic delivery fall-back for service continuity within NR.

-	The network shall only use the 5GC individual MBS Traffic delivery method for a UE served by a RAN node not supporting 5G MBS.

-	The network shall support MBS traffic delivery via shared N3 (GTP-U) tunnels using transport layer IP multicast method. The network shall support MBS traffic delivery via MB-UPF distributing over multiple shared N3 (GTP-U) Point-to-Point tunnels. Which method is used depends on the deployment.

-	The network shall be able to prepare and start multicast traffic transmission for a MBS session after MBS service is requested and started from AF, or based on configuration, or based on the first UEs joining a session.

-	The network shall support selection of MB-SMF or SMF by NEF or MBSF at session start. The network shall support selection of MB-SMF or SMF by AMF when a UE joins a session.

Editor’s Note: Terminology needs to be updated based on selected architecture

-	An MB Session shall be uniquely identified by an ID.

-	For N6 connectivity, solutions shall support tunnelling of MBS data and untunnelled MBS data (IP multicast based).

# * * * * End of change * * * *