3GPP TSG-SA2 Meeting #140-e	S2-2005338r01

# Online, , 19st Aug 2020 – 2nd Sep 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | draftCR | rev | - | Current version: | 16.5.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Procedures for satellite backhaul connected UPF selection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_ARCH |  |  |  |  |  | Date: |  |  | 2020-08-13 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The SA2 #139e approved contribution S2-2004705 with a comment below: Whether and when QoS limitation defined in Solution #11 needs to be considred for UPF selection may be discussed in the normative work Signalling flow to tansport all backhaul connections that an AN node has for the accessing UE is missing in PDU Session Establishment procedures. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add signalling flow to transport Backhaul-related Information as diffrences and clarifications on Figure 4.3.2.2.1-1: UE-requested PDU session Establishment for non-roaming and roaming with local break out. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | QoS limitation awared UPF selection cannot be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.xx (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*************** First change (all text is new) **************

4.xx	Support for using satellite backhaul

4.xx.x	PDU Session establishment procedures

4.xx.x.1	Non-roaming and Roaming with Local Breakout

The signalling flow for a PDU Session establishment using satellite access when the UE is not roaming, or when the UE is roaming and the PDU Session Anchor (PSA) is located in the VPLMN, is based on the signalling flow in Figure 4.3.2.2.1-1 with the following differences and clarifications:

-	In step 1, the AN provides Satellite Category in UL NAS Transport message.

The AMF determines if the AN has satellite backhaul connection based on Satellite Category received from the AN, or TAI, or local configuration.

-	In step 3, if the AMF determined that the AN has satellite backhaul connection in step 1, the AMF provides the Satellite Category to the SMF.

-	In step 8, the SMF selects UPF(s) based on Satellite Category from the AMF and QoS limitation Information of User Plane peers of the candidate UPFs.

*************** End of changes ***************