3GPP TSG-WG SA2 Meeting #140E	S2-2005357r02

Aug 19 – Sep 01, 2020, Elbonia	(revision of S2-20xxxx)

Source:	China Mobile

Title:	KI #1, Sol #7: Update to clarify the solution

Document for:	Approval

Agenda Item:	8.1

Work Item / Release:	FS_eNA_ph2 / Rel-17

Abstract: This contribution proposes clarification on Solution #7.

1. Introduction

This contribution proposes clarification on Solution #7.

2. Text Proposal

It is proposed to capture the following clarification in TR 23.700-91.

* * * * Start of Changes * * * *

6.7	Solution #7: NWDAF functionality Split

6.7.1	Description

This solution addresses Key Issue #1: NWDAF functionality Split.

As shown in Figure 6.7.1-1, the 5GC could support data analytics as:

-	Training Function in NWDAF could provide training services to 5GC NF(s).

-	Inference Function in NWDAF could invoke the training service provided by Training Function in NWDAF, if needed.

Figure 6.7.1-1: NWDAF functionality Split

As shown in Figure 6.7.1-1, NWDAF is to support data analytics cross the 5GC NFs, and AF, which are composed of the following functionalities:

1)	Data collection;

2)	Training function: refer to the process of creating a machine learning model, which involves the use of a machine learning algorithm and a training data set;

3)	Inference function: refer to the process of using the trained machine learning model to make predictions with new data and exposing network work data analytics to the consumer NF.

As indicated in TS 23.288 [5], the NWDAF instance(s) can be collocated with a 5GS NF.

6.7.2	Procedures

Figure 6.7.2-1: Procedure for Inference Function in NWDAF request for Training service provided by Training Function in NWDAF

1a-1b.	Training service provided by Training function in NWDAF 1 or NWDAF 2 sends registration request by invoking Nnrf_NFManagement_NFRegister to register its own service in the NRF, including some parameters, e.g. IP address, location, load, algorithm information (Algorithm ID, supported accuracy, supported convergence time).

2a-2b.	The Consumer NF invokes the Nnwdaf_AnalyticsSubscripion_Subscribe or Nnwdaf_AnalyticsInfo_Request service provided by Inference Function in NWDAF 3 for analytics.

NOTE:	In step 2, the data analytics is triggered when the Inference Function in NWDAF 3 finds that the Analytics ID consumed by the consumer NF is not supported.

3.	Inference Function in NWDAF 3 invokes Nnrf_NFDiscovery_Request service operation using parameters such as algorithm information (Algorithm ID, supported accuracy, supported convergence time).

4.	NRF matches the requested query and sends this information via Nnrf_NFDiscovery_Request response service to Inference Function in NWDAF 3 including one or more Training services provided by the Training Function in NWDAF 1 or NWDAF 2.

5.	Inference Function in NWDAF 3 selects Training Function in NWDAF 1 based on the output of NRF.

6.	Inference Function in NWDAF 3 sends request including some parameters, e.g. Analytics ID for a machine learning model to invoke the Training service provided by the Training Function in NWDAF 1 for machine learning model generation.

7.	Training Function in NWDAF 1 performs data collection and machine learning model training.

8.	Inference Function in NWDAF 3 receives the response from the Training Function in NWDAF 1 including machine learning model response and some parameters, e.g. machine learning model identification, input parameter and output parameter.

9.	Inference Function in NWDAF 3 completes the machine learning model deployment and inference. In this step, the Inference Function in NWDAF 3 is using the received trained model to perform inference, but the model is not deployed anywhere else.

10.	Inference Function in NWDAF 3 sends analytics response to the requested NF.

11.	[Optional] Inference Function in NWDAF 3 may sends Analytics Registration Request to the NRF with analytics ID and address for further Analytics enquiry.

6.7.3	Impacts on services, entities and interfaces

NWDAF is decomposed into Training Function in NWDAF and Inference Function in NWDAF.

Training Function in NWDAF:

-	Support to register its NF profile (IP address, location, load, algorithm information (Algorithm ID, supported accuracy, supported convergence time)) into the NRF;

-	Support to provision a machine learning model to Inference Function in NWDAF.

Inference Function in NWDAF:

-	Support to discover a suitable Training Function in NWDAF via NRF;

-	 Support to request a machine learning model from the Training Function in NWDAF.

NRF:

-	Support to accept the NF registration from Training Function in NWDAF;

-	Support to accept the discovery of the Training Function from Inference Function in NWDAF .

* * * * End of Changes * * * *