3GPP TSG-WG SA2 Meeting #140E 	S2-2005359r01

Aug 19 – Sep 01, 2020, Elbonia	(revision of S2-200xxxx)

Source:	China Mobile

Title:	KI #2, New Sol: Supporting re-selection of serving NWDAF due to UE mobility

Document for:	Approval

Agenda Item:	8.1

Work Item / Release:	FS_eNA_ph2 / Rel-17

Abstract: This contribution proposes to add the new solution of re-selection of serving NWDAF due to UE mobility.

Discussion

The consumer NF should select an appropriate NWDAF among multiple NWDAF instances to make subscriptions for data analytic. There will be some cases that serving NWDAF of one consumer NF may need to change due to UE mobility event. Upon the change of UE location, AMF is required to inform the serving NWDAF, and the serving NWDAF decide whether to continue analysis service or start NWDAF re-selection. It is also required to inform the related consumer NF of the NWDAF re-selection event and also to update subscriptions between the NWDAF and the consumer NF.  The analytic data and stored input data can be retrieved from the previous serving NWDAF instance by new NWDAF, if applicable, to facilitate producing analytic for new NWDAF.

This contribution proposes a new solution for re-selection of NWDAF which allows transmission of stored analytics data and/or input data between NWDAF instances.

Proposal

It is proposed to adopt the text in the TR 23.700-91.

* * * Start of Change * * * *

6.X	Solution X: Reselection of NWDAF due to mobility change

6.X.1	Description

This solution is for Key Issue #2: Multiple NWDAF instances.

The consumer NF should select an appropriate NWDAF among multiple NWDAF instances to make subscriptions for data analytic. There will be some cases that serving NWDAF of one consumer NF may need to change due to UE mobility event. Upon the change of UE location, AMF is required to inform the serving NWDAF, and the serving NWDAF decides whether to continue analysis service or start NWDAF re-selection. It is also required to inform the related consumer NF of the NWDAF re-selection event and also to update subscriptions between the NWDAF and the consumer NF.  The analytic data and stored input data can be retrieved from the previous serving NWDAF instance by new NWDAF, if applicable, to facilitate the new NWDAF to produce the analytics.

6.X.2	Procedures

6.X.2.1  Re-selection of NWDAF without AMF change

Figure 6.X.1.1.1-1: Procedure for re-selection of NWDAF without AMF change

1.	Consumer NF sends an analytics subscription to the NWDAF1 by invoking an Nnwdaf_AnalyticsSubscription_Subscribe service; the NWDAF1 subscribes to fetch the network data or management data from the NF(s) or the OAM and the NF(s) or the OAM deliver the network data or management data, as defined in 3GPP TS 23.288 [5].

2.	The NWDAF1 subscribes the UE mobility event notification from AMF by invoking an Namf_EventExposure_Subscribe service.

3.	UE mobility event is detected by AMF and the AMF notifies the UE mobility event to the NWDAF1 by invoking the Namf_EventExposure_Notify service if the TAI of the UE changes. NWDAF1 determines if the UE location is in the serving area of NWDAF1. If yes, the NWDAF1 could continue the analytics service, otherwise, the NWDAF1 triggers the re-selection of NWDAF.

4.	The NWDAF1 interacts with NRF by sending Nnrf_NFDiscovery_Request to discover an appropriate NWDAF2 for the UE.

5.	The NWDAF1 invokes the Nnwdaf_AnalyticsRelocation_Request service to the NWDAF2 to trigger relocation. The service includes subscription information, consisting of Subscription ID, Analytics ID, Target of Reporting and Event Reporting Information. The NWDAF2 responds to the NWDAF1 with a Nnwdaf_AnalyticsRelocation_Response service operation which includes information on the subscription selected by the NWDAF2.

6.	The NWDAF1 sends Nnf_EventExposure_Unsubscribe to the NF(s) or the OAM to unsubscribe the fetching the network data.

7.	<option A>The NWDAF1 invokes the Nnwdaf_AnalyticsSubscription_Notify service to the consumer NF to notify the release of the subscriptions for the consumer NF and also notify consumer NF about the information of NWDAF2 taking over the analytics subscription.

<option B>The NWDAF2 invokes the Nnwdaf_AnalyticsSubscription_Notify service to the consumer NF to notify the release of the subscriptions of NWDAF1 and take over the analytics subscription for the consumer NF.

8.	The NWDAF2 invokes the Nnf_EventExposure_Subscribe service to the source NF(s) or OAM to subscribe the data in the source NF.

6.x.2.2 Re-selection of NWDAF with AMF change

Figure 6.x.1.1.2-1: Procedure for re-selection of NWDAF with AMF change

1~2.	Steps 1~2 are executed as defined in clause 6.x.1.1.1.

3.	The AMF2 invokes the Namf_Communication_UEContextTransfer service operation to the AMF1 to fetch the UE context which includes the subscription of the UE mobility event from AMF1.

4~9.	Steps 3~8 are executed as defined in clause 6.x.1.1.1.

6.x.3	Impacts on Existing Nodes and Functionality

NWDAF:

- Supports a subscription transfer procedure.

- Transfers and receives stored analytics and input data.

Consumer NF:

- Supports subscription renewal to new NWDAF.

* * * End of Change * * * *