3GPP TSG-WG SA2 Meeting #140E e-meeting 	S2-2005370

Elbonia, August 19 – September 2, 2020	(revision of S2-200xxxx)

# Title:	[DRAFT] Reply LS to RAN3 for clarification of QoS Parameters Update at Xn Handover

# Response to:	LS (S2-2004809/R3-204325) on QoS Parameters Update at Xn Handover from RAN3

# Release:	Rel-16

# Work Item:	5G_URLLC/Vertical_LAN

Source:	[Huawei to be] SA2

To:	RAN3

Cc:

Contact Person:

Name:	Hui Ni

Tel. Number:

E-mail Address:	Hui DOT Ni AT huawei DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	S2-2005371/S2-2005372 (to be updated)

1. Overall Description:

SA2 would like to thank RAN3 for the LS on QoS Parameters Update at Xn Handover in S2-2004809/R3-204325. SA2 discussed the questions and agreed the following answers.

Q1: Whether only the CN PDB parameter can be updated in Path Switch Request Acknowledge message, or any other parameter can also be updated.

SA2 answer: For existing QoS parameters, only CN PDB can be updated in Path Switch Request Acknowledge message in current SA2 specifications. Besides QoS parameters, SA2 discussed also that TSCAI for the accepted QoS Flows may also need to be updated during Xn HO since the Burst Arrival Time of TSCAI is calculated based on CN PDB.

Q2: The expected actions of the NG-RAN node upon receiving the updated parameters (e.g. CN PDB), given that the Path Switch Request Acknowledge message has no reply.

SA2 answer: SA2 assumes that NG-RAN stores the received updated parameters (e.g. CN PDB) and apply the updated parameters to the QoS flow. .

SA2 has agreed updates to TS 23.501 and TS 23.502 based on the above answers and kindly asks RAN3 to give feedback, if any.

2. Actions:

To RAN3 group.

ACTION: 	SA2 kindly asks RAN3 to take above information into consideration.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #141e			TBD				Elbonia