3GPP TSG-WG SA2 Meeting #140E e-meeting 	S2-2005413

Elbonia, August 19 – September 1, 2020	(revision of S2-200xxxx)

Source:	Huawei (Rapporteur)

Title:	KI#1: Principles for categorization of Solutions for KI1

Document for:	Approval

Agenda Item:	8.9

Work Item / Release:	FS_5MBS / Rel-17

Abstract: This document is trying to find the common aspects of the solutions and achieve the interim conclusions.

# 1. Introduction/Discussion

# This document captures specific principles for 5G MBS.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.757.

# * * * * First change * * * *

# 7	Evaluations

## 7.X	Evaluation for Key Issue #1: MBS session management

Evaluation should be based on the following principles for this study:

- For multicast solutions, signalling from the UE to the network to join a multicast session shall be supported by UE and network.

Editor’s Note: It is FFS if the network and UE shall support multicast session join/leave operation via CP (NAS) signalling and/or via UP e.g. IGMP Join/Leve.

- For N3 transport of the shared delivery method, GTP-U tunnelling using a transport layer IP multicast method or shared N3 (GTP-U) Point-to-Point tunnel shall be supported with support for QoS.

- For N6 connectivity, solutions shall support tunnelling of MBS data towards the external network and untunnelled MBS data (IP multicast based) towards the external network.

- Solutions of KI#1 shall consider UE Authorization for multicast.

- At least one QoS Flow per MBS Session shall be supported.

Editor’s Note: Whether to limit to one QoS Flow per MBS Session is FFS.

- Both 5GC Shared MBS traffic delivery method and 5GC Individual MBS traffic delivery method shall be standardized for multicast data delivery within RAN node supporting 5G MBS .

- ..

- The network shall be able to prepare and start multicast traffic transmission for a MBS session after MBS service is requested and started from AF.

- The network shall support selection of MB-SMF or SMF (depending on architecture) at session start. The network shall support selection of MB-SMF or SMF (depending on architecture) by AMF at session join.

# * * * * End of changes * * * *