3GPP TSG-WG SA2 Meeting #140E e-meeting 	S2-2005415

Elbonia, August 19 – September 2, 2020	(revision of S2-200xxxx)

# Title:	[DRAFT] LS on RAN impact of FS_5MBS Study

# Release:	Release 17

# Work Item:	FS_5MBS, NR_MBS-Core

Source:	[Huawei to be] SA2

To:	RAN2, RAN3

Cc:

Contact Person:

Name:	Meng Li

Tel. Number:

E-mail Address:	Raymond DOT limeng AT huawei DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	TR 23.757 v0.4.0 (version before SA2#140e)

Note: TR 23.757 v0.5.0 containing the results of SA2#140e will be available shortly after SA2#140E

1. Overall Description:

SA2 has been discussing on Rel-17 FS_5MBS solutions since SA2 #135 meeting in TR 23.757.

SA2 would like to kindly inform RAN2 and RAN3 the following interim agreements in SA2:

-	SA2 will provide means to provide QoS requirements for an MBS Session to RAN nodes.

-	SA2 agrees that for each MBS session an own shared N3 tunnel will be used for DL MBS data delivery from MB-UPF to NG-RAN nodes. Either IP multicast transport or point-to-point unidirectional shared N3 tunnels can be used from MB-UPF to NG-RAN nodes.

SA2 would like to inform that no solution or combination of solutions has been adopted yet, and would like to receive feedback from RAN2 and RAN3 on the RAN impacts produced by some of the company proposed solutions.

SA2 would like to kindly ask RAN2 and RAN3 the following questions:

- SA2 agreed that the UE shall be able to receive on-going data of a multicast MBS session while in CM-CONNECTED state.  SA2 would like to ask whether the UE can receive on-going data of a multicast MBS session also while in CM-IDLE state . SA2 would appreciate RAN2 and RAN3 feedback on this topic.

- As one approach, UEs are receiving multicast data only in CM-CONNECTED. It was also proposed that a UE in an MBS session should be able to transit into CM-IDLE while no multicast data are received. SA2 would appreciate RAN2 and RAN3 feedback on this topic.

- SA2 has studied Xn/N2 handover solutions in the SA2 study documented in the TR. SA2 would appreciate RAN2 and RAN3 feedback on these. In particular, is temporary MBS data forwarding from S-RAN to a T-RAN supporting MBS desired?

- The SA2 study also includes some solutions for broadcast transmission. Do RAN2 and RAN3 intend to also support broadcast transmission (i.e. transmission of data without knowledge about whether there are UEs receiving the broadcasted data) in Rel-17?

2. Actions:

To RAN2 and RAN3 group.

ACTION:

SA2 respectfully requests RAN2 and RAN3 to take the above information into account and to answer the questions raised by SA2 and to provide any other feedback to SA2.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #141E		October 12 – 23, 2020			Elbonia

TSG-SA2 Meeting #142E		November 16 – 20, 2020			Elbonia