SA WG2 Meeting #140E	S2-2005502r01

19 Aug – 2 Sep, 2020 Elbonia	(revision of S2-20nnnnn)

Source:	MediaTek Inc.

Title:	KI #1: Evaluation and interim conclusion on the usage of Home SP SIB indication

Document for:	Approval

Agenda Item:	8.2

Work Item / Release:	FS_eNPN / Rel-17

Abstract: This contribution provides evaluation and conclusion for KI#1.

# 7	Evaluation

## 7.X	Key Issue #1: Enhancements to support SNPN along with credentials owned by an entity separate from the SNPN

Editor's note:	This clause will provide a general evaluation and comparison of the solutions per Key Issue #<X>.

### 7.X.1	SNPN Subscription Owner indication

The SNPN Subscription Owner (SSO) indication broadcast by an SNPN cell is to indicate a UE whether accessing this cell is allowed using the UE’s SSO subscription.

If such indication is not introduced, the UE may select any suitable SNPN cell and attempt registration which may fail. This may lead to a lot of wasted signalling and wasted UE battery power both of which are highly undesirable.

It is highly desirable to prevent the above from happening and therefore to control at the Access Stratum whether UE access is allowed using its SSO subscription – said otherwise a UE not having a subscription to a cell’s SNPN would not consider this cell suitable unless it is broadcasting this indication.

### 7.X.2	Conditions for using SNPN Subscription Owner  indication

The Table below shows the conditions that an SNPN cell broadcasts the SSO indication only if it supports to use SSO subscription to access. UE using SSO subscription in SNPN access mode shall select an SNPN cell only when this SNPN cell broadcasts SSO indication (or when the cell’s SNPN is the UE’s SSO).

Table 7.X.2-1 Conditions for using SCO Indication

|  | Rel-16 UE configured with SNPN IDs (e.g., SNPN 1, SNPN 2, SNPN 3) | Rel-17 UE configured with the SSO ID (e.g. SNPN 4) and SNPNs supporting SSO subscription (e.g., SNPN 1, SNPN 5) |
| --- | --- | --- |
| Condition 1: an SNPN cell (e.g., SNPN 1 cell) broadcasts the Rel-16 information (e.g. SNPN ID = SNPN 1) | UE in SNPN access mode is allowed to select this SNPN cell (e.g. SNPN 1 cell) and registers to this SNPN (e.g., SNPN 1) UE not in SNPN access mode is not allowed to select this SNPN cell (e.g. SNPN 1 cell) | UE in SNPN access mode is not allowed to select this SNPN cell (e.g. SNPN 1 cell) UE not in SNPN access mode is not allowed to select this SNPN cell (e.g. SNPN 1 cell) |
| Condition 2: an SNPN cell (e.g., SNPN 1 cell) broadcasts the Rel-16 information for SNPN 1 (e.g., SNPN ID = SNPN 1) and Rel-17 Information (SSO indication) |  | UE in SNPN access mode is allowed to select this SNPN cell (e.g. SNPN 1 cell) and registers to this SNPN (e.g. SNPN 1) using the SSO (e.g. SNPN 4) UE not in SNPN access mode is not allowed to select this SNPN cell (e.g. SNPN 1 cell) |

# 8	Conclusions

## 8.X	Key Issue #1: Enhancements to support SNPN along with credentials owned by an entity separate from the SNPN

Editor's note:	This clause will capture conclusions for Key Issue #<X>.

According to the above evaluation on KI#1, the following interim conclusion is derived:

-	A Rel-17 SNPN cell shall broadcast (SIB) whether it supports access using SNPN Subscription Owner (SSO) subscription. If the indication is not set or is not included the UE shall assume access using the SSO subscription is not allowed.