SA WG2 Meeting #140E	S2-2005503

19 Aug – 2 Sep, 2020 Elbonia	(revision of S2-20nnnnn)

Source:	MediaTek Inc.

Title:	KI #4: Evaluation and interim conclusion

Document for:	Approval

Agenda Item:	8.2

Work Item / Release:	FS_eNPN / Rel-17

Abstract: This contribution provides evaluation and conclusion for KI#4.

# 7	Evaluation

## 7.X	Key Issue #4: UE Onboarding and remote provisioning

Editor's note:	This clause will provide a general evaluation and comparison of the solutions per Key Issue #<X>.

In KI#4, it has two components for the clarification of the issues. Component 1 is to address the issues for UE Onboarding to get 3GPP connectivity and Component 2 is to address the issues for UE provisioning of NPN subscription/credentials to allow access to NPN services.

Solutions proposed to address the UE Onboarding are evaluated as follows:

-	UE Onboarding for a SNPN

-	Solutions 33/34/39 are proposed to use the Default UE credentials set to PLMN credentials to get 3GPP connectivity

-	Solutions 5/27/29/30/31/35/36/37/40 are proposed to use the Default UE credentials not set to PLMN credentials and the UE is configured with the parameters (e.g., SNPN ID list, S-NSSAIs, DNNs) to get 3GPP connectivity

-	UE Onboarding for PNI-NPN

-	The PNI-NPN can get 3GPP connectivity via PLMN subscription/credentials. Therefore is no Onboarding for PNI-NPN Onboarding

Solutions proposed to address the UE provisioning are evaluated as follows:

-	provisioning the NPN subscription/credentials to the UE can use Control Plane (CP) provisioning or User Plane (UP) provisioning regardless of which Onboarding method is used.

-	a single solution shall be specified in normative work.

# 8	Conclusions

## 8.X	Key Issue #4: UE Onboarding and remote provisioning

Editor's note:	This clause will capture conclusions for Key Issue #<X>.

According to the above evaluation on KI#4, the following interim conclusion is derived:

Principles for the UE Onboarding are evaluated as follows:

-	UE Onboarding for a SNPN

-	the UE is equipped with Default UE credentials

-	use the default credentials set to PLMN credentials to get 3GPP connectivity

-	use the default credentials set to SNPN credentials and the UE is configured with the parameters (e.g., SNPN ID list, S-NSSAIs, DNNs) to get 3GPP connectivity

-	UE Onboarding for PNI-NPN

-	the PNI-NPN can get 3GPP connectivity via PLMN subscription/credentials. Therefore is no Onboarding for PNI-NPN Onboarding

Principles for the UE provisioning are evaluated as follows:

-	using Control Plane (CP) provisioning or User Plane (UP) provisioning is regardless of which Onboarding procedure is used

-	a single solution for UE provisioning is selected and specified in normative work