3GPP TSG-WG SA2 Meeting #140E e-meeting  	S2-2005542r01

# Elbonia, August 19 – September 1, 2020	(revision of S2-200xxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.287 | CR | 0148 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on PC5 RAT selection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eV2XARC |  |  |  |  |  | Date: |  |  | 2020-08-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | During the study phase, SA2 and RAN WGs exchanged LS on PC5 RAT selection mechanism in S2-1901380. However, such PC5 RAT selection mechanism is not well written in the spec. In addition, the following description (introduced by S2-2003418 at SA2#138E) doesn’t reflect the case when both PC5 RAT types are selected. 3) Policy/parameters for PC5 RAT selection and for PC5 Tx Profile selection: - The mapping of V2X service types (e.g. PSIDs or ITS-AIDs) to PC5 RAT (e.g. LTE PC5 or NR PC5), and to the corresponding Tx Profiles (see TS 36.300 [9] and TS 38.300 [11] for further information). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Corret the unclear and wrong description |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear and wrong description |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.1.2, 5.1.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r01: minor chages regarding 5QI |  |  |  |  |  |  |  |  |

*** 1st change ***

5.2.1.2	Broadcast mode communication over PC5 reference point

Broadcast mode of communication is supported over both LTE based PC5 reference point and NR based PC5 reference point. Therefore, when broadcast mode is selected for transmission over PC5 reference point, PC5 RAT selection needs to be performed based on configuration described in clause 5.1.2. The V2X layer in the UE determines, based on UE configuration described in clause 5.1.2 and UE capabilities, the applicable PC5 RAT(s) for any particular V2X service and passes the packet to the applicable PC5 AS(Access Stratum) layer(s). If the V2X layer selects multiple PC5 RATs for a V2X service, it passes the packets of this V2X service to each AS layer of the selected PC5 RATs with the appropriate PC5 QoS parameters (i.e. PPPP/PPPR for LTE RAT, PQI and and conditionally other parameters such as MFBR/GFBR, etc. for NR RAT).

For LTE based PC5 reference point, broadcast mode is the only supported communication mode, and the operation details are defined in TS 23.285 [8].

For NR based PC5 reference point, the broadcast mode also supports enhanced QoS handling as defined in clause 5.4.1.

*** 2nd change ***

5.1.2	Authorization and Provisioning for V2X communications over PC5 reference point

5.1.2.1	Policy/Parameter provisioning

The following sets of information for V2X communications over PC5 reference point is provisioned to the UE:

1)	Authorization policy:

-	When the UE is "served by E-UTRA" or "served by NR":

-	PLMNs in which the UE is authorized to perform V2X communications over PC5 reference point when "served by E-UTRA" or "served by NR".

For each above PLMN:

-	RAT(s) over which the UE is authorized to perform V2X communications over PC5 reference point.

-	When the UE is "not served by E-UTRA" and "not served by NR":

-	Indicates whether the UE is authorized to perform V2X communications over PC5 reference point when "not served by E-UTRA" and "not served by NR".

-	RAT(s) over which the UE is authorized to perform V2X communications over PC5 reference point.

NOTE 1:	In this specification, {When the UE is "served by E-UTRA" or "served by NR"} and {When the UE is "not served by E-UTRA" and "not served by NR"} are relevant to V2X communications over PC5 reference point.

2)	Radio parameters when the UE is "not served by E-UTRA" and "not served by NR":

-	Includes the radio parameters per PC5 RAT (i.e. LTE PC5, NR PC5) with Geographical Area(s) and an indication of whether they are "operator managed" or "non-operator managed". These radio parameters (e.g., frequency bands) are defined in TS 36.331 [14] and TS 38.331 [15]. The UE uses the radio parameters to perform V2X communications over PC5 reference point when "not served by E-UTRA" and "not served by NR" only if the UE can reliably locate itself in the corresponding Geographical Area. Otherwise, the UE is not authorized to transmit.

NOTE 2:	Whether a frequency band is "operator managed" or "non-operator managed" in a given Geographical Area is defined by local regulations.

3)	Policy/parameters for PC5 RAT selection and for PC5 Tx Profile selection:

-	The mapping of V2X service types to PC5 RATs (e.g. LTE PC5, NR PC5 or both), and to the corresponding Tx Profiles (see TS 36.300 [9] and TS 38.300 [11] for further information).

4)	Policy/parameters related to privacy:

-	The list of V2X service types, with Geographical Area(s) that require privacy support.

-	A privacy timer value indicating the duration after which the UE shall change each source Layer-2 ID self-assigned by the UE when privacy is required.

5)	Policy/parameters when LTE PC5 is selected:

Same as specified in TS 23.285 [8] clause 4.4.1.1.2 item 3) Policy/parameters except for the mapping of V2X service types to Tx Profiles and the list of V2X services with Geographical Area(s) that require privacy support.

6)	Policy/parameters when NR PC5 is selected:

-	The mapping of V2X service types to V2X frequencies with Geographical Area(s).

-	The mapping of V2X service types to the default mode of communication (i.e. broadcast mode, groupcast mode or unicast mode).

-	The mapping of V2X service types to Destination Layer-2 ID(s) for broadcast.

-	The mapping of V2X service types to Destination Layer-2 ID(s) for groupcast mode communication.

-	The mapping of V2X service types to default Destination Layer-2 ID(s) for initial signalling to establish unicast connection.

NOTE 3:	The same default Destination Layer-2 ID for unicast initial signalling can be mapped to more than one V2X service types. In the case where different V2X services are mapped to distinct default Destination Layer-2 IDs, when the UE intends to establish a single unicast link that can be used for more than one V2X service types, the UE can select any of the default Destination Layer-2 IDs to use for the initial signalling.

-	The mapping of V2X service types to PC5 QoS parameters defined in clause 5.4.2 (i.e. PQI and conditionally other parameters such as MFBR/GFBR, etc.).

-	AS layer configurations (see TS 38.331 [15]), e.g. the mapping of PC5 QoS profile(s) to radio bearer(s), when the UE is "not served by E-UTRA" and "not served by NR".

-	The PC5 QoS profile contains PC5 QoS parameters described in clause 5.4.2, and value for the QoS characteristics regarding Priority Level, Averaging Window, Maximum Data Burst Volume if default value is not used as defined in Table 5.4.4-1.

7)	Validity timer indicating the expiration time of the V2X Policy/Parameter.

The above parameter sets from bullet 2) to 6) may be configured in the UE through the V1 reference point by the V2X Application Server.

*** End of the changes ***