SA WG2 Meeting #140e	S2-2005561r01

Aug 19 – Sep 01, 2020, Elbonia

Source:	OPPO, Convida Wireless

Title:	KI#8, Update to solution#28 for clarification

Document for:	Approval

Agenda Item:	8.1

Work Item / Release:	FS_eNA_ph2 / Rel-17

Abstract of the contribution:  It is to update solution#28 in TR 23700-91.

1. Introduction

Updating the solution#28 in TR 23700-91 a following:

- illustrate only the UE knows the enforcement of URSP. Neither core network nor application server realize if the an URSP rule is used by UE as expected.

- Update the table for input data and output data for easier reading.

- adding a procedure for better illustration

- adding description of impact

2. Proposal

The following changes are proposed for solution#28 in TR 23.700-91.

* * * * Start of Change* * * *

6.28	Solution #28: UE assisted analysis for usage of network slice

6.28.1 	Description

6.28.1.1 	General

This solution is proposed to address KI#8.

URSP is a key point for network slicing service. The URSP evaluation is a behaviour in NAS layer (an example is shown in figure-6.28.1.1), however as described in sub-clause 6.6.2.3 in TS 23.503, the UE may associate the application traffic to different PDU session (incl. S-NSSAI) per time, location, URSP enforcement, etc. Also, as described in subcaluse 4.2.3 in TS24.526, UE shall ignore the unknown or unexpected URSP rules when evaluating the URSP rules to associate an application to PDU session.

For the UE behaviour mentioned above, UE does not notify the network the result of URSP evaluation. In other words, once the URSP is distributed to UE, neither core network nor the application server knows which URSP rule(s) are enforced. Thus, the network cannot know whether the URSP rules (e.g. the parameters in URSP rule, the Rule precedence) are set properly.

Figure 6.28.1.1   Example of URSP evaluation

The network may need to verify that the traffic is routed over a PDU session that complies with the characteristics sent in the URSP rule. Although the NWDAF may receive information such as service behaviour analytics and/or service experience analytics from an AF, service behaviour analytics and/or service experience analytics is not sufficient for the network determine if the URSP rules that are installed in a UE need to be updated. In order verify proper URSP enforcement and optimize the distribution of URSP, the NWDAF needs to collect data from UE and other NF about how URSP rules are being applied by the UE and what URSP rules are installed. The NWDAF can provide the PCF information about how URSP rules are being applied and the PCF uses this information to check that the UE is correctly applying URSP Rules and decide if new URSP rules need to be sent to the UE or if URSP rules need to be removed from the UE.

It is proposed that UE provides data related to usage of URSP to NWDAF. To achieve it, a new container can be used for direct communication between UE and NWDAF. Thus the UE can include the data into the container and use the NAS message to send the container to NWDAF. Also, NWDAF may include an indication that a URSP Enforcement Report is desired in the Container. The indication may be used to indicate two options:

1)  When URSP (re)evaluation happens, UE sends URSP Enforcement Report to NWDAF (e.g. the newly adopted URSP rules/RSDs) to NWDAF if the association between Application and PDU session is changed. Or

2) UE stores the URSP Enforcement Report when it receives the indication and sends to NWDAF every time it further receives the indication.

The URSP enforcement report can indicate the time information as well.

NOTE: there could be an alternative way for data transfer between UE and NWDAF, which is depending on the conclusion of KI#8 (how to transfer data between UE and NWDAF.

The NWDAF collects the data from UE and other NF, to derive the analytics for usage of network slice.

6.28.1.2 	Input data

Since the URSP evaluation is a behaviour in NAS layer, once the URSP is distributed to UE, neither core network nor the application server knows which URSP rule(s) are enforced. In order to verify proper URSP enforcement and optimize the distribution of URSP, NWDAF shall be able to collect data from UE and other NF, as described below:

Table 6.28.1.2: data for URSP enforcement collected by NWDAF

| Information | Source | Description |
| --- | --- | --- |
| URSP Enforcement Report | UE | The UE sends a URSP Enforcement Report based on an indication from network. The indication is sent by NWDAF. It can indicate the time information. |
| The mapping between traffic descriptor and the Application(s) | PCF/AF | There can be multiple traffic descriptors are corresponding to a certain application. PCF/AF can notify which application a Traffic Descriptor is corresponding to. |
| The URSP rules distributed to UE | PCF/AF | URSP may be updated. PCF/AF can notify which URSP rules are distributed to UE per time/location. |

6.28.1.3 	Output Analytics

NWDAF can analyse the data collected from UE and NFs and provide the output that is shown in the table below. The output analytics may be used by the PCF to check that the UE is correctly applying URSP Rules and decide if new URSP rules need to be send to the UE or URSP rules need to be removed from the UE.

Table 6.28.1.3: statistics for UE’s usage of Network slice

| Analytics |  | Description |  |
| --- | --- | --- | --- |
| List of URSP rule/RSDs | The URSP rule/RSDs distributed to UE |
| Applied URSP rule/RSDs | The applied URSP rule/RSDs by UE |
| Which network slice(s) (S-NSSAI) for a given application are used. |  | Due to URSP evaluation, the UE does not always use the most prioritized or latest URSP rule/RSDs. This entry provides the statistics about if and how often for an S-NSSAI is used for a given UE. |  |
| How long for a network slices is used for the application. |  |  |  |
| Time information |  | Indicates the Analytics target period |  |

Table 6.28.1.4: Prediction for UE’s usage of Network slice

| Analytics | Description |
| --- | --- |
| List of URSP rule/RSDs for distribution | The URSP rule/RSDs to be distributed to UE |
| Applied URSP rule/RSDs | The URSP rule/RSDs to be applied by UE |
| Which network slice(s) (S-NSSAI) for a given application are used. | Due to URSP evaluation, the UE does not always use the most prioritized or latest URSP rule/RSDs. This entry provides the statistics about if and how often for an S-NSSAI is used for a given UE. |
| How long for a network slices is used for the application. |  |
| Time information | Indicates the Analytics target period |
| Confidence |  | Confidence of the prediction |  |

The NWDAF can share the analytics output (statistics and/or predictions) to consumers (e.g. PCF and AF). And based on the output, PCF can aware the enforcement of URSP rules and adjust the URSP rules accordingly. For example, if an URSP rule is never used for a certain UEs, then it might be due to the Traffic Descriptor is misconfigured and MNO, OTT, UE vendor may collaborate to fix it.

6.28.1.4 	Procedure

Figure 6.28.1.4 Procedure for analytics of usage of network slice and update URSP to UE accordingly

1. NWDAF collects data for URSP enforcement from UE as described in Table 6.28.1.2.

2. NWDAF may collects data from PCF and/or AF as described in Table 6.28.1.2.

3. Based on the input in step-1 and 2, NWDAF derives the analytics result as described in Table 6.28.1.3.

4. The analytic result can be exposed to PCF and/or AF. PCF and AF may interact to update the URSP rules. Then PCF may distribute the updated URSP to UE using UCU procedure as specified in sub-clause 4.2.4.3 in TS 23.502.

6.28.2 	Impacts on services, entities and interfaces

UE:

Provides the data for URSP enforcement as described in Table 6.28.1.2;

PCF/AF:

Provides the the URSP rules distributed to UE and the mapping between traffic descriptor and the Application(s) as described in Table 6.28.1.2;

NWDAF:

- Support analytics of usage of network slice based on input from UE and PCF/AF as described in Table 6.28.1.3

- Exposure the analytics to other NF (e.g. PCF and AF);

AMF:

Support to transfer a new container (i.e. a new value for payload container) between UE and NWDAF.

* * * * End of Change * * * *