SA WG2 Meeting #S2-140E	S2-2005582r09

19 Aug - 2 Sep, 2020, Elbonia

Source: 	Ericsson

Title: 	KI#2: Evaluation and conclusion

Document for: 	Approval

Agenda Item: 	8.2

Work Item / Release:	FS_eNPN / Rel-17

Abstract of the contribution: This paper proposes the evaluation and conclusion for KI#2

# Discussion

# Proposal

Add the following changes to TR 23.700-07.

*** BEGIN CHANGES ***

# 3	Definitions of terms and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Default UE credentials: Information that the UE have before the actual onboarding procedure to make it uniquely identifiable and verifiably secure.

Default Credential Server (DCS): The server that can authenticate a UE with default UE credentials or provide means to another entity to do it.

NPN: Non-Public Network as defined in TS 23.501 [4]. The terminology NPN refers to both SNPN and PNI-NPN in this TR unless otherwise stated.

NPN credentials: Information that the UE uses for authentication to access a NPN. NPN credentials may be 3GPP credentials or non-3GPP credentials.

Onboarding Network (ON): The network providing initial registration and/or access to the UE for UE Onboarding.

Provisioning Server: The server that provisions the authenticated/authorized UE with the subscription data and optionally other configuration information.

Subscription Owner (SO): The entity that stores and as result of the UE Onboarding procedures provide the subscription data and optionally other configuration information via the PS to the UE.

UE Onboarding: Provisioning of information, to a UE and within the network, required for the UE to get authorized access and connectivity to an NPN.

Unique UE identifier: Identifying the UE in the network and the DCS and is assigned and configured by the DCS.

NOTE 1:	The unique UE identifier is assumed to be unique within the DCS. It takes the form of a Network Access Identifier (NAI) using the NAI RFC 7542.

NOTE 2:	the definition of terms has the scope to provide a common language compared to the definitions in specific solutions. It is up to solutions to use the common terms, when applicable.

*** NEXT CHANGES ***

# 7	Evaluation

# 8	Conclusions

## 8.X	Key Issue #2: NPN support for Video, Imaging and Audio for Professional Applications (VIAPA)

Editor's note:	These are INTERIM conclusions for Key issue #2.

When UE only has single subscription, the data service from both V-SNPN and Home SP (PLMN or Home SNPN), as well as service continuity is to be evaluated by KI#1.

It is concluded that the existing Rel-16 N3IWF-architecture is used as the basis to address data service from both networks and service continuity between the two networks.

*** END CHANGES ***