SA WG2 Meeting #S2-140E	S2-2005582r15

19 Aug - 2 Sep, 2020, Elbonia

Source: 	Ericsson, Huawei, HiSilicon

Title: 	KI#2: Evaluation and conclusion

Document for: 	Approval

Agenda Item: 	8.2

Work Item / Release:	FS_eNPN / Rel-17

Abstract of the contribution: This paper proposes the evaluation and conclusion for KI#2

# Discussion

# Proposal

Add the following changes to TR 23.700-07.

*** BEGIN CHANGES ***

# 3	Definitions of terms and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Default UE credentials: Information that the UE have before the actual onboarding procedure to make it uniquely identifiable and verifiably secure.

Default Credential Server (DCS): The server that can authenticate a UE with default UE credentials or provide means to another entity to do it.

NPN: Non-Public Network as defined in TS 23.501 [4]. The terminology NPN refers to both SNPN and PNI-NPN in this TR unless otherwise stated.

NPN credentials: Information that the UE uses for authentication to access a NPN. NPN credentials may be 3GPP credentials or non-3GPP credentials.

Onboarding Network (ON): The network providing initial registration and/or access to the UE for UE Onboarding.

Provisioning Server: The server that provisions the authenticated/authorized UE with the subscription data and optionally other configuration information.

Subscription Owner (SO): The entity that stores and as result of the UE Onboarding procedures provide the subscription data and optionally other configuration information via the PS to the UE.

UE Onboarding: Provisioning of information, to a UE and within the network, required for the UE to get authorized access and connectivity to an NPN.

Unique UE identifier: Identifying the UE in the network and the DCS and is assigned and configured by the DCS.

NOTE 1:	The unique UE identifier is assumed to be unique within the DCS. It takes the form of a Network Access Identifier (NAI) using the NAI RFC 7542.

NOTE 2:	the definition of terms has the scope to provide a common language compared to the definitions in specific solutions. It is up to solutions to use the common terms, when applicable.

Overlay network: When UE is accessing SNPN service via PLMN, SNPN is the overlay network. When UE is accessing PLMN services via SNPN, PLMN is the overlay network.

Underlay network: When UE is accessing SNPN service via PLMN, PLMN is the underlay network. When UE is accessing PLMN services via SNPN, SNPN is the underlay network.

*** NEXT CHANGES ***

# 7	Evaluation

## 7.X	Key Issue #2: NPN support for Video, Imaging and Audio for Professional Applications (VIAPA)

The evaluation is based on major principles from the KI#2 solutions. The evaluation separates to different areas for each principle as below table:

Table 7.x-1: Evaluation of KI#2 related principles

| Principle | What it solves | Benefit, drawback | System impact | Open issues | Reference |
| --- | --- | --- | --- | --- | --- |
| N3IWF architecture | Service continuity aspect. Simultaneous data service aspect. | Benefit: Drawback: | Based on the existing release 16 architecture, No new architecture impact |  | Solution #13,#14,#15,17,#18. S2-2005532 |
| N3IWF architecture with dual radio capability (2RX/2TX) UE enhancement. | Same as N3IWF architecture. | Benefit: Drawback: |  |  | Solution #13,#15 |
| N3IWF architecture with application layer service continuity support | Same as N3IWF architecture | Benefit: Drawback: |  | NOTE: There is no solution based on this principle documented in the TR yet. | Solution S2-2005532 |
| N3IWF architecture with enhancement to keep UE in CM-Connected state in overlay network. | Paging aspect. | Benefit: Drawback: | UE logic to keep one PDU session in underlay network. Existing liveness check tool to be used over NWu interface to keep the NWu connection open. Apply to single radio UE |  | Solution #13,#14 |
| N3IWF architecture with enhancement to keep UE in CM-Connected state in both overlay network and underlay network. | Paging aspect. | Benefit: Drawback: | RAN and CN impact to keep UE always in RRC-Inactive in the underlay network. PDU session in underlay network remains active. Support single radio UE | Whether it is necessary to keep UE in CM-Connected in underlay network is FFS. | Solution #14 |
| N3IWF architecture with enhancement to introduce "inactive" mode on NWu interface. | Paging aspect. | Benefit: Drawback: |  | IKEv2 impact is FFS. How does the DL IPSec data reach UE when NAT timeout is FFS. | Solution #17 |
| N3IWF architecture with enhancement to split DL traffic to Uu interface and UL traffic to NWu interface for 2RX/1TX UE. | Simultaneous data service aspect. | Benefit: Drawback: | RAN impact for uplink control communication with single Tx UE/AMF impact to support split indication. UE/SMF/UPF impact to support user data plane traffic path steering. |  | Solution #18 |
| N3IWF architecture with enhancement to enable underlay network to subscribe "paging/downlink data notification" service from the overlay network via exposure function. | Paging aspect. | Benefit: Drawback: |  | NOTE: There is no solution based on this principle documented in the TR yet. | S2-2005672 |
| N3IWF architecture with enhancement to assist UE to choose either SNPN or PLMN to use Uu interface for better QoS. | Simultaneous data service aspect. | Benefit: Drawback: |  | NOTE: There is no solution based on this principle documented in the TR yet. | S2-2005296 S2-2005124 |
| Inter-PLMN like architecture between SNPN and PLMN. | Service continuity aspect. | Benefit: Drawback: | To be concluded from KI#1 evaluation. |  | Solution #1, #2. |
| N3IWF architecture with MUSIM like implementation | Concurrent paging and data service for single radio UE. | Benefit: Drawback: |  | The principle is not clearly stated in the solution. | Solution #15. |

# 8	Conclusions

## 8.X	Key Issue #2: NPN support for Video, Imaging and Audio for Professional Applications (VIAPA)

Editor's note:	These are INTERIM conclusions for Key issue #2.

When UE only has single subscription, the data service from both V-SNPN and Home SP (PLMN or Home SNPN), as well as service continuity is to be evaluated by KI#1.

It is concluded that the existing Rel-16 N3IWF-architecture is used as the basis to address data service from both networks and service continuity between the two networks.

Editor’s note: With Rel-16 N3IWF architecture, how is it ensured that the service obtains QoS in the underlay network when VIAPA applications (or even voice services/emergency services for KI#3) needs to be supported.

*** END CHANGES ***