3GPP TSG-SA WG2 Meeting #140E 	S2-2005591

19 August – 2 September, 2020, Elbonia	 (revision of S2-200xxxx)

Source:	CATT

Title:	KI # 2&3, New Sol: UAV Re-authentication and Re-authorization

Document for:	Approval

Agenda Item:	8.7

Work Item / Release:	FS_ID_UAS-SA2 / Rel-17

Abstract of the contribution: this paper proposes to add a new solution related to UAV re-authentication and re-authorization in TR 23.754.

Discussion

This pCR proposes new solution related to UAV re-authentication and re-authorization. In this solution:

-	The UTM can trigger UAV re-authentication and re-authorization. In 5GS, the AMF triggers UUAA procedure to re-authenticate and re-authorize the UAV. In EPS, the PGW initiates bearer modification procedure to transfer the re-authentication and re-authorization information via PCO.

-	The UTM can update the UAV and UAVC pairing authorization and RITI. In 5GS, SMF could be used to transfer the updated authorization data and RITI to UAV. In EPS, the PGW initiates bearer modification procedure to transfer the authorization data and RITI via PCO to UAV.

Proposal

It is proposed to add a new solution related to UAV re-authentication and re-authorization.

First change

6.X	Solution #X: UAV Re-authentication and Re-authorization

6.X.1	Introduction

This solution addresses Key Issue #2 and #3 to support UAV Re-authentication and Re-authorization, UAV and UAVC pairing re-authorization.

This solution is applicable to EPC and 5GS.

6.X.2	Functional Description

The UTM can trigger UAV re-authentication and re-authorization at any time. In 5GS, the AMF triggers UUAA procedure to re-authenticate and re-authorize the UAV. In EPS, the PGW initiates bearer modification procedure to transfer the re-authentication and re-authorization information via PCO.

The UTM can update the UAV and UAVC pairing authorization and RITI at any time. In 5GS, the SMF could be used to transfer the updated authorization data and RITI to UAV. In EPS, the PGW initiates bearer modification procedure to transfer the authorization data and RITI via PCO to UAV.

6.X.3	Procedures

6.X.3.1	5GS procedures

6.X.3.1.1	UTM triggered UAV Re-authentication and Re-authorization in 5GS

Figure 6.X.3.1.1-1: UTM triggered UAV Re-authentication and Re-authorization

1.	The UTM requests the re-authentication and re-authorization for the UAV identified by the CAA-level UAV ID.

2-3.	UFES gets AMF ID from UDM using Nudm_UECM_Get with the CAA-level UAV ID in the received request in step 1. If UFES does not store the mapping between CAA-level UAV ID and 3GPP UAV ID, the 3GPP UAV ID is also included in the message of step 3.

4.	The UFES notifies Re-auth event to the AMF to re-authenticate/re-authorize the UAV using Nufes_UUAA_Notify with the 3GPP UAV ID and the UAV re-auth event.

5.	The AMF triggers the UUAA procedure.

6.X.3.1.2	UTM triggered UAV and UAVC pairing Re-authorization in 5GS

Figure 6.X.3.1.2-1: UTM triggered UAV Re-authentication and Re-authorization

1.	The UTM requests to update the UAV and UAVC pairing authorization data and RITI.

2-3.	UFES gets SMF ID from UDM using Nudm_UECM_Get service operation. If UFES does not store the mapping between CAA-level UAV ID and 3GPP UAV ID, the 3GPP UAV ID is also included in the message of step 3.

4.	The UFES notifies the UAV and UAVC pairing authorization data and RITI to the SMF to update the authorization using Nufes_UUAA_Notify.

5.	The SMF sends PDU Session Modification Command to UAV. The message includes the UAV and UAVC pairing authorization data and RITI.

6.	The UAV sends PDU Session Modification Command Ack to SMF.

6.X.3.2	EPS procedure: UTM triggered Re-authentication and Re-authorization in EPS

Figure 6.X.3.2-1: UTM triggered Re-authentication and Re-authorization in EPS

1.	The UTM sends UAV Re-auth request (CAA-level UAV ID, authorization data and RITI) to UFES.

2.	The UFES sends UAV operation request to PGW.

3.	The PGW initiated bearer modification procedure in clause 5.4.2.1 in TS 23.401 to send authorization data and RITI to UE via PCO.

4.	The PGW sends UAV operation response to UFES. The message includes the information that the UE provided in PCO.

5.	The UFES sends UAV Re-auth response to UTM. The message includes the information that the UFES received in step 4.

6.X.4	Impacts on services, entities and interfaces

For 5GS:

-	The AMF is enhanced to trigger UUAA when re-auth event or authorization data is received

-	The SMF sends authorization data and RITI to UE via PDU Session modification command.

For EPS:

-	The PDN GW initiates bearer modification procedure to transfer re-authentication and re-authorization data in PCO.

For both EPS and 5GS:

-	An UFES function is introduced as described above. The UFES may be mapped to existing functionality, e.g. SCEF, NEF, AAA-P depending on the specific implementation of the interfaces between the 3GPP system and the UTM.

End of changes