3GPP TSG-SA/WG2 Meeting #140e	S2-2005600

Elbonia, 19 Aug-1 Sep, 2020

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.401 | CR | 3612 | rev |  | Current version: | 15.11.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on Paging retransmission for WUS capable UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOTenh-Core, CIoT_ext |  |  |  |  |  |  | Date: |  | 2020-08-13 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In LS R2-2005985/ S2-2005090, RAN2 has identified another potential issue with the solution described in R2-2004317/S2-2003217 if it is possible for the eNB to release the RRC connection with RRCConnectionRelease without establishing/releasing the S1 connection (e.g. in case of CN overload or MME reset). The UE would be unreachable while it remains camped on this cell or till the UE establishes/resumes RRC connection (i.e. MO data, MO signalling). RAN2 assumes the same issue could also happen in 5GC. One possible solution for this issue would be that the MME doesn’t provide the Recommended Cells for Paging as Assistance Data for Recommended Cells IE in the retransmitted S1-AP Paging message. This will trigger the RAN to perform paging of the UE in cells which belong to tracking areas as indicated in the TAI List for Paging IE. This solution has no impact on stage 3. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that the MME shall not provides the Recommended Cells for Paging as Assistance Data for Recommended Cells IE in the retransmitted S1-AP Paging message to the RAN |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE would be unreachable while it remains camped on this cell |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.21.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR … CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

4.3.21.4.1	General

The RAN and UE may use a Wake Up Signal (WUS) to reduce the UE's idle mode power consumption. The RAN sends the WUS shortly before the UE's paging occasion. The WUS feature enables UEs to determine that in the paging occasions immediately following their WUS occasion they will not be paged if their WUS is not transmitted, or that they might be paged if their WUS is transmitted (see TS 36.304 [34]).

To avoid waking up UEs due to an MME paging other UEs across multiple cells (e.g. due to frequent UE mobility and/or for low paging latency services such as VoLTE), the use of WUS by the eNB and the UE is restricted to the last used cell, i.e. the cell in which the UE's RRC connection was last released. To support this:

a)	WUS-capable eNBs should provide the Recommended Cells for Paging IE in the Information On Recommended Cells And eNBs For Paging IE (see TS 36.413 [36]) to the MME in the S1 UE Context Release Complete or UE Context Suspend Request messages for WUS capable UEs;

b)	if received during the last S1 UE Context Release Complete or UE Context Suspend Request message, the MME provides (without modification) the Recommended Cells for Paging as Assistance Data for Recommended Cells IE in the S1-AP Paging message to the RAN (see also TS 36.413 [36]); and

c)	the MME shall delete (or mark as invalid) the Information On Recommended Cells And eNBs For Paging when a new S1-AP association is established for the UE; and

d)	based on the Paging strategies the MME may not provide the Recommended Cells for Paging as Assistance Data for Recommended Cells IE in the retransmitted S1-AP Paging message to the RAN.

In the S1-AP Paging message, the last used cell ID is sent in the Assistance Data for Recommended Cells IE in the Assistance Data for Paging IE (see TS 36.413 [36]). When receiving an S1-AP Paging message for a WUS-capable UE that also includes the Assistance Data for Recommended Cells IE then a WUS-capable eNB shall only broadcast the WUS on the cell that matches the last used cell ID.

* * * * End of Changes * * * *