SA WG2 Meeting SA2-140e	S2-2005685r14

19th August – 2nd September, 2020, E-meeting	(revision of S2-201xxxx)

Source:	Intel

Title:	KI #1, Proposed way forward

Document for:	Discussion / Approval

Agenda Item:	8.10

Work Item / Release:	FS_MUSIM / Rel-17

Abstract of the contribution: Proposes to interim conclusions on KI#1.

# 1	Discussion

During the FS_MUSIM conference call Intel proposed an evaluation of solutions and a way forward proposal on KI#1:

https://www.3gpp.org/ftp/tsg_sa/WG2_Arch/TSGS2_140e_Electronic/INBOX/CCs/2020_08_06_FS_MUSIM_PreSA2%23140E/INT%20S2-200xxxx_FS_MUSIM_wf.pptx

This contribution updates the way forward proposal and formats it to fit TR 23.761.

# 2	Proposal

It is proposed to update TR 23.761 as indicated below.

* * * * Next Change * * * *

# 7	Evaluation

Editor's note:	This clause will provide a general evaluation of the solutions.

* * * * Next Change * * * *

# 8	Conclusions

Editor's note:	This clause will capture conclusions from the study.

## 8.1	Key Issue #1: Handling of Mobile Terminated service with Multi-USIM device

Editor’s note: Additional solution principles can be selected for further evaluation.

Based on the evaluation in clause 7.1, the following interim concusions are agreed for the baseline functionality:

Among the KI#1 solution principles the following principles at least shall be considereded for further evaluation:

-	A Paging Cause for both EPS and 5GS indicating at least “voice” service.

Editor’s note: It will be determined whether the Paging Cause is applied only for UEs that have indicated MUSIM capability or to all UEs indicriminately.

Editor’s note:  Feedback from SA3 is expected whether there are any privacy issues with sending the Paging Cause as cleartext.

Editor’s note: Paging Cause values other than “voice” are FFS.

Editor's note:	RAN feedback is expected on Paging Cause as requested in the LS out (S2-200xxxx).

Editor's note:	Whether this solution can be acceptable from a system perspective if it is not possible to perform a Busy indication from a RAN perspective is TBD.

-	A Negotiated Short Absence for 5GS, evaluated together with RRC-based coordinated leaving, pending feedback from RAN WGs.

-	A Busy indication pending feedback from RAN WGs

-	MUSIM Assistance Information for paging filtering

Editor’s note:	The decision related to the other add-ons (Sol#9, Sol#11) and other potential solutions is postponed until Q4.

* * * * End of Change * * * *