3GPP TSG SA WG2 Meeting #140-e		S2-2005689

Electronic, 19 Aug – 02 Sep, 2020

Title:	LS on System support for Multi-USIM devices

Response to:

Release:	Rel-17

Work Item:	FS_MUSIM

Source:	SA2

To:	RAN2, RAN3

Cc:	-

Contact person:	Sašo Stojanovski

saso stojanovski intel com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

SA2 would like to inform RAN2 and RAN3 that they have progressed the study on FS_MUSIM (TR 23.761) and have agreed several interim conclusions on all three key issues. The interim conclusions list a set of principles that SA2 will consider for further study.

SA2 point out that these interim conclusions are subject to further revision taking into account feedback received from other WGs.

The table below lists the principles that are considered for further study and have RAN impact or require RAN feedback. The table also contains the applicability to key issues in TR 23.761, as well as questions to RAN WGs.

As a reminder, the relation between SA2 Study Key Issues and RAN WI Objectives is as follows:

- 	Key Issue #1: Objective #3

-	Key Issue #2: Objective #1

-	Key Issue #3: Objective #2

| Solution principle for further study in SA2 | Applicability to Key Issues | Question to RAN WGs |
| --- | --- | --- |
| Paging Cause (as in Solution #1) | KI#1. | Q1: Please confirm feasibility of sending a Paging Cause in [Uu] Paging message for EPS and for 5GS. [RAN2, RAN3] |
| Busy indication (as in Solution #3) | KI#1. | Q2: Please confirm it is good to respond to the paging and stop the paging escalation and save paging resources when the UE determines that it cannot engage in the MT triggered connection. [RAN2, RAN3] Q3: Solution 3 proposes to send the Busy Indication as a NAS message (no RAN impact). Please provide feedback if you think it is feasible and whether you see any optimization gains if the busy Indication is sent as an RRC Busy Indication [RAN2, RAN3] |
| RRC-based leaving.and returning with the following assumptions: - the leaving is initially considered as “short absence duration”. If the UE does not return within “short absence duration” the “short absence duration” is automatically converted into a “long absence duration”. The RRC states and respective procedures corresponding to “short” and “long” duration are to be determined by RAN WGs. NOTE: How short absence duration is negotiated between UE and Network, or if it is not required to be negotiated and its network controlled timer is FFS. - The UE uses the solution to perform a MO procedure (e.g. periodic mobility registration, keep-alive message, etc.) or a MT procedure (e.g. pick-up an SMS, inspect a MT service invite, respond to a network-initiated C-plane procedure, etc.) in the other network. - It is assumed that the RRC request for leaving is explicitly acknowledged by the network. However, in the absence of network acknowledgement the UE can still proceed with leaving. | KI#1, KI#3. | Q3: Please confirm feasibility of the RRC-based leaving and returning procedure in 5GS. [RAN2, RAN3] Q4: Please let us know whether changes to 5GS/E-UTRA (Option 5) to support this functionality are in scope of the RAN work item. [RAN2, RAN3] |

SA2 would also like to point out that TR 23.761 also contains several solutions to KI#2 requiring  RAN’s feedback. The solution principles in these solutions can be categorized as follows:

-	UE -requested 5G-GUTI reassignment using the Mobility Registration Update procedure (as in Sol#14 and Sol#20). It is noted that the Mobility Registration Update systematically requires 5G-GUTI reassignment by the network.

-    Changes related to the UE_ID (UE Identity Index) that is used for calculation of PF/PO. In EPS the UE Identity Index is provided by the MME to the eNB as part of the paging message. In 5GS the UE Identity Index value is provided by the AMF in the RRC-Inactive Assistance information but not in the paging message.:

-    Calculation of PF/PO by using an Alternative UE_ID which is not derived from either the UE’s GUTI or IMSI. (Sol#15) Proposed for both EPS and 5GS.

-    Calculation of PF/PO by using an UE_ID which is derived from IMSI+offset value. The offset value negotiated between UE and MME (Sol#16). Proposed for EPS

-    Calculation of PF/PO based on MUSIM Assistance Information (Sol#17)

-     Paging on consecutive POs as in Sol#18. For this solution SA2 was not clear whether the network requires an explicit MUSIM capability so that a different paging strategy can be applied to MUSIM- and non-MUSIM capable devices.

-	Implementation-based solution to address overlapping POs (Sol#19).

-	Access Stratum-based solution with scheduling gap (Sol#21).

Q5: SA2 would like to ask RAN2 whether Solution #14-#20 are feasible for effective avoidance of paging collision for the RAT combination scenarios to which each of them applies.

NOTE:	While the 5G-GUTI reassignment principle in Sol#14 and Sol#20 has no RAN impact, SA2 welcomes any RAN2 feedback on the assumption that no assistance information is needed from the UE in order to minimize the probability that the new 5G-GUTI will also result in PO collisions.

Q6:  SA2 would like to ask RAN2 whether Solution #15-#19 are compliant with the assumption “no EUTRA related impact”? And SA2 also would like RAN2 and RAN3 to clarify the exact meaning of “no EUTRA related impact”, e.g. whether it also means no impact on S1/N2 interface and UE behaviour defined in 36.304, 36.413 and 38.413 or the meaning is restricted to impacts on Uu only.

Qx: Solution 18 increases the use of paging resources especially for UE with idle mode mobility. SA2 would like to ask RAN2 to provide feedback on the increased use of paging resources in the system.

Q7: SA2 would like to ask RAN2 and RAN3 to take these solutions into consideration and provide any other feedback or recommendation on the applicability of these solutions, or any other Access Stratum-based solutions for KI#2.

# 2	Actions

To RAN2, RAN3

ACTION: 	SA2 kindly asks RAN2 and RAN3 to take into consideration the information above and provide answers to questions Q1, Q2Q3, Q4, Q6and Q7.

To RAN2

ACTION: 	In addition, SA2 kindly asks RAN2 to provide answers to questions Q5.

# 3	Dates of next TSG SA WG 2 meetings

SA2#141E		12-23 October				Electronic meeting

SA2#142E		16-20 November			Electronic meeting