SA WG2 Meeting #140e	S2-2005700r02

19 August – 02 September 2020, e-meeting	(revision of S2-200xxxx)

Source:	InterDigital

Title:	Conclusions for KI#4

Document for:	Approval

Agenda Item:	8.7

Work Item / Release:	[FS_ID_UAS] / [Rel-17]

Abstract: This contribution proposes conclusion texts for KI#4

# 1. Discussion

# 2. Proposal

It is proposed to capture the following texts as the conclusion for KI#4 in TR 23.754.

* * * * First Change * * * *

# 8	Conclusions

Editor's note:	This clause will capture conclusions from the study.

For KI#4 it is agreed that:

-  An UAV or an UAV-C is identified by the 3GPP System using a 3GPP UAV ID assigned by the MNO. The 3GPP UAV ID is included by the USS/UTM for tracking and to request location services from the MNO.

-  A new NF (e.g. UAVF) or a subsystem within an existing NF provides location and monitors UAV/UAV-C presense in a a given area to UTM/USS based on the request from UTM/USS.

- For location tracking USS/UTM provides geographic coordinates as it is not aware of 3GPP level area identifiers (e.g. TAs/Cell Ids). 3GPP system then converts the gerographic co-ordinates to 3GPP level area identifiers e.g. TA/CellIds etc

- Existing location service procedures (with necessary enhancements) are used to receive UAV/UAVc location information.

* * * * End of Change * * * *