SA WG2 Meeting #140E	S2-2005737

19 Aug – Sept 2 2020, Online

Source:	Nokia, Nokia Shanghai Bell

Title:	KI#3A, Sol #5: Update to Solution #5 for Capability Reporting

Document for:	Approval

Agenda Item:	8.5

Work Item / Release:	FS_IIoT

Abstract of the contribution: This contribution proposes updates to Solution #5 so device connectivity and 5GS delay information can be provided to an AF in the Capability Report

# 1		Discussion

Solution #5: Deterministic QoS for 5GS currently proposes an interaction between the AF and NEF where the AF can obtain capability information from the 5GS. Specifically, the Solution currently states:

The 5GS exposes the following Deterministic QoS Capability information to aid the AF in formulating a request for TSC QoS that will be acceptable to the 5GS:

-	5GS Support for TSC.

-	5GS Support for providing TSC assistance information.

-	Support for UE and UPF Hold and Forward Buffers.

-	Minimum 5GS Delay supported.

In a typical 5GS TSC scenario, devices requiring TSC QoS supply their stream requirements to the AF via an Ethernet PDU session and non-TSN bridges, as depicted in Figure 1 below. For example, the AF may receive device stream requirements via a UNI interface as defined in 802.1Q. Since devices sending requirements usually have no knowledge of the network topology, requests may come from devices that are conneted via the 5GS (through a UE) and/or from devices that are connected via the DN, and as a result, the AF will not be informed which devices are connected via the 5GS. Note this scenario is for a non-TSN DN - there is no CNC and no TSN Bridges, and the non-TSN bridges have sufficiently low delay, but optimization of the 5GS to support TSC is needed. Also note that the steps shown in Figure 1 are all currently supported without IEEE or 3GPP standards changes. As per TS 23.501 clause 5.8.2.5.3, the UPF applies MAC learning to Ethernet packets, and Source MAC addresses of identified devices behind the UE may be sent to the SMF.

Figure 1: AF receives Stream Requirements via the User Plane from a UE connected device

For the 5GS to provide UE and PDU Session specific QoS Capability Information such as “Minimum Delay Supported” and “Support for UE and UPF Hold and Forward Buffers”, the AF must be able to specify in the Capabiltiy Request sufficient information to identifiy the UE(s) and PDU Session(s) of interest.

We therefore propose the following updates to Solution #5:

- The AF may specify in the Nnef_TSC_Capability_Request one or more MAC addresses of devices, GPSIs or IP addresses.  For example the AF may specify a MAC address of devices for which it receives stream requirements via the user plane. Note that since the AF may be unaware of network topology, some of these devices may be connected via a UE, and some may be connected via the DN. In the example shown in Figure 1, the AF request could include MAC-A and other MAC addresses from devices in the DN with which the device communicates.

- For devices whose MAC address was been detected by the UPF as a Source Address in an uplink frame (and hence the device is connected via the associated PDU session), or where the IP address and GPSI is associated with a UE, the 5GS provides in the Nnef_TSC_Capability_Response:

  - Whether TSC can be supported with this UE

  - The “minimum 5GS delay supported”, taking into account UE Residence time and the delay associated with a TSC 5QI.

  - An indication whether the UE and UPF associated with the PDU session can support hold-and-forward buffers.

- For other devices (those whose MAC address has NOT been detected by the UPF as a Source Address in an uplink frame), or where the IP address or GPSI is not recognized, the 5GS indicates in the Nnef_TSC_Capability_Response that the device has not been detected as connected via a PDU Session.

The information provided in the Capability Report allows the AF to:

- Know whether it may request TSC, hold-and-forward buffering and supply TSCAI

- Divide an e2e delay budget between two UEs or between a UE and a DN endpoint, and request the resutant delay for PDU sessions.

- Specify in the request for TSC QoS, the relevant MAC address, GPSI or IP address. Note for Ethernet PDU sessions,  “UE MAC address” is a required input parameter for the  Nnef_AFsessionWithQoS_Create service operation – see TS23.502, clause 5.2.6.9.2.

We proposed to capture the following solution in TR 23.700-20.

*** Start Change ***

## 6.5	Solution #5: Deterministic QoS for Native 5GS

### 6.5.1	Introduction

For Key Issue #3A: Exposure of deterministic QoS, a solution is needed that allows "any AF that has knowledge of deterministic application requirements" to "be able to request TSC services from the 5GS and as authorized, be notified of pertinent network events." This solution provides a mechanism to do that whereby the AF may learn 5GS capabilities to support TSC, request QoS with specified requirements and supply information that can be used to derive TSCAI for 5GS QoS flows. The solution is applicable for all PDU Session Types (IP, Ethernet and unstructured), and is independent of DN TSN protocols (IEEE 802.1Qbv [8], IEEE 802.1Qci, IEEE 802.1Qcc [7]).

To enable the above capabilities, this solution proposes to enhance External Exposure of Network Capability (see TS 23.501 [2], clause 5.20), specifically for Provisioning and Monitoring capabilities. An update to the "Setting up an AF session with required QoS" procedure in TS 23.502 [3], clause 4.15.6.6 is proposed for setting QoS for TSC.

To cover a wide range of possible QoS parameters for TSC applications, the QoS Reference currently used in AF QoS requests is supplemented with requirements for specific parameters. When a requirement for a specific parameter is provided in the AF request, the PCF may, for the referenced 5QI, dynamically replace default values with values that meet the specific requirement provided by the AF.

### 6.5.2	Functional Description

The following capabilities are proposed using QoS request from AF:

-	The AF requests TSC QoS and provides traffic pattern as assistance parameters via External Parameter Provisioning, supplying the NEF. with requirements for one or more of a 5GS delay, Guaranteed Flow Bit Rate, Flow Direction, Burst Arrival Time at UE (uplink) or UPF (downlink), Burst Size, Burst Periodicity, Survival Time, e2e delay, and a Timing Domain. The AF also provides a Traffic Description, Target UE Identification, AF Identification, and a QoS Reference. AF may also request subscription to events defined in Table 6.1.3.18-1, TS 23.503 [12]. Example relevant events are Resource allocation outcome, QoS targets can no longer (or can again) be fulfilled, QoS Monitoring parameters. Service Data Flow deactivation (when the QoS flow is removed); and then the PCF may decide to subscribe to PCRTs in the SMF example PCRTs that may be used as defined in, Table 6.1.3.5-1, TS 23.503 [12] such as GFBR of the QoS Flow can no longer (or can again) be guaranteed. Additional events and PCRTs may be considered in the normative phase. Note GPSI may be applied to identify the individual UE in a manner similar to that used for AF influence on Traffic Routing (see TS 23.501 [2], clause 5.6.7). The NEF authorizes the AF request and forwards the request to the PCF. Then:

-	The PCF may, according to PCC rule authorization, choose a 5QI based on the QoS Reference and dynamically set the PDB and/or MDBV according to the received 5GS Delay and Burst Size requirements. As authorized, AF specified parameter values are used to over-ride default values, for the 5QI. If an AF request for a parameter value exceeds an authorization, the PCF may assign the highest authorized value. The GFBR for QoS Flow is set according to the Guaranteed Flow Bit Rate.

-	The 5GS may set the TSCAI Burst Arrival Time (BAT), Periodicity and Survival Time according to the requested Periodicity, Burst Arrival Time and Survival Time received from the AF.

-	If Timing Domain information was supplied by the AF and the 5GS uses the same Timing Domain, then no adjustment is needed for deriving TSCAI information. If the Timing Domain provided by AF is different from 5GS Timing Domain and the SMF has clock drift information for that Timing Domain (i.e. clock drift between 5G timing and AF supplied Timing Domain), then 5GS may adjust the TSCAI information so that it reflects the 5GS Clock. If Timing Domain information is not provided or the SMF does not have synchronization information for a requested Timing Domain, then the TSCAI information will be used without adjustment.

-	If the AF provides burst spread, the 5GS will provide burst spread as part of TSCAI to the NG-RAN.

When subscribed events are met the PCF reports them to the NEF, and NEF will notify the AF. The subscribe event may be reported once the SMF reports a PCRT to the PCF.

-	Following the procedure in clause 4.16.5.1, TS 23.502 [3]. PCF reports them to the NEF, and NEF notifies the AF.

-	The 5GS exposes the following Deterministic QoS Capability information to aid the AF in formulating a request for TSC QoS that will be acceptable to the 5GS:

-	5GS Support for TSC.

-	5GS Support for providing TSC assistance information.

-	Support for UE and UPF Hold and Forward Buffers.

-	Minimum 5GS Delay supported.

-	The 5GS provides the response to a request from an AF for TSC QoS.

Editor's note:	Whether and how to apply Hold and forward buffers for IP PDU Sessions and Ethernet PDU sessions if CNC is not present  is FFS.

The following capabilities are proposed for TSC connectivity monitoring:

-	The AF sends request to NEF to monitor the status of TSC connectivity via a set of events for a UE or group of UEs.

-	If the request is authorized, the NEF subscribes the required monitoring events to suitable NFs (e.g. UDM, AMF, SMF, PCF).

-	When the NF detects the subscribed events occurs, the NF sends the event notification to NEF. The NEF forwards to the AF the received reporting event.

Editor's note:	The list of monitoring events applicable for the suitable NF needs to be defined, as it is FFS.

To support notification, the additional events may be added to the list of monitoring capabilities specified for the NEF in TS 23.502 [3] (see clause 4.15.3.1). Existing events can be subscribed to, such as: PDU Session Status, Communication failure, and Loss of Connectivity.

### 6.5.3	Procedures

An overall procedure for Native 5GS deterministic QoS is illustrated in Figure 6.6.3-1:

Figure 6.6.3-1: Procedure for Exposure the deterministic QoS

1.	PDU Session Establishment for UE1 as specified in clause 4.3.2.2.1 of TS 23.502 [3]. During this step NFs may notify the NEF of Deterministic QoS Capability information to be exposed in Step 2.

2.	The NEF exposes Deterministic QoS Capability to AF, as shown in Figure 6.5.3-2

3.	Setting up an AF session with required QoS procedure specified in the TS 23.502 [3] clause 4.15.6.6. is executed with the following modifications:

-	The parameters described in clause 6.5.2 "Function Description" may be specified in the AF request to the NEF

-	The NEF may forward to the PCF the additional parameters received at the NEF, and the PCF derives the 5QI and sets the PCC Rule accordingly. The PCF includes the Time Domain to the PCC Rule.

-	The response from PCF to NEF, and NEF to AF contains additional information on the TSC resources allocated for the request.

4.	The PCF creates the PCC rule and provides to SMF. And PCF also includes the TSC traffic pattern information from NEF/AF.

5.	The SMF calculates the TSCAI similar to TS 23.501 [2] clause 5.27.2, with the following modifications:

The SMF assigns the TSCAI with the Time Domain as received from the PCF to a QoS Flow. If the SMF receives clock drift information from UPF for that Timing Domain (i.e. clock drift between 5G timing and AF supplied Timing Domain), then SMF may adjust the TSCAI information of the QoS Flow so that it reflects the 5GS Clock.

6.	The SMF configures the U-plane and sends the N2 message with TSCAI to NG-RAN. The SMF distributes the parameters for Hold and Forward Buffering to the UE and UPF.

Editor's note:	How the parameters for H&F buffering are created and sent to the UE and UPF is FFS.

AF requested TSC QoS:

The following procedure is used to expose 5GS information to aid the AF in formulating a request for TSC QoS (step 3 of Figure 6.6.3-1).

Figure 6.5.3-1: 5GS Capability exposure towards AF

The NEF obtains 5GS capability information using services provided by other NFs.

Method 2 for 5GS Capability exposure towards AF:

Since the “Minimum 5GS Delay supported” and “Maximum 5GS Delay supported” are dependent on UE capabilities and residence time, the following is supported to report this information:

- The AF may specify in the Nnef_TSC_Capability_Request for deterministic QoS, one or more MAC addresses of devices, GPSIs or IP addresses. Note some of these MAC addresses may belong to devices connected via a UE, and some to devices connected via the DN as the AF may be unaware of network topology.

- For devices where UPF MAC learning has detected the AF requested MAC address as a Source Address in an uplink frame (and hence the device is connected via the associated PDU session), or where the IP address and GPSI is associated with a UE, the 5GS provides in the Nnef_TSC_Capability_Response:

  - Whether TSC can be supported with this UE.

  - The “minimum 5GS delay supported” and “maximum 5GS delay supported”, taking into account UE Residence time and the delay associated with a TSC 5QI.

  - An indication whether the UE and UPF associated with the PDU session identified by the MAC address can support hold-and-forward buffers.

- For other devices (those whose MAC address has not been detected by the UPF as a Source Address in an uplink frame), or where the IP address or GPSI is not recognized, the 5GS indicates in the Nnef_TSC_Capability_Response that the device is not connected via a PDU Session.

The information provided in the Capability Response can aid the AF in formulating a request for TSC QoS by allowing the AF to:

- Know whether it can request TSC and supply input parameters for TSC assistance container.

- Requesting delay based on 5GS reported minimum and maximum delay (between two UE(s) or UE and DN) for PDU Sessions.

- Specify in the request for TSC QoS, the relevant MAC address, GPSI or IP address. Note for Ethernet PDU sessions, “UE MAC address” is a required input parameter for the Nnef_AFsessionWithQoS_Create service operation – see TS23.502, clause 5.2.6.9.2.

TSC connectivity monitoring:

Figure 6.5.3-3: Procedure for TSC connectivity monitoring

1.	The AF sends Nnef_EventExposure_Subscribe Request to NEF to monitor the status of TSC connectivity via a set of events for a UE or group of UEs. The events are those listed in clause 5.2.6.2.2 of TS 23.502 [3].

Event Reporting Information defines the type of reporting requested (e.g. one-time reporting, periodic reporting or event based reporting, for Monitoring Events).

2.	If the reporting event subscription is authorized by the NEF, the NEF sends the Subscribe Requests to the related NFs that detect the events, as described in TS 23.502 [3] procedures for subscription to these events.

3.	The NEF sends Nnef_EventExposure_Subscribe Response to AF as described in TS 23.502 [3] procedures for subscription to these events.

4.	The NFs detects the monitored event occurs and sends the event report to the NEF as described in TS 23.502 [3] procedures for subscription to these events.

5.	The NEF sends to the AF the notification of TSC communication monitoring event changed as described in TS 23.502 [3] procedures for subscription to these events.

Editor's note:	Whether there is a need to define new monitoring events for TSC monitoring and which ones are those is FFS.

### 6.5.4	Impacts on services, entities and interfaces

Impacts on existing entities and interfaces

The solution has the following impacts:

1.	An additional Nnef service is used to expose 5GS TSC capabilities to the AF

2.	Additional parameters are included in existing messages in the "AF session with required QoS procedure" specified in the TS 23.502 [3] clause 4.15.6.6.

3.	The PCF uses the additional AF supplied parameters to calculate a PCC rule

4.	"Burst Spread" is sent from the PCF to the SMF, which uses it to determine a Burst Spread TSCAI parameter.

5.	Time Domain is provided from PCF to the SMF, which is used by the SMF to associate the QoS Flow to a Time Domain.The solution assumes that the 5GS can be made aware of the timing in the AF supplied Timing Domain, this can be provided via Time Synchronization service as described in Solution #7.

Editor's note:	UE and UPF impacts are FFS.

AF:

-	Deliver TSC connectivity monitoring request message to NEF.

-	Deliver TSC QoS request with TSC QoS reference, TSC traffic pattern, and subscription to events to the PCF.

*** End Change ***