SA WG2 Meeting #140E	S2-2005741

19 August - 02 September 2020

Source:	Nokia, Nokia Shanghai Bell, NTT DOCOMO, Qualcomm Incorporated

Title:	KI#3B: Sol#7: Updates on Solution #7 for Exposure of Time Synchronization

Document for:	Discussion/Approval

Agenda Item:	8.5

Work Item / Release:	FS_IIoT / Rel-17

Abstract of the contribution: This contribution updates solution #7 for Exposure of Time Synchronization

# 1		Discussion

This contribution updates Key Issue #3B solution #7 for Exposure of Time Synchronization.

Reference from TS 22.104 for time sync accuracy:

### 5.6.2	Clock synchronisation service performance requirements

Table 5.6.2-1: Clock synchronization service performance requirements

| User-specific clock synchronicity accuracy level | Number of devices in one Communication group for clock synchronisation | Clock synchronicity requirement | Service area | Scenario |
| --- | --- | --- | --- | --- |
| 1 | Up to 300 UEs | < 1 µs | ≤ 100 m x 100 m | - Motion control - Control-to-control communication for industrial controller |
| 2 | Up to 10 UEs | < 10 µs | ≤ 2500 m2 | - High data rate video streaming |
| 3 | Up to 100 UEs | < 1 µs | < 20 km2 | - Smart Grid: synchronicity between PMUs |

Reference from IEEE 802.1AS for sending rate:

- This standard specifies a way to request that a neighbor port reduce the rate of sending Sync (and Follow_Up if optional one-step processing is not enabled), peer delay, and Announce messages, and also to inform the neighbor not to compute neighbor rate ratio and/or propagation delay on this link.

# 2		Proposal

The following change is proposed for TR 23.700-20.

*** Start Change ***

## 6.7	Solution #7: Exposure of Time Synchronization

### 6.7.1	Introduction

For Key Issue #3B: Exposure of time synchronization, a solution is needed that allows "any AF that has knowledge of deterministic application requirements" to "be able to request TSC services from the 5GS and as authorized, be notified of pertinent network events." This solution provides a mechanism to do that whereby the AF may learn 5GS capabilities to support time synchronization, request time synchronization with specified requirements, and supply information that can be used to optimize and configure time synchronization procedure for connected devices.

The solution is applicable for IP and Ethernet types of PDU Sessions. The solution is applicable for non-TSN deployments, i.e. when the 5GS is not part of an IEEE TSN network.

The solution is applicable for the following synchronization methods:

1)	assuming use of a gPTP client (IEEE 802.1AS [6] Time Aware System) or PTP client (IEEE 1588-2008 [13]), e.g. including leveraging the DS-TT and NW-TT capabilities defined in Rel-16 with IEE802.1AS protocol support.

2)	assuming use of a gPTP client (IEEE 802.1AS [6] Time Aware System) or PTP client (IEEE 1588-2008 [13]) in UPF/NW-TT, where the time source is provided by the 5GS, and UPF/NW-TT is configured to create the (g)PtP message for conveying the timing information.

3)	assuming use of the 5GS time source by 5GS; where the 5G-AN provides a 5GS reference time to the UE via 3GPP radio layer and UE may provide it to the applications or devices behind the UE by implementation specific means out of scope of 3GPP.

4)	assuming use of the 5GS time source by 5GS; where the 5G-AN provides a 5GS reference time to the UE via 3GPP radio layer and DS-TT provides (g)PTP messages for conveying the timing information to devices behind the UE.

This solution can be used together with e.g. Solution #1 for the time source to locate in a TSN node behind the DS-TT/UE.

To enable the above capabilities, this solution proposes to enhance External Exposure of Network Capability (see TS 23.501 [2], clause 5.20), specifically for Provisioning and Monitoring capabilities.

To cover a wide range of possible time synchronization requirements for TSC applications, the AF requests is supplemented with requirements for synchronization. When a requirement for a specific synchronization parameter is provided in the AF request, the PCF may dynamically replace default values with values that meet the specific requirement provided by the AF.

### 6.7.2	Functional Description

The following capabilities are proposed:

-	The AF requests Time Synchronization service via External Parameter Provisioning, supplying the NEF with requirements for one or more time synchronization method(s), supported (g)PTP versions, Timing Domain, grandmaster priorities, required synchronization accuracy (both in terms of required gPTP rate for each DN port, synchronization accuracy in microseconds per device link, and number of connected gPTP slaves in case 5GS provides the Timing Domain). The AF also provides Target UE(s) Identification or Spatial Validity Condition (i.e. to target a geographical area), AF Identification. Note GPSI may be applied to identify the individual UE in a manner like that used for AF influence on Traffic Routing (see TS 23.501 [2], clause 5.6.7). Then:

-	Time synchronization method 1 (the time source is located in the DN or device side):

Timing Domain is provided from the DN or Device Side and gPTP or PTP method is applied, The PCF may, according to PCC rule authorization, choose a 5QI based on the QoS Reference and dynamically set the PDB and/or MDBV according to requirements for gPTP protocol (e.g. required number of synchronization clients and gPtP messaging rate). The 5GS acts as a PTP Boundary Clock. For Ethernet type PDU sessions, both PTP over UDP/IP and gPTP over Ethernet are applicable. For IP type PDU sessions, PTP over UDP/IP is applicable. In this case, neither the NW-TT port or DS-TT port is a (g)PTP grandmaster.

NOTE 1:	It is assumed that 5G internal system clock is made available to all user plane nodes in the 5G system (e.g. to UEs via SIB/RRC messages).

-	Time synchronization method 2 (the time source is provided by the UPF/NW-TT in 5GS via gPTP or PTP):

Timing Domain is to be provided from the 5GS (or the AF and the 5GS shares the same Timing Domain) The PCF may, according to PCC rule authorization, choose a 5QI based on the QoS Reference and dynamically set the PDB and/or MDBV according to requirements for gPTP protocol. Further, the PCF may, according to PCC rule authorization, choose a gPTP message generation rate for the UPF/NW-TT adaptively. If the AF does not provide a required gPTP message rate, the 5GS may determine a gPTP message rate based on 5GS constraints. The 5GS acts as a (g)PTP grandmaster. For Ethernet type PDU sessions, both PTP over UDP/IP and gPTP over Ethernet are applicable. For IP type PDU sessions, PTP over UDP/IP is applicable. The PTP grandmaster resides in NW-TT.

-	Time synchronization method 3 (the time source is provided by the 5GS and 5G-AN provides a 5GS reference time to the UE via 3GPP radio layer; UE provides time information to attached devices using implementation specific means):

Timing Domain is not applicable. A PCC rule for time synchronization is not required. The method is applicable for both IP type and Ethernet type PDU sessions.

-	Time synchronization method 4 (the time source is provided by the DS-TT in 5GS via gPTP or PTP):

Timing Domain is to be provided from the 5GS (or the AF and the 5GS shares the same Timing Domain). The 5GS acts as a (g)PTP grandmaster. For Ethernet type PDU sessions, both PTP over UDP/IP and gPTP over Ethernet are applicable. For IP type PDU sessions, PTP over UDP/IP is applicable. The PTP grandmaster resides in DS-TT. Solution #X: "PTP GM support by DS-TT" describes the details for the PTP grandmaster in DS-TT. In case the DS-TT cannot act as a GM, then the GM is activated in NW-TT. NEF can be the coordinator to activate/deactivate the PTP functionalities in DS-TT and NW-TT.

Editor’s note:	whether there is a need to send requested synchronization accuracy to UE/DS-TT (e.g. when it supports proprietary sync protocol) is FFS.

-	If the AF does not provide a required synchronization accuracy over the air interface, the 5GS may determine a synchronization accuracy based on 5GS constraints.

-	For all time synchronization methods, the PMIC can be used to manage the (g)PTP operation for the DS-TT influenced by the AF requested synchronization configuration. Following are some parameters that need to be included as part of PMIC for Time Sync:

- Requested synchronization accuracy.

- If DS-TT has indicated that it is capable of acting as (g)PTP GM, then the network may request the DS-TT to generate (g)PTP messages to devices attached to the UE by providing (g)PTP parameters (incl. (g)PTP version, domain number(s), sending rate, etc.) and an instruction to start sending (g)PTP messages This is also described in solution 9.

- DS-TT capabilities for acting as (g)PTP GM, which version(s) the DS-TT supports, (g)PTP default Data Set (e.g. clock identity, clock class, clock accuracy, etc),

- (g)PTP port Data Set (e.g. port number, port role, port enabled, announce interval, etc).

-	If an AF request is received for UE/DS-TT for which PDU Session does not exists, then the NEF may store the information in the UDR until the target UE establishes the PDU Session. Also, the NEF may store DS-TT/NW-TT capabilities for UE(s) with active PDU Sessions.

NOTE 2:	this is intended for a wide range of applications, ex. motion control use-cases, VIAPA, smart grid applications where requirements for synchronization accuracy and deployment scenario may be very diverse.

-	The 5GS exposes the following to aid the AF in formulating a request for Time Synchronization that will be acceptable to the 5GS:

-	5GS Support for Time Synchronization to 5GS.

-	supported time synchronization methods (i.e. method(s) 1, 2, 3, 4).

-	For method 2, additional information is exposed, including grandmaster clock quality and clock identity that are provided by UPF/NW-TT.

-	supported (g)PTP versions:

-	IEEE 802.1AS [6] (i.e. gPTP), and/or

-	PTP over UDP/IPv4 according to IEEE 1588-2008 [13] Annex D, and/or

-	PTP over UDP over IPv6 according to IEEE 1588-2008 [13] Annex E.

-	Minimum Time Synchronization accuracy supported.

-	Minimum gPTP or PTP message generation supported, and maximum number of clients that can be supported at the minimum gPTP or PTP message rate.

-	Considering the AF requested time synchronization service, NEF determines the time distribution method to use to provide the service, the allowed time synchronization requirements (e.g. clock synchronization requirement, time domain, message rate), etc. Depending on the time synchronization service to configure, time synchronization policies may be associated to a PDU Session (for example for methods 1 and 2), or the policies may be independent of the user plane configuration the UE has (for example for methods 3 and 4).

To support notification, the additional events may be added to the list of monitoring capabilities specified for the NEF in TS 23.502 [3] (see clause 4.15.3.1).

### 6.7.3	Procedures

The following procedure is used to expose 5GS information to aid the AF in formulating a request for Time Synchronization in 5GS.

Figure 6.7.3-1: Time Synchronization capability exposure towards AF

The following procedure is used by the AF to request activation/deactivation/modification of the Time Synchronization service in 5GS.

Figure 6.7.3-2: Activate/Deactivate/Modify UE's Time Synchronization Service

0.	It is assumed that a PDU session is established between UE/DS-TT and UPF/NW-TT. It could also be established using device trigger procedure as defined in TS 23.501[x], TS 23.502[y].

1.	An AF sends a Time Sync Service Request indicating e.g. the target UE, group of UEs, or DNN and S-NSSAI, and the clock domain to the PCF via NEF to activate/deactivate/modify UE's time synchronization service.

If the request is target for multiple UEs or for future PDU session(s), the NEF stores the time synchronization service policy to UDR. The time synchronization service policy contains the NEF address and a subscription for the PCF to send notifications to this NEF address. The PCF(s) receive(s) a Nudr_DM_Notify notification of data change from the UDR (as described in 23.501 clause 6.3.7.2 for existing or future PDU Sessions).

2.	The NEF ensures the necessary authorization control and sends PMIC/BMIC signaling to configure the (g)PTP functionality in DS-TT and NW-TT, respectively. PCF derives QoS policies and forwards the PMIC/BMIC information to SMF.

3.	The PCF notifies the SMF via Npcf_SMPolicyControl_UpdateNotify message;

4.	The SMF triggers the SM Policy Association establishment/modification procedure to create a QoS flow (or release the QoS flow) for transmitting the clock domain's (g)PTP message.

5a.	The SMF forwards BMIC to the UPF/NW-TT through the N4 session level modification message, if needed.

5b.	The SMF forwards PMIC to DS-TT

6a.	Upon reception of the notification in step 5, UPF/NW-TT performs consequent behaviours to support time synchronization of corresponding GM, for example if the time source for the Time Sync service requested to be activated is in the UPF/NW-TT, then the UPF will start to create and distribute the (g)PTP messages of the 5G GM to the UE.

6b.	Upon reception of the notification in step 5, UE/DS-TT starts or stops to handle the (g)PTP messages as indicated in the notification.

7.	If PCF receive Time Sync service request from NEF, the PCF responses the NEF and the NEF responses the AF with the Time Sync service request result.

If the request in Step 1 is targeted for multiple UEs or for future PDU Sessions and a PDU session is established, the PCF receives time synchronization service policy from the UDR and notifies the NEF for the event when a new PDU session is established. The NEF may then obtain the capabilities from the DS-TT via PMIC, and send PMIC signaling to the DS-TT (via PCF) to configure the (g)PTP functionality in the DS-TT.

### 6.7.4	Impacts on services, entities and interfaces

AF:

-	Provides the parameters to the NEF to configure the Time Synchronization service.

-	Provides the priorities for grandmaster in 5GS (DS-TT or NW-TT) for use during BMCA procedure.

NOTE:	The priorities can be used to ensure that the time source provided by 5GS (DS-TT or NW-TT) is selected as a grandmaster towards the devices outside 5GS.

NEF:

-	Exposes the 5GS capabilities to support Time Synchronization service. Authorizes the Time Synchronization service request from the AF. Controls the (g)PTP functionality in DS-TT and NW-TT via SMF.

UE:

-	Distributes the 5G reference time to DS-TT and optionally to applications or devices behind the UE (by mechanism out of scope of 3GPP), for IP and Ethernet types of PDU Sessions.

DS-TT:

-	Indicates a support for PTP/UDP/IP residence time calculation in Port Management Information container to the network.

-	When activated by NEF, calculates and adds the measured residence time between the NW-TT and DS-TT into the Correction Field (CF) of the PTP time synchronization messages conveyed over UDP/IP, applies to IP and Ethernet types of PDU Sessions.

-	When activated by NEF, (if the time source is located behind the DS-TT as in Solution #1): Inserts a timestamp to the PTP time synchronization messages conveyed over UDP/IP, applies to IP and Ethernet types of PDU Sessions.

-	When activated by NEF, (if the time source is located in the DS-TT): generates PTP time sync messages (with the indicated PTP version, domain number(s), sending rate, etc), applies to IP and Ethernet types of PDU Sessions. Solution #X: "PTP GM support by DS-TT" describes the details for the PTP grandmaster in DS-TT.

UPF/NW-TT:

-	When activated by NEF, (if the time source is located in the DN): Inserts a timestamp to the PTP time synchronization messages conveyed over UDP/IP, applies to IP and Ethernet types of PDU Sessions.

-	When activated by NEF, (if the time source is located in the NW-TT): generates PTP time sync messages (with the indicated PTP version, domain number(s), sending rate, etc), applies to IP and Ethernet types of PDU Sessions.

-	Forwards the PTP time synchronization messages conveyed over UDP/IP in DL direction to the PDU Sessions, applies to IP and Ethernet types of PDU Sessions.

*** End Change ***