SA WG2 Meeting #140	S2-2005743r02

19 August - 02 September 2020

Source:	Nokia, Nokia Shanghai Bell, NTT DOCOMO

Title:	KI #3B: Evaluation and conclusion

Document for:	Discussion/Approval

Agenda Item:	8.5

Work Item / Release:	FS_IIoT / Rel-17

Abstract of the contribution: This contribution gives the evaluations for the solutions of KI#3B and proposes the conclusion for KI#3B.

1		Discussion

This contribution proposes conclusions and way forward for Key Issue #3B for Exposure of Time Synchronization.

2		Proposal

The following change is proposed for TR 23.700-20.

# * * * * First change * * * *

7.X	Key Issue #3B: Exposure of Time Synchronization

Editor's note:	This clause will provide a general evaluation and comparison of the solutions per Key Issue #<X>.

Solution #7 supports the AF to be able to request Time Synchronization service via External Parameter Provisioning. It can interwork with IEEE TSN or TSC use cases, and it can target a UE or a group of UEs.

Solution #8 describes how the AF is able to trigger a PDU Session establishment proceude.

Solution #9 supports the DS-TT acting as a (g)PTP grand master (GM) for devices attached to the DS-TT.

# * * * * Second change * * * *

8.X	Key Issue #3B: Exposure of Time Synchronization

Editor's note:	This clause will capture conclusions for Key Issue #<X>.

Following are interim principles to be considered as the basis for normative work:

- Time Synchronization service can be provided for Ethernet PDU sessions and IP PDU sessions.

- The AF requests for Time synchronization service are authorized by the NEF as described in TS 23.501.

- If the AF needs to trigger a PDU Session establishment procedure from one or more UEs, the existing device trigger procedures can be used as described in TS 23.502.