SA WG2 Meeting #140e	S2-2005768

19 August – 02 September 2020, e-meeting	(revision of S2-200xxxx)

Source:	InterDigital

Title:	KI#3, Sol#5: Update to remove ENs

Document for:	Approval

Agenda Item:	8.6

Work Item / Release:	[FS_eATSSS] / [Rel-17]

Abstract: This contribution proposes the update to Solution #5 in TR 23.700-93.

# Discussion

In this contribution, the following Editor’s Notes are addressed:

Editor's note:	it is FFS to describe the benefit of creating a new solution with regard to adapting the solution defined in 23.316 [12] clause 4.12.3 to other devices than 5G RG.

This solution addresses a scenario which we think is not covered by 23.316, that is, a UE with existing MA-PDU Session in 5GS moves to EPS and replace the 3GPP access leg with PDN in EPS. And this Editor’s Note is actually an evaluation concern. We propose to remove this EN.

2.

Editor's note:	Figure 4.11.2.2-1 in TS 23.502 [4] refers to Interworking procedures without N26 interface. It is FFS whether SR mode with N26 is supported by the solution.

In our analysis, UE using N26 can reuse the existing interworking procedure, with the additional step at the network (step 6) and UE side (Step 7) to associate the EPC user plane resources with the existing non-3GPP access leg in the 5GS as a MA-PDU. This is clarified in Step 4 and 5.

Impact analysis is also provided in this update.

# 2. Proposal

It is proposed to capture the following update to Solution #5 in TR 23.700-93.

* * * * First Change * * * *

## 6.5	Solution #5: Replacing 3GPP access leg of MA-PDU Session with PDN connection in EPC

### 6.5.1	Introduction

This solution enables a capable UE and network to replace the 3GPP access leg of a normal MA PDU Session with a PDU connection in EPC, while keeping the non-3GPP access leg in 5GC.

.

This solution is meant as a complement to solution “Supporting a PDN connection in EPC as a 3GPP access leg of MA-PDU Session” so taking the R16 solution (for 5G RG) as the baseline to support any UE BUT with a focus on the support of mobility of a MA PDU Session with a 3GPP leg over 5GC (PDU Session) towards a MA PDU session with a 3GPP leg over EPC (PDN connection);

### 6.5.2	High-level Description

In this solution, it is assumed that the UE is able to attach to the EPC over E-UTRAN and simultaneously register with the 5GC over non-3GPP access. Initially, the UE is registered with 5GC via both 3GPP access and non-3GPP access and has established a MA-PDU Session with UP resources for both 3GPP access and non-3GPP access in 5GC.

Editor's note:	The UE may also establish from scratch a MA PDU with 3GPP access leg in EPC, i.e. without pre-existence of a normal MA-PDU. This case is not covered in this solution and may be addressed by other solutions.

For brevity, the scenario of 5GS to EPS mobility in IDLE mode without N26 interface is used to describe the solution.

The UE may indicate its support for 3GPP access leg in EPC for a MA-PDU Session as part of ATSSS capability during the MA-PDU Session establishment. If the network also supports it, and based on the network local policies, the network may indicate that the MA-PDU Session is allowed to have its 3GPP access leg replaced by a PDN connection in the EPC.

During the 5GS to EPS mobility procedure, for the MA-PDU Session which is allowed to have 3GPP access leg in EPC, the UE may provide an indication of "MA PDU Request", in addition to MA-PDU Session ID in PCO, in the UE initiated PDN Connectivity request (Step 13 of Figure 4.11.2.2.-1 in TS 23.502 [4]). The "MA PDU Request" indicates that the UE requests to keep the PDN Connection as the 3GPP access leg of the original MA PDU Session. If this is accepted by the network, the PGW-C/SMF should not release the UP resources of the non-3GPP access leg in 5GC.

After the PDN Connection is successfully established in the EPC, the network may initiate the session modification procedure with the PGW-U/UPF to associate the PDN Connection with the MA-PDU Session and modify the forwarding rules. The network may also initiate PDU Session Modification procedure in the 5GC over non-3GPP access. The network indicates to the UE that the 3GPP access leg of the MA-PDU Session has been replaced by the PDN connection. The UE then locally associates the PDN connection as the 3GPP access leg of the MA-PDU Session. The network may also update the ATSSS rules over the 5GC non-3GPP access.

At this point, the original MA PDU Session is converted to a new MA PDU Session with the 3GPP access leg in EPC. In the uplink, for the data associated with the PDN connection, if the PDN connection is part of a MA PDU Session, the UE further checks the ATSSS rules and according to the rules, the UE may steer the data toward the PDN connection in EPC, or non-3GPP connection in 5GC.

When the UE moves back from the EPS to 5GS, the UE may re-establish the MA-PDU Session over 3GPP access of 5GC.

### 6.5.3	Procedures

Figure 6.5.3-1: Replacing 3GPP access leg of MA PDU Session with PDN connection in EPC

1.	The UE is registered in the 5GS over both 3GPP access and non-3GPP access.

2.	The UE established a normal MA PDU Session in 5GS, with UP resources in 5G for both 3GPP access and non-3GPP access.

3.	The UE moves from 5GS to EPS due to mobility.

4.	In case of 5GS-EPS mobility without N26 interface, Steps 1-12 of Figure 4.11.2.2-1 in TS 23.502 [4] are performed.

In case of 5GS-EPS mobility with N26 interface, Steps 1-19 of Figure 4.11.1.3.2-1 in TS 23.502 [4] are performed, and Step 5 below is skipped

5.	In case of 5GS-EPS mobility without N26 interface, in Step 13 of Figure 4.11.2.2-1 in TS 23.502 [4], the UE provides additional indication of "MA PDU request" (e.g. in PCO), and the PDU Session ID of the MA PDU Session. This indication informs the network that the UE hopes to keep the PDN Connection as the 3GPP access leg of the MA PDU Session.

If the MA-PDU session is the only PDU Session that needs to be handed over to the EPS, Step 5 may be realized in the PDN Connectivity Request combined in EPS Attach procedure of step 4. And in this case step 5 is not needed.

6.	If the network accepts the PDN Connection as the 3GPP access leg of the MA PDU Session, the PGW-C+SMF initiate the N4 session modification procedure to associate the PDN Connection with the MA-PDU Session and modify the forwarding rules accordingly.

7.	. The UE locally associates the PDN connection with the MA PDU Session. The network may also update the ATSSS rules to make them more adapted to the new MA PDU Session with one leg in EPC.

### 6.5.4	Impacts on services, entities, interfaces and IETF Protocols

The solution has the following impacts on entities and interfaces:

- PGW-C+SMF/PGW-U+UPF:

  - Supports the MA-PDU Session Request indication and PDU Session ID in the PCO PDN Connection request as in R16

  - Supports the session modification procedure to associate the EPS UP resource with the existing MA-PDU Session as in the R16

- PGW-U+UPF

  - Supports the session modification procedure to associate the EPS UP resource with the existing MA-PDU Session

- UE:

  - Supports the association of the EPS UP connection with the existing MA-PDU Session.

* * * * End of Change * * * *