SA WG2 Meeting #141E (e-meeting)	S2-2007423

Oct 12 – Oct 23, 2020, Electronic meeting

Source:	Samsung, InterDigital, Apple, NEC

Title:	KI#1, KI#3, MUSIM Assistance Information

Document for:	Approval

Agenda Item:	8.10

Work Item / Release:	FS_MUSIM/ Rel-17

# Introduction

This paper proposes the parameters UE will indicate to the network to filter the Paging messages. So that network resources can be optimized.

# Discussion

The MUSIM AI is provided by UE when it is leaving the network over 3GPPA following principles of sol#10 or sol#5. i.e. whenever UE is not reachable over 3GPPA for some or all the services to save network resources.

# Proposal

Its proposed to agree below changes to the TR 23.761

## Annex #X: MUSIM Assistance Information

Table X.1: MUSIM AI

Type of downlink traffic

PDU session ID(s).

Services

N3GPPA paging

Timer

The MUSIM AI is provided by UE when it is leaving the network over 3GPPA either in IDLE mode (following principles of sol#10) or in connected mode (e.g. sol#5). i.e. whenever UE is not reachable over 3GPPA for some or all the services to save network resources. One or more of the following values are included:

1)	PDU session ID(s): The list of PDU session ID(s) for which UE will respond to paging message. If not included UE will not respond for paging of any downlink data packet.

2)	Services: List of services (voice, SMS, CP-Signalling, default) UE will respond to paging. If not included UE will not respond for following services voice, SMS, CP-Signalling, default.

3)	N3GPPA paging: If registered on same PLMN for both 3GPPA and N3GPPA, the UE guides network to page (i.e. send NOTIFICATION message) the UE only on N3GPPA taking into account other MUSIM AI parameters.

4)	Timer value: Indicates the duration UE will not be able to receive the paging message for some or all the services.

NOTE:	Whether explicit “N3GPPA paging” flag is required as part of MUSIM AI or AMF can implicitly understand as part of leaving procedure will be determined.