SA WG2 Meeting #142E	S2-2008780

Nov. 16 – Nov. 20, 2020, Elbonia

Source:	OPPO

Title:	KI#1, update to solution#28

Document for:	Approval/Discussion

Agenda Item:	8.10

Work Item / Release:	FS_MUSIM / Rel-17

Abstract of the contribution: This contribution is to update the soln#28 to remove the EN

1. Introduction

In SA2#141E, an solution “important service” paging cause was introduced, which has similar impact as in Solution #1 except with only one Paging Cause. However, an EN below was added to discuss.

Editor's note:	It is FFS that how does network determine which services need to be considered as "important service" (e.g. by UE negotiate with the network).

To resolve the EN, we see two methods

1. Subscription is used to indicate which service is important

User’s requirement can be fulfilled using this method. Subscription can be changed based on service agreement between user and operator.

2. UE sends NAS message to AMF to indicate the important service

This method is used when user can set which service is important in handset and UE sends the information to AMF via NAS.

From our view, using subscription is good and propose alt.1 is chosen as an method to resolve the EN.

3. Proposal

* * * * Start of Change * * * *

6.28	Solution #28: "Important Service" paging cause

6.28.1	Introduction

The solution applies to Key Issue #1 "Handling of MT service".

The solution reuses the mechanism of Solution #1 regarding how AMF determines a paging cause for "Important Service" in the paging message if AMF decides that the paging is for an important service.

Regarding the how does the network determine which service is considered as “important service”:

-  By default, the IMS voice is important service;

-   Optionally, regarding user’s request, the subscription can be used to indicate to AMF which service is important per UE granularity.

6.28.2	Functional Description

A "important service" indication (one paging cause) is needed to indicate the important service i.e. voice service or others as UE requested (or based on operator policy. Optionally, AMF may determine services other than voice service as important services when UDM (subscription) piggyback the important service(s). However "Voice service" is always considered as the important service by default.

Table 6.28.2-1: Important Service indication

|  | Description |
| --- | --- |
| Important Service indication | The downlink traffic is corresponding to the important service (by default it is voice service). UE determines to respond to the paging based on the indication. |

AMF determines it is an important service based on HPLMN/DNN/5QI configuration and the ARP/PPI received from the SMF, or based on operator policy. Similar mechanism applies to NG-RAN for UE in RRC-Inactive and to MME for UE in EPS. How the AMF (or MME) determines the cause is the same as the mechanism in Solution #1.

NOTE:	In future release the UE can negotiate additional services with the network that need to be considered as "important service".

6.28.3	Impacts on services, entities and interfaces

UE, AMF, MME, RAN:

-	Similar impact as in Solution #1 for the case with only one Paging Cause.

-    By default, AMF considers the IMS voice as important service, and optionally, Subscription for AMF is used to indicate which service is important service

* * * * End of Change * * * *