SA WG2 Meeting #142e	S2-2008969

16-20 November 2020, e-Meeting	(revision of S2-200xxxx)

Source:	Xiaomi

Title:	Discussion on new standardized 5QI for AIS

Document for:	Discussion / Approval

Agenda Item:	8.13

Work Item / Release:	5G_AIS / Rel-17

Abstract of the contribution: This paper is to discuss current status of KPI definition for AIS scenarios in SA1 and SA4 and the proposed way forward in SA2.

- Introduction

Based on the objective of 5G_AIS as defined in SP-190564, new standardized 5QI(s) corresponding to QoS requirements from SA1 NCIS work item in TS 22.261 may be defined, which includes:

- Required latency for uplink transmission from UE to UPF plus downlink transmission from UPF to UE.

- Required reliability for uplink sensor/pose data and downlink pre-rendered/rendered audio/visual data

- Required high data rate in downlink direction related to SA1 agreed KPIs including FPS (frame-per-second) and resolution etc.

It is also stated in the WID that the KPIs defined in SA1 will be evaluated in SA4 and additional requirements for QoS enhancement may come from SA4’s ongoing work on 5G XR.

Status in SA1

Table 7.6.1-1 defined in TS 22.261 lists KPIs for additional high data rate and low latency service.

| Use Cases | Characteristic parameter (KPI) |  |  | Influence quantity |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Max Allowed End-to-end latency | Service bit rate: user-experienced data rate | Reliability | # of UEs | UE Speed | Service Area (note 2) |
| Cloud/Edge/Split Rendering (note 1) | 5ms (i.e. UL+DL between UE and the interface to data network) (note 4) | 0.1-[1] Gbit/s supporting visual content (e.g. VR based or high definition video) with 4K, 8K resolution and up to120fps content. | 99.99% in uplink and 99.9% in downlink (note 4) | - | Stationary or Pedestrian | Countrywide |
| Gaming or Interactive Data Exchanging (note 3) | 10ms (note 4) | 0.1-[1] Gbit/s supporting visual content (e.g. VR based or high definition video) with 4K, 8K resolution and up to120fps content. | 99.99% (note 4) | ≤ [10] | Stationary or Pedestrian | 20 m x 10 m; in one vehicle (up to 120 km/h) and in one train (up to 500 km/h) |
| Consume VR content via tethered VR headset (note 6) | [5 -10] ms (note 5) | 0.1-[10] Gbit/s (note 5) | [99,99%] | - | Stationary or Pedestrian | - |
| NOTE 1: Unless otherwise specified, all communication via wireless link is between UEs and network node (UE to network node and/or network node to UE) rather than direct wireless links (UE to UE). NOTE 2: Length x width (x height). NOTE 3: Communication includes direct wireless links (UE to UE). NOTE 4: Latency and reliability KPIs can vary based on specific use case/architecture, e.g. for cloud/edge/split rendering, and may be represented by a range of values. NOTE 5: The decoding capability in the VR headset and the encoding/decoding complexity/time of the stream will set the required bit rate and latency over the direct wireless link between the tethered VR headset and its connected UE, bit rate from 100 Mbit/s to [10] Gbit/s and latency from 5 ms to 10 ms. NOTE 6: The performance requirement is valid for the direct wireless link between the tethered VR headset and its connected UE. |  |  |  |  |  |  |

Observation 1: UL+DL between UE and the interface to data network is defined for “Cloud/Edge/Split Rendering” instead of “Max Allowed End-to-end latency” and there are several KPI values still open, i.e.:

- “Max Allowed End-to-end latency” for use case “Consume VR content via tethered VR headset”

-  “Service bit rate” for all the 3 use cases

- “Reliability” for use case “Consume VR content via tethered VR headset”

Status in SA4

Initial Traffic Characteristics for different architectures are defined in TR 26.928 / Table 6.3-1 as below:

| Architecture | DL Rate range | UL Rate range | DL PDB | UL PDB | RTT | DL PER range | UL PER range | Traffic periodicity range | Traffic file size distribution |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Viewport independent streaming | 100 MBPs | HTTP requests every second. TCP handshake | See adaptive streaming | See adaptive streaming | See adaptive streaming and TCP equation | 10e-6 | 10e-6 | Almost constant | Almost constant |
| Viewport dependent streaming | 25 MBPs | More frequent HTTP requests every 100ms. TCP handshake | See adaptive streaming | See adaptive streaming | See adaptive streaming and TCP equation | 10e-6 | 10e-6 | Almost constant | Almost constant |
| Viewport Rendering in Network case 1 | 100 MBit/s | FFS | FFS | FFS | FFS | FFS | FFS | FFS | FFS |
| Viewport Rendering in Network case 2 | 1 GBit/s | FFS | FFS | FFS | FFS | FFS | FFS | FFS | FFS |
| Viewport Rendering in Network case 3 | 10 Gbit/s | FFS | FFS | FFS | FFS | FFS | FFS | FFS | FFS |
| Raster-based Split Rendering with Pose Correction | 100 Mbit/s | 500 kbit/s | 20ms | 10ms | 50ms | FFS | FFS | Almost constant | FFS |
| Generalized Split Rendering | FFS | FFS | FFS | FFS | FFS | FFS | FFS | FFS | FFS |
| XR Distributed Computing | FFS | FFS | FFS | FFS | FFS | FFS | FFS | FFS | FFS |
| XR Conversational | FFS | FFS | FFS | FFS | FFS | FFS | FFS | FFS | FFS |
| XR Conferencing Details are FFS | 3Mbit/s up to 50Mbit/s per user | 3Mbit/s up to 50Mbit/s | Allowing real time communication | Allowing real time communication | Allowing real time communication | FFS | FFS | almost constant (with peek during start-up) | > 50Mb at the beginning, depending on media consumption no or almost constant |

Observation 2: For Raster-based split rendering, DL/UL PDB is recommended to be 20 ms / 10 ms and service bitrate for DL/UL is 100m bps/500 kbit/s, which is different from the definition in TS 22.261.

Proposal 1: SA2 shall follow the aligned QoS KPI values as recommended by SA1 and SA4 while developing new KPIs for AIS service scenarios, and in case of conflict, the value recommended by SA4 takes precedence, i.e.:

- For “Cloud/Edge/Split Rendering”, the value of DL/UL PDB is recommended to be 20 ms / 10 ms and service bitrate for DL/UL is 100m bps/500 kbit/s

Observation 3: There’s no QoS KPI SA4 recommendation for “Gaming or Interactive Data Exchanging” and “Consume VR content via tethered VR headset” .

Proposal 2: Postpone the potential new standardized QoS definitions for “Gaming or Interactive Data Exchanging” and “Consume VR content via tethered VR headset” until the related values are clarified in SA1 and SA4.

Proposal 3: Send a LS to SA1 and SA4 to clarify the values in conflict and the values which are still open.

# 2. Proposal

It is proposed to approve below way forward for the definition of new standardized 5QI for AIS in SA2:

Proposal 1: SA2 shall follow the aligned QoS KPI values as recommended by SA1 and SA4 while developing new KPIs for AIS service scenarios, and in case of conflict, the value recommended by SA4 takes precedence, i.e. for “Cloud/Edge/Split Rendering”, the value of DL/UL PDB is recommended to be 20 ms / 10 ms and service bitrate for DL/UL is 100m bps/500 kbit/s.

Proposal 2: Postpone the potential new standardized QoS definitions for “Gaming or Interactive Data Exchanging” and “Consume VR content via tethered VR headset” until the related values are clarified in SA1 and SA4.

Proposal 3: Send a LS to SA1 and SA4 to clarify the values in conflict and the values which are still open.