3GPP TSG-WG SA2 Meeting #1412 e-meeting 	S2-2009039

Elbonia, November 16 – 20, 2020, Electronic meeting

Source:	Qualcomm, NEC, Apple, Nokia, Nokia Shanghai Bell

Title:	KI#6 evaluation and conclusions

Document for:	Agreement

Agenda Item: 	8.4

Work Item / Release:	FS_eNS_Ph2 / Rel-17

Abstract of the contribution: This contribution proposes evaluation and conclusions for KI#6.

# 1	Proposal

The paper analyses solutions for KI #6 and proposes conclusions.

Start of Change

# 7	Overall Evaluation

## 7.x	Evaluation of Solutions for Key Issue #6 - Constraints on simultaneous use of the network slice

The related solutions are #26, #27, #28, #39, #40, #41, #42

| Solution #26 | Network controlled enforcement of simultaneous usage of network slices based on user preference | - it enables the UE to determine the priority for all the requested slices, or a subset of them, based on the services/applications active in the UE. - It enables the CN to determine the Allowed NSSAI even if the UE does not provide any prioritization (basically in this case there is no particular behaviour and in fact the solution does nothing compared to rel-16 – so it is unlikely to effectively enforce any policy) - it requires coordination between the UE and the network, and that the network interprets the prioritization provided by the UE. - priority may conflict with compatibility based selection in the network, so it may be that the UE is limited to use very small set of compatible slices at a time |
| --- | --- | --- |
| Solution #27 | Network slices simultaneous usage incompatibility support | Somehow similar to solutions #28, #41, and #42 in the sense that the UE is provided configuration information wrt network slice incompatibility attribute per allowed S-NSSAI for simultaneous use. - It is deterministic in the CN knowledge of whether the UE can support network slices simultaneous usage incompatibility feature, due to the UE explicit indication - The UE is provided explicit incompatibility information per allowed S-NSSAI. - It allows the UE to register to slices that are incompatible while using UP resources only for compatible sets of slices. The incompatible Slices, however, still need to be supported in all the TAs in the RA. - it relies on UE or network to release or deactivate PDU sessions if UE needs to establish or activate a PDU session for an incompatible slice - it does not require UE to re-register to get user plane resources for incompatible slices wrt the current slices with active PDU Sessions - it allows an option for light mode of operation where the incompatibility attribute is not provided to the UE and the incompatibility enforcement is done in the network only. The light mode can be applied to UEs not supporting the incompatibility feature. In this case the incompatibility enforcement is done by the network only and an existing reject cause is returned to the UE. - The incompatibility for simultaneous use attribute is a subscription information, so it is based on HPLMN policies |
| Solution #28 | Compatibility Class based support for simultaneous use of the network slice | Somehow similar to solutions #27, #41, and #42 in the sense that the UE is provided configuration information wrt the sets of slices that are compatible. It proposes that the network provides to the UE for each S-NSSAI a "simultaneous use information" compatibility class defining the compatibility with other Network Slices, and that the UE shall not include in the Requested NSSAI S-NSSAIs that map to HPLMN S-NSSAIs which are incompatible with each other - The UDM provides the AMF with compatibility class info if any applies to a S-NSSAI in subscription, in other words the incompatibility for simultaneous use attribute is a subscription information, so it is based on HPLMN policies. - This is provided to the UE together with the Configured NSSAI - It allows the UE to be aware of the Compatibility Class information during the UE registration, so it avoids registering with incompatible slices. - If a VPLMN does not support the feature, only compatible slices can be sent as subscription data to that VPLMN. Thus it has no impact on existing networks. - If the UE does not support the feature, the Configured NSSAI the AMF sends only includes compatible slices, based on HPLMN decision (they are highlighted in subscription data as the compatible set to be provided in this case) - It prevents the UE to attempt to register with incompatible slices upfront as the UE is aware of what slices are compatible according to the policy it receives from the network |
| Solution #39 | Support registration for incompatible network slice(s) in UE's current serving RA or different one | The solution does not address all the scenarios. - It imposes minimal impact to the UE, by adding a cause code indicating a rejected s-NSSAI is rejected due to compatibility with slcies in the allowed NSSAI - It does not differentiate the Rejected S-NSSAIs which are outside of the UE’s current serving TA/RA. NOTE: The new Rejection Cause is necessary to allow same TA/RA to serve the compatible/incompatible slices because today Rejection Cause forbid UE to re-register Rejected S-NSSAI within the same TA/RA. |
| Solution #40 | Separate SUPI/GPSI per isolated set of S-NSSAIs | This creative solution enables different “UE profiles” associated to different SUPIs/GPSIs and that identify specific set of S-NSSAIs that are compatible. - It assumes roaming agreements ensures that compatibility between S-NSSAIs is coordinated. - It does not require UE to declare what it supports to the network nor requires any coordination between VPLMN and HPLMN for upgrades since it is backward compatible with existing procedures and mechanism for allowed/configured/subscribed slices. What is not understood is just ignored. - it has NAS impacts in terms of new registration procedure and new NAS mechanisms internal to the UE (e.g. switch between SUPIs) - it seems to have USIM impact as the UE is said to register with the SUPI of each independent slice group. |
| Solution #41 | Simultaneous use of the network slice via Configured NSSAI | The solution is similar to #27, #28, and #42 in the sense that the UE is provided configuration information wrt the sets of slices that are compatible. - It can inform the UE of all the deployment and business constraints related to the serving network's network slices (based on network configuration, SLA with HPLMN and possibly dynamic information). - It requires UE to be aware of the SUG (Simultaneous Usage Group) information during the UE registration. - the supporting UE is not expected to register at the same time for incompatible slices - as since Rel-15, it requires UE to re-register to get data resources for incompatible slices wrt the current slices with active PDU Sessions - the HPLMN constraints are not part of subscription data, so every change of policy needs roaming agreement update. - It addresses VPLMN constraints that are already supported by the current system and it is unclear how this is related to GSMA NG.116 attribute that is related to HPLMN only |
| Solution #42 | UE handling of constraints on simultaneous use of network slices based on network assistance | The solution is similar to #27, #28, and #41 in the sense that the UE is provided configuration information wrt the sets of slices that are compatible. - It requires UE to be aware of the category information during the UE registration. Furthermore, this solution proposes multi-level network slice compatibility. - the UE can never register at the same time for incompatible slices - it requires UE to re-register to get data resources for incompatible slices wrt the current slices with active PDU Sessions |

Next Change

# 9	Conclusions

## 8.6	Conclusion for Key Issue #6

It is proposed that:

The UEs supporting network slices incompatibility for use enforcement feature indicate its support for it in the Registration Request message.

The AMF is provided incompatibility constraints from UDM

The AMF provides supporting UEs with incompatibility information.

The UDM only provides compatible set of slices to VPLMN that does not support the feature.

The AMF only provides compatible set of slices for UEs that do not support the feature

A PLMN can operate in a Control plane-centric simultaneous network slice use mode

If so:

-	The AMF enforces compatibility constraints at registration time.

-	A supporting UE shall not register with incompatible slices in the Configured NSSAI. according to the received information. If a supporting UE requests incompatible network slices, the AMF provides a compatible set of Network Slices as allowed NSSAI, rejects incompatible slices the UE had requested with a specific cause code, and reconfigures the UE

A PLMN can operate in a user plane-centric simultaneous network slice use mode

If so:

-	The AMF enforces incompatibility constraints at Session management time.

-	A supporting UE can register with incompatible slices in the Configured NSSAI. according to the received information;

-	A supporting UE shall not request for PDU session on a network slice that is incompatible for simultaneous use on a network slice on which there is an already active PDU session. If the UE requests to establish a PDU session for an incompatible network slice, the AMF rejects the PDU session establishment with a specific cause code

-	If a non-supporting UE registers, the network allows only compatible network slices, or allow also incompatible slice and operate in light mode. If incompatible slices are allowed and light mode is activated, if request for a PDU session is rejected for incompatibility with existing PDU session for other slices, an existing reject cause is used with optional back-off timer.

NOTE 1: it is recommended that light mode of operation is not used with supporting UEs.

NOTE 2: in light mode of operation (which is only used with non-supporting UEs), the UE may obey to a backoff timer.

A UE can indicate it supports only one of the simultaneous network slice use mode or both. The network decides which mode to use. If a UE indicates a mode to the network and the network does not intend to support the mode for the network slices, it only provides the UE with a compatible set of network slices in the configured NSSAI, unless the network intends to enable the user plane-centric simultaneous network slice use mode and use light mode for the non-supporting UE.

End of Changes