SA WG2 Meeting #S2-142e	S2-2008429r03

16 – 20 November, 2020, Electronic

Source:	Ericsson, Nokia, Nokia Shanghai Bell  , NTT Docomo?, Qualcomm?, vivo

Title:	KI #1: Conclusion update

Document for:	Approval

Agenda Item:	8.5

Work Item / Release:	FS_IIoT/ Rel-17

Abstract of the contribution: This contribution clarifies KI#1’s conclusion.

Discussion

The objective of this Key Issue is to introduce support for Time Synchronization with TSN GM in the TSN network attached to the device. Therefore, TSN time synchronization with gPTP according to IEEE 802.1 AS is within the scope.

Observation 1: The BMCA procedure to which 8.1 conclusion of KI#1 referred to, is unclear, as 5GS internal time synchronization process may also involve BMCA procedure. KI#1 should only address TSN BMCA procedure.

KI#1’s scope is uplink synchronization, the TSN BMCA procedure involves three cases:

Figure 1. TSN BMCA procedure (uplink case 1): TSN GM is at device side and TSN node(s) with gPTP clock(s) is at network (N6) side

In uplink case 1, as shown in Figure 1, the DS-TT port (port 1 in figure) receives announce messages from TSN GM, the DS-TT forwards announce messages to NW-TT for BMCA processing. The NW-TT port (port 2 in figure) then generates announce messages towards TSN nodes connected at N6 side.

Figure 2. TSN BMCA procedure (uplink case 2): TSN GM is at device side and TSN node(s) with gPTP clock(s) is at another device side

In uplink case 2, as shown in Figure 2, the DS-TT port (port 1 in figure) receives announce messages from TSN GM, the DS-TT forwards announce messages to NW-TT for BMCA processing. The NW-TT determines the port states of another DS-TT port (port 3 in figure).

If the state of the DS-TT port 3 is already “Master”, DS-TT generates announce messages towards TSN nodes connected to DS-TT port 3.

If the state of the DS-TT port 3 is other than “Master”, NW-TT changes the state of DS-TT port 3 to “Master”. Then DS-TT can generate announce messages towards TSN nodes, or NW-TT can generate announce messages on behalf of DS-TT port 3.

Figure 3. TSN BMCA procedure (uplink case 3): when 5GS clock is the best, 5GS acts as TSN GM

In uplink case 3, We do not cover the case where TSN GM is attached to the network side, but we cover the case that 5GS can become the GM for TSN (e.g., after a failure of the original TSN GM deployed in the device side), as shown in Figure 3. The DS-TT port (port 1 in figure) receives announce messages from TSN GM, the DS-TT forwards announce messages to NW-TT for BMCA processing. In the case that 5GS clock is the best (as a result of BMCA procedure), the 5GS acts as the newly elected TSN GM.

The NW-TT changes the port state in DS-TT port (port 1 in figure) from “Slave” to “Master”, and then DS-TT port 1 generates announce messages towards TSN GM.

NW-TT port (port 2 in figure 3) is in “Master” state and generates announce message for the TSN nodes connected at N6 side.

Observation 2: The NW-TT port at N6 side is always set in “Master” state when it is involved in KI#1 uplink synchronization cases.

Observation 3: Neither DS-TT nor NW-TT can act as grandmaster, in the case 3 it is the 5GS as a whole (time-aware system) that acts as a GM for TSN. DS-TT port and NW-TT port are gPTP and implement the gPTP ports of the newly elected grandmaster. DS-TT and NW-TT ports are set to “Master” state. Instead of NW-TT acting as grandmaster or DS-TT acting as grandmaster, it would be more correct to say that NW-TT or DS-TT implement the grandmaster fundamental functions, e.g., generate the Sync and Announce messages.

TR 23.700-20 Clause 8.1 states:

“The TSN AF configures the grandmaster in DS-TT port (if supported) via PMIC; otherwise TSN AF configures the grandmaster in NW-TT via BMIC for PDU sessions where the DS-TT does not support acting as a gPTP GM.”

Observation 4: It is not clear what the grandmaster is in DS-TT and NW-TT.

Observation 5: The role of TSN AF and BMCA procedure in NW-TT is vague, which entity can configure port states is unclear.

Merging with S2-2009072 and S2-2008379

Proposal

Based on the discussion above, it is proposed to update 23.700-20 as follows.

* * * * Start change * * * *

8.1	Key Issue #1: Uplink Time Synchronization

To enable support for Uplink Time Synchronization, following principles are recommended for the basis of normative work:

-	The following principles are applied for transmission of the gPTP event messages (carried within Sync message for one-step operation or Follow_up message for two-step operation):

-	DS-TT which is attached by one or more TSN GMs will perform exactly the same operations for UL gPTP messages as what the NW-TT performs for the DL gPTP messages as specified in clause 5.27.1.2.2 of TS 23.501 [2].

-	The PDU session of DS-TT port receiving the source GM's gPTP messages is used to forward the gPTP messages to the UPF/NW-TT:

-	the UPF/NW-TT forwards the gPTP messages to the NW-TT and DS-TT ports in Master state.

-	If the egress port is in NW-TT, the NW-TT port performs exactly the same operations for UL gPTP messages as what the DS-TT performs for the DL gPTP messages as specified in clause 5.27.1.2.2 of TS 23.501 [2].

-	If the egress port is in other DS-TT, the other DS-TT(s) perform the operation as specified in clause 5.27.1.2.2 of TS 23.501 [2].

-	As specified in TS 23.501 [2] already for Rel-16 Time Synchronization, 5GS is required to guarantee delivery of gPTP message in less than 10ms as recommended by IEEE 802.1AS[6] clause B2.2, if the operator decides to follow such recommendation.

-	The following principles are applied for TSN BMCA procedure:

-	The NW-TT maintains the PTP port state for each DS-TT port and NW-TT port. The PTP port states may be either locally configured in NW-TT or evaluated by BMCA procedure.

-

When the 5GS Bridge PTP port states are evaluated by BMCA procedure, the NW-TT needs to process the received Announce messages (from NW-TT port(s) and over user plane from the DS-TT(s)) for BMCA procedure, determine port states within the 5GS, and maintain Master-Slave hierarchy.

-	When the 5GS Bridge PTP port states are locally configured in NW-TT, 5GS behaviour specified as in TS 23.501[2] Annex H.3 applies, except the DS-TT port may be configured in Slave state.

-	When the NW-TT is determined to be the grandmaster function for PTP ports in DS-TT and NW-TT, the NW-TT generates the Announce messages for the Master ports on the NW-TT and DS-TT(s). The NW-TT/UPF forwards the generated Announce messages to the PDU session(s) related to the Master ports on the DS-TT(s).

-  When the TSN GM is external to the 5GS, the NW-TT regenerates the Announce messages based on the received Announce messages for the Master ports on the NW-TT and DS-TT(s). The NW-TT/UPF forwards the regenerated Announce messages to the PDU session(s) related to the Master ports on the DS-TT(s).

-	When the DS-TT(s) is determined to be the grandmaster function for PTP port in the DS-TT the DS-TT generates the Announce messages for the Master port in this DS-TT and the NW-TT generates the Announce messages for the Master ports in the NW-TT.

-

If the TSN AF has subscribed for the BMCA result reports, the NW-TT reports BMCA result to the TSN AF via BMIC.

-	DS-TT forwards the received Announce messages to NW-TT over User plane.

* * * * End change * * * *