SA WG2 Meeting #S2-142E	S2-2008741r20

16 - 20 November, 2020, Electronic meeting

Source:	China Mobile, China Unicom, NTT DOCOMO, Huawei, ZTE, CATT, NEC

Title:	Conclusion for key 1 and key 2 of new NF

Document for:	Approval

Agenda Item:	8.4

Work Item / Release:	FS_eNS_Ph2/ Rel-17

Abstract: c

# 1. Introduction

In conclusion, it is preferred to introduce a NF to handle the attributes in key issues 1&2 in normative work.

# 2. Text Proposal

It is proposed to capture the following changes in TR 23.700-40.

# * * * * First change * * * *

## 8.1	Conclusion for Key Issue #1

To enable a 5GS to support network slice related quota on the maximum number of UEs, no change is required in the RAN. The following new functionalities in the 5GS are needed:

-	Storing of network slice related quota information: If a network slice is subject to a network slice quota checking on a maximum number of UEs, it is assumed that the O&M should have for this network slice a) the information of the quota of maximum number of UEs. To enable the network slice related quota enforcement, this information is configured and stored in 5GC.

-	Managing and updating the network slice related quota on maximum number of UEs registered for the network slice: This functionality is part of the 5GC and it manages the NW Slice quota of maximum number of UEs in a S-NSSAI, monitors the current number of UEs being registered for the network slice subject to a network slice quota checking.

-	Enforcing the network slice related quota on the maximum number of UEs: This functionality is part of the 5GC and it controls the registration request on the S-NSSAI subject to the quota management by accepting or rejecting the request on the S-NSSAI. In case of rejection, the function may provide a rejection cause and optionally with a back-off timer.

-	A new NF is defined to support the storing of network slice information (i.e., the maximum number of UEs allowed to be served by a network slice), and managing and updating , enforcing the network slice information for the purpose of network slice access control. The new NF  can be deployed as standalone or co-located withan existing NF e.g. PCF.

-	SA WG5 is informed of the work done in this TR and is provided with a reference to solution #38 so that they can consider further if they want to develop a solution for charging. Any potential alignment will be progressed as usual.

-	Both centralized and distributed quota handling enforcement of the number of UEs per network slice shall be supported.

NOTE:	Whether to use an existing rejection cause and a back-off timer or a new rejection cause and a back-off timer, this is to be determined in Stage-3.

## 8.2	Interim conclusion for Key Issue#2

To enable a 5GS to support network slice related quota on the maximum number of PDU Sessions, the following new functionalities in the 5GS are needed:

-	Storing of network slice related quota information: If a network slice is subject to a network slice quota management on a maximum number of PDU Sessions, it is assumed that the O&M should have for this network slice a) the information of the quota of maximum number of PDU Sessions. To enable the network slice related quota enforcement, this information is configured and stored in 5GC.

-	Managing and updating the network slice related quota on maximum number of PDU Sessions established in a S-NSSAI: This functionality is part of the 5GC and it manages the NW Slice quota of maximum number of PDU Sessions in a S-NSSAI, and updates the current number of PDU Sessions successfully established in the network slice subject to a network slice quota checking on a maximum number of PDU Sessions.

-	Enforcing the network slice related quota on the maximum number of PDU Sessions: This functionality is part of the 5GC and it controls the establishment of PDU session of a S-NSSAI subject to the quota management by accepting or rejecting the request. In case of rejection, the function may provide a rejection cause and optionally with a back-off timer.

NOTE:	Whether to use an existing rejection cause and back-off timer or a new rejection cause and back-off timer, this is to be determined in Stage-3.

-	A new NF is defined to support the storing of network slice information (i.e., the maximum number of PDU Sessions allowed to be served by a network slice), and managing and updating, enforcing the network slice information for the purpose of network slice access control. The new NF can be deployed as standalone or co-located with an existing NF e.g. PCF.

-	SA WG5 is informed of the work done in this TR and is provided with a reference to solution #38 so that they can consider further if they want to develop a solution for charging. Any potential alignments will be progressed as usual.

-	Both centralized and distributed quota enforcement handling of the number of PDU Sessions per network slice shall be supported.

# * * * * End of changes * * * *