SA WG2 Meeting #142e	S2-2008366r01

16 - 20 November 2020, e-meeting	(revision of S2-200xxxx)

Source:	InterDigital

Title:	KI#4: conclusion update

Document for:	Approval

Agenda Item:	8.7

Work Item / Release:	FS_ID_UAS / Rel-17

Abstract: This contribution proposes to add an Editor's Note to conclusion text for KI#4 regarding privacy protection of UAV during location tracking.

# 1. Discussion

It was concluded during meeting SA2#141e that Solution #25 was adopted as the basis for normative work.

In particular the 3GPP network is to support the following possible UAV tracking capabilities:

- reporting the location of a particular UAV;

- monitoring and reporting of a UAV presence relative to a monitoring area (i.e. UAV moving in or out of that area).

- reporting of any UAV in a given target area

However what is not currently specified is how the 3GPP system ensures that the above tracking capabilities are protected so that to provide privacy sensitive information (i.e., 3GPP UAV ID, UAV location) only to authorized entities (e.g., the USS/UTM serving the UAV being tracked). For example, when reporting the 3GPP UAV ID of any UAVs served by a PLMN in a target area the 3GPP system may expose privacy sensitive information of several UE/UAVs to a USS/UTM or entity that may not be authorized to access that information (e.g., USS/UTM other than the one serving the UAV, attacker impersonating a USS/UTM).

This is not dissimilar from existing LCS capabilities in the sense that an LCS client or AF (e.g., USS/UTM) must be authorized to use the location service (see TS 23.273). However unlike "regular" UE LCS based tracking, UAV tracking will be a regulatory mandate where the end-user has no control of who can obtain its location (e.g., using UE LCS privacy profile as per TS 23.273). So it is crucially important for the 3GPP system to ensure that only authorized entities can track a particular UAV (e.g., based on local privacy regulations).

Summary: without the proper protection in place, the UAV tracking capabilities provided by the 3GPP system may be exploited to expose privacy sensitive information about UAVs. Potential security solutions may have impacts on the location tracking API work during the normative phase.

Proposal: an Editor's Note is added to KI #4 conclusion to track the above issue to be discussed in coordination with SA3.

# 2. Proposal

It is proposed to capture the following change in the conclusion for KI#4 in TR 23.754.

* * * * First Change * * * *

## 6.25	Solution #25: Network Exposure to support UAV and UAVC tracking

### 6.25.1	Introduction

This is a solution for Key Issue #4 and #7. It's a partial merge of solution 1, 9, 13,14,15,16, and 17 where certain aspects where commonality exist are brought together here in this solution.

In the Figure 6.25.3-1:

Step1, 3a, 4, 5 are from the concept of solution 15, with the difference that the UAVF is added to expose the location service to UTM/USS.

Step2, 3a, 3b, 7, 8a, 8b are from the concept of solution 16, with replacing NEF in solution 16 by the UAVF, the GMLC enhancement is not included.

Step 6, 9, 10a are from the concept of solution 13, 14, 16, and 17 that the 3GPP network can monitoring the UAV presence in the monitoring area by comparing the UAV location with the monitoring area and can expose other monitoring events notifications (e.g. Loss of Connectivity, UE reachability) to USS/UTM.

Step 8b, 10b are from the concept of solution 13 with replacing NEF in solution 13 by the UAVF, and preconfigure the traffic routing policies at UAVF instead of at SMF.

Step 11,12, 15 are from the concept of solution 1 for tracking unknow UAVs in the target area which can be mapped to 3GPP network areas. The difference is to reuse the existing AMF event "Number of UEs present in a geographical area" or "location Reporting" with Any UE in the event filter.

Step 13 is from the concept used in solution 9 for tracking unknown UAVs in the target area but applied here for the case that the target area cannot be mapped to 3GPP network areas. Here while the AMF event "Number of UEs present in a geographical area" or "location Reporting" can be also used, upon receiving the list of UEs from AMF, the UAVF invokes GMLC procedures for obtaining location to resolve the target areas that cannot be mapped to 3GPP areas.

### 6.25.2	Functional Description

This solution proposes the following principles:

-	A 3GPP network NF UAVF is introduced in this solution, the following functions are supported by UAVF:

-	This NF is operated by the PLMN serving the target UAV/UAVC UE: it is part of 5GC/EPC, reached by USS/UTM via NEF/SCEF (or may encompass the NEF/SCEF functionality) and has access to PLMN based information such as SUPI, TA, cell Id.

NOTE 1: in the scope of this solution, the term UAVC is used to indicated exclusively a networked UAVC.

-	Providing to UTM/USS location information about the UAV and /or UAVC. Based on the request from UTM/USS this location information could be:-

-	Immediate reporting or deferred reporting (e.g. periodic reporting) based on the request from UTM/USS. It's shown in the first dotted box of Figure 6.25.3-1.

-	Monitoring the UAV presence in the monitoring area (e.g. moving in or out of the monitoring area) and providing the monitoring report to UTM/USS. The UAVF reuses the Area of Interest mechanism to receive the reporting of UE presence in the monitoring area from AMF in case the monitoring area can be mapped to 3GPP network areas. Or the UAVF determines the UAV presence in the monitoring area by comparing the location report from GMLC with the monitoring area from UTM/USS. It's shown in the second dotted box of Figure 6.25.3-1.

-	Providing the list of 3GPP UAV IDs corresponding to the UAVs served by the PLMN being queried and present in the identified target area to UTM/USS. It could be immediate reporting or deferred reporting (e.g. periodic reporting) based on the request from UTM/USS. It's shown in the third dotted box.  The AMF event "Number of UEs present in a geographical area" or "location Reporting" with Any UE in the event filter can be reused. If the target area can be mapped to 3GPP network areas, UAVF identifies the UAVs from the UE list received from AMF, and feedback the identified UAVs to UTM/USS.

If the target area cannot be mapped to 3GPP network areas, UAVF provides a TA List which is larger than the target area to AMF for the list of UEs to be queried. Then upon receiving the list of UEs from AMF, UAVF queries the UEs' location from GMLC, and then identifies the UAVs in the target area and feedback to UTM/USS.

The list reported to the USS/UTM contains only UAVs.

Editor's note:	Providing UAV list to the UTM/USS for unknown UAV tracking may raise privacy concerns (e.g., if UTM/USS is not the one serving the UAV). The security solutions for privacy protection is FFS and should be discussed in coordination with SA3.

NOTE 2: UAVF exposes the above UAV and UAVC locations report and UAV presence monitoring report capabilities to UTM/USS, it's decided by UTM/USS to choose one of or a combination of these capabilities. The UTM/USS logic is out of 3GPP scope.

NOTE 3: Enhancement on GMLC to support the reporting of UE presence in the monitoring area is not considered in this solution.

-	Mapping the 3GPP UAV ID provided by UTM/USS to the 3GPP UE IDs required by location related NFs. i.e. AMF, GMLC.

-	Receiving a 3GPP UAV ID from UTM/USS in the location request messages and sending the CAA-level UAV ID to UTM/USS in the location report(s).

-	Mapping the monitoring area from UTM/USS (e.g., longitude/latitude, zip code, UE trajectory, etc) to 3GPP network area information e.g. a list of TAs/Cell ids.

-	Decides the related NF, i.e. AMF or GMLC to invoke, e.g. based on the UAV capability or network capability, the location accuracy, monitoring area received from UTM/USS.

-	Be preconfigured or receives from UTM/USS the traffic routing policies, when the location of UAV or the UAV presence in the monitoring area matches a policy, UAVF based on the policy indicates SMF to take the appropriate network layer actions, e.g. revoke the connectivity between UAV and UAV controller, steering the traffic toward/ from the UAV to UTM/USS/TPAE for further analysis, etc. UAVF considers those policies as active and ongoing instructions from UTM/USS without constant or repeated triggers/requests from UTM/USS. The traffic routing policy includes 3GPP UAV ID(s) (e.g. GPSI or External Group Identifier) to identify the UAV(s)/UAVC(s) and the corresponding network layer actions e.g. revoke the resources of the related C2 communications.

This solution applies to both 5GC and EPC. With replace the AMF by the MME, replace the SMF by the PGW.

### 6.25.3	Procedures

Figure 6.25.3-1

Step 1-5 shows the procedure that the 3GPP network exposes the UAV and UAVC location service to UTM/USS.

1.	The UTM/USS sends the location request to UAVF to request UAV and/or UAVC locations from network. The UAV/UAVC ID provided by UTM/USS follows the conclusion of KI#1. The request also includes the location accuracy

[Conditional] If the UTM/USS needs to calculate the distance between UAVC and UAV, the UTM/USS may indicate the UAVF to provide location information of UAVC and/or UAV at the same time, e.g., using the same start time for periodic reporting or a synchronization indication for immediate reporting in the location request.

NOTE 1:	Whether the request also includes other attributes is to be determined in the normative phase.

2.	UAVF decides the related NF, i.e. AMF/MME or GMLC for location reporting, e.g. based on the UAV/UAVC capability or network capability, location accuracy etc.

[Conditional] If UAVF needs to provide location information of UAVC and UAV at the same time, it can request the AMF/MME or GMLC to provide the position of the UAVC and/or UAV at the same time by providing the start time of reporting.

3.	UAVF maps the UAV/UAVC ID provided by UTM/USS to the UE IDs required by the NF decided by step2. i.e. AMF/MME or GMLC. UAVF gets the UAV/UAVC location from AMF/MME or GMLC by the current location services supported by AMF or GMLC.

4.	UAVF provides the UAV and/or UAVC locations reporting to UTM/USS. The UAVF includes the UAV and/ or UAVC CAA-level UAV ID in the location reporting message to UTM/USS as well as a location information (geo co-ordinate) suitable for USS/UTM (that is not assumed to know what TA and Cell Id are).

5.	UTM/USS can use the output received at step 4 from UAVF to e.g. compares the received UAV location with the monitoring area (e.g. allowed area or non-allowed area for Geofencing).

Step 6-10b shows the procedure that the 3GPP network exposes the UAV presence monitoring service to UTM/USS.

6.	The UTM/USS sends the UAV presence monitoring request to UAVF to request UAV presence events from 3GPP network (e.g. moving in or out of the monitoring area). The request also includes the monitoring area, location accuracy and other attributes. The monitoring area could be area inside or outside the fence for Geofencing.

NOTE 2: The UTM/USS can also subscribe to the UAVF other Monitoring Events (e.g. Loss of Connectivity, UE reachability) already defined in clause 4.15.3.1 of TS 23.501, the UAVF can obtain corresponding Monitoring Events Notifications from related NFs (e.g. AMF, UDM), and respond to the UTM/USS.

7.	UAVF decides the related NF, i.e. AMF or GMLC for location reporting, e.g. based on the UAV capability or network capability, location accuracy, the monitoring area etc.

8.	UAVF maps the 3GPP UAV ID provided by UTM/USS to the UE IDs required by the NF decided by step7. i.e. AMF or GMLC. In case the monitoring area can be mapped to 3GPP network areas, UAVF provides the monitoring area to AMF and gets the reporting of UE presence in the monitoring area by reusing the Area of Interest mechanism. Otherwise UAVF gets the UAV location from GMLC by the current location services supported by GMLC. It's also possible UAVF gets the UAV location from GMLC if it determines that a finer granularity of location is needed when the report is received about the UAV presence of a coarse granularity area.

9.	UAVF determines the UAV presence in the monitoring area by the reporting of UE presence in the monitoring area from AMF or by comparing the location report from GMLC with the monitoring area from UTM/USS.

10.	UAVF reports the UAV presence in the monitoring area to UTM/USS by including its CAA-Level UAV-ID in the report(s). The UAVF may include the UAVC's CAA-level UAV ID and 3GPP UAV ID in the reports to enable to the UTM/USS to look up the UAVC associated with the UAV. Or if traffic routing policies are stored in the UAVF, based on the policy and the UAV presence in the monitoring area events detected at step9, UAVF indicates SMF to take the appropriate network layer actions, e.g. revoke the connectivity between UAV and UAV controller, steering the traffic toward/ from the UAV to UTM/USS/TPAE for further analysis, etc. UAVF considers those policies as active and ongoing instructions from UTM/USS without constant or repeated triggers/requests from UTM/USS. The UAVF will provide the 3GPP UE ID(s) of the UAV(s)/UAVC(s) and the corresponding network layer actions to SMF.

Steps 11-14 shows the procedure that allows the 3GPP network to provide the UAV identity for each UAV or UAVC served by the queried PLMN, and corresponding to the USS/UTM query conditions (e.g. a target area), to UTM/USS.

11.	The UTM/USS sends the UAV list request to UAVF to request UAV UE identity, providing query conditions (e.g. including a target area) from 3GPP network.

12.	UAVF decides the AMF(s) based on the query conditions (e.g. target area information) and obtains the UE list in the target area from AMF by reusing the event "Number of UEs present in a geographical area" or "location Reporting" with Any UE in the event filter. If the target area cannot be mapped to 3GPP network areas, UAVF provides a TA List which is larger than the target area to AMF for the list of UEs to be queried. The 3GPP system separates out the UEs that are actual UAVs based on e.g. available CAA-Level UAV ID, checking for UEs with aerial subscriptions, and whether the UAV has successfully registered and has been authorized by the USS/UTM.

13.	[Optional] From the list of UEs generated in step 12, for UAVs that are in target areas that do not map to 3GPP network areas, the UAVF then queries the UAV(s) location from GMLC.

14. If step 13 is executed, from the list of locations returned by the GMLC the UAVF compares the UAV locations to the target area (provide in step 11) to identify the UAVs to report. If the target area can be mapped to 3GPP network areas, the list received in step 12 is used.

15.	UAVF feedbacks the identified UAVs to UTM/USS.

### 6.25.4	Impacts on services, entities and interfaces

-	Add a 3GPP NF which is UAVF as described above together with the corresponding service API of the UAVF exposed via NEF.

-	No impact to other NF (than NEF).

* * * * End of Change * * * *