3GPP TSG-SA/WG2 Meeting #142e	S2-2008373

# Elbonia, Nov. 16 - 20, 2020			   (revision of S2-200xxxx)

Source:	Intel, Interdigital

Title:	KI#3, Conclusion Update on PDU Session related aspect

Document for:	Agreement

Agenda Item: 	8.8

Work Item / Release:	FS_5G_ProSe / Rel-17

Abstract of the contribution: This contribution proposes to update conclusion for KI#3 on the PDU Session related aspect.

# 1	Proposal

The current wording of the priority between the policy and parameters provided by PCF and ProSe Application Server is not very clear, and furthermore, this sentence seems to belong to a NOTE.

It is proposed to agree the changes:

/**************Start of Change *************/

## 8.3	Key Issue #3: Support of UE-to-Network Relay

For Key Issue #3 (Support of UE-to-Network Relay), the followings are taken as interim conclusions:

UE-to-Network Relay conclusions are subject to confirmation from RAN2 and SA3 for normative work.

The final decision on whether or not to proceed with Layer-2 and/or Layer-3 into normative work will be made in cooperation with other WGs.

The following is taken as interim conclusions for the L3 UE-to-Network Relay solution:

-	No showstopper has been identified by SA2 for L3 UE-to-Network solution. SA2 recommends L3 UE-to-Network Relay proceed into normative work, subject to RAN2 and SA3 conclusion: Sol#6 is taken as baseline.

-    For L3 Relay discovery procedure, it is proposed to adopt the standalone discovery procedure (i.e. Model A and Model B), and, the additional information advertised by Relay UE as described in Sol#28 as the basis for normative work.

-  For the L3 relay operation support, Remote UE uses URSP rules to route the traffic on suitable communication path (as described in Sol#26).

-  For the PDU Session relaying the Remote UE’s traffic, when the Remote UE has the mapping between RSC and PDU Session parameters, the Remote UE provides RSC to Relay UE and the NW relay retrieves PDU session parameters associated to the RSC according to received policy/parameters as described in subclause 6.35.2.3 Sol#35; otherwise (i.e. the Remote UE has no mapping or the mapping does not include specific PDU session parameters), the Remote UE also provides DNN, SSC mode, PDU session Type, the Traffic Descriptor to the NW relay.

-	Security aspects require confirmation from SA3.

NOTE: The procedures to support authentication of Remote UE and Relay UE by the network will be determined by SA3 WG.

-	For the Remote UE to use the network resources (e.g. PDU Session and Network Slice) of the Relay UE’s serving network, the network-controlled authorization procedures will be determined in the normative phase with coordination with SA3 WG. The alignment with the associated security procedures to authenticate the Remote UE and Relay UE will be carried out in normative phase via coordination with SA3 WG.

-	The secondary authentication for a Remote UE will be determined by SA3. The alignment with the associated security procedures for secondary authentication of the Remote UE will be carried out in normative phase via coordination with SA3 WG.

Editor's Note: The radio aspects of relay (re-)selection criteria and procedures, and service continuity for L3 U2N Relay are still under discussion in RAN2 in TR 38.836 [x] and will be determined by RAN2.

The followings are taken as interim conclusions for the L2 UE-to-Network Relay solution:

-	No showstopper has been identified by SA2 for L2 UE-to-Network Relay solution. SA2 recommends L2 UE-to-Network Relay solution proceed into normative work, subject to RAN2 and SA3 conclusion.

-	Adopt Control and User Plane Protocols as described in Annex A, with confirmation from RAN WGs.

-	The Remote UE has a NAS connection with 5GC and Remote UE Registration and Connection establishment/management, the related procedure in solution #7 can be taken as baseline.

-	For Relay discovery, both Model A and Model B are supported.

Editor’s note: For paging aspect, it’s FFS whether the concluded solution in clause 6.6.2 of TR 23.733[26] can be reused based on the assumption captured in clause 4.5.5.2 of TR 38.836 [xx] adopted by RAN WG2.

NOTE 1: It is left to RAN2 and to decide how to support end-to-end QoS between the Remote UE and RAN.

NOTE 2: It is left to RAN2 and SA3 to decide the details of how to support end-to-end security between the Remote UE and RAN.

Editor's Note: The radio aspects of relay (re-)selection criteria and procedures, and service continuity for L2 U2N Relay are still under discussion in RAN2 in TR 38.836 [x] and will be determined by RAN2.

Editor’s note: For mobility issue, SA2 may need further study after RAN WG progresses.

/*******************End of Change****************/